LightAPI Interfaces User Guide

Manual Version: V4.00

- Contents

1	Preface	5

1.1	LightAPI Overview	5

1.2	About This Guide	5

1.3	Important Notice	5

1.4	Four HTTP Methods Supported by LightAPI	6

1.5	URL Rules	6

1.6	Get structure Info	7

2	Functions	8

2.1	Alarm Notification	8

2.1.1	Function Introduction	8

2.1.2	Interface Overview	8

2.1.3	Interface Calling Procedure	9

2.2	Search by Image	11

2.2.1	Function Introduction	11

2.2.2	Interface Overview	11

2.2.3	Interface Calling Procedure	12

3	Interfaces	14

3.1	System	14

3.1.1	Time	14

3.1.2	Reboot	16

3.1.3	Logs	16

3.1.4	DeviceInfo	18

3.1.5	ChannelInfos	19

3.1.6	Upgrade	19

3.1.7	Json Block	23

3.2	PTZ	40

3.2.1	PTZCtrl	40

3.2.2	Presets	40

3.2.3	Capabilities	41

3.2.4	AreaZoom	42

3.2.5	Patrols	42

3.2.6	Json Block	46

3.3	Media	51

3.3.1	Capabilities	51

3.3.2	Video	52

3.3.3	Audio	53

3.3.4	LiveStreamURL	54

3.3.5	Records	55

3.3.6	Talk	59

3.3.7	OSD	59

3.3.8	Snapshot	61

3.3.9	Json Block	63

3.4	Image	84

3.4.1	Capabilities	84

3.4.2	Enhance	84

3.4.3	Focus	85

3.4.4	LampCtrl	85

3.4.5	Exposure	86

3.4.6	Illumination	86

3.4.7	Reset	86

3.4.8	Json Block	87

3.5	Storage	106

3.5.1	Capabilities	106

3.5.2	Containers	107

3.5.3	Schedule	108

3.5.4	Json Block	109

3.6	Alarm	114

3.6.1	Capabilities	114

3.6.2	MotionDetection	115

3.6.3	VideoLoss	117

3.6.4	TamperDetection	119

3.6.5	AudioDetect	120

3.6.6	HumanShapeDetection	121

3.6.7	Json Block	123

3.7	IO	139

3.7.1	InputSwitch	139

3.7.2	OutputSwitch	141

3.7.3	Json Block	143

3.8	Smart	147

3.8.1	Capabilities	147

3.8.2	CrossLineDetection	148

3.8.3	IntrusionDetection	150

3.8.4	LeaveZone	152

3.8.5	AccessZone	154

3.8.6	FaceDetection	156

3.8.7	HybridDetection	158

3.8.8	AttributeCollect	160

3.8.9	Json Block	161

3.9	PeopleLibraries	243

3.9.1	PeopleLibraries	243

3.9.2	People	245

3.9.3	MonitorRule	246

3.9.4	PassRecord	251

3.9.5	Json Block	252

3.10	Event	265

3.10.1	Event	265

3.10.2	Json Block	275

3.11	SearchByImage	296

3.11.1	SearchByImage	296

3.11.2	Json Block	301

4	Extended Data Types	307

4.1	Common Color (CommonColorType)	307

4.2	Structured Attributes (AttributeType)	308

Preface

LightAPI Overview

LightAPI is the HTTP message part of "Multiple Controls in One" and adopts HTTP non-persistent connection mechanism. During the process of an interaction, the client notifies the server of the requested resource through the HTTP URL, and the server returns information about the URL-specified resources in Json format.

About This Guide

This guide describes interfaces of the system, PTZ, media, storage, network, image, alarm, serial port, smart services, etc, and uses the following conventions for parameter requirements.

| Format | Meaning |
| --- | --- |
| M | Parameter is mandatory. |
| C | Parameter is required according to conditions. |
| O | Parameter is optional. |

Important Notice

All the LAPI interfaces in this manual adopt Digest authentication. For more information, please refer to https://tools.ietf.org/html/rfc2617

The calculation rules of digest authentication are as follows:

If qop=Undefined

Digest=MD5(MD5(A1):<nonce>:MD5(A2))

If qop="auth:"

Digest=MD5(MD5(A1):<nonce>:<nc>:<cnonce>:<qop>:MD5(A2))

If qop="auth-int:"

Digest=MD5(MD5(A1):<nonce>:<nc>:<cnonce>:<qop>:MD5(A2))

A1 and A2 bits calculate the 2 pieces of data content of the digest

For MD5

A1=<user>:<realm>:<password>;

For MD5-sess

A1=MD5(<user>:<realm>:<password>):<nonce>:<cnonce>.

If qop is undefined or if qop="auth:"

A2=<request-method>:<uri-directive-value>;

If qop="auth-int:"

A2=<request-method>:<uri-directive-value>:MD5(<request-entity-body>).

Four HTTP Methods Supported by LightAPI

| Method | GET | PUT | POST | DELETE |
| --- | --- | --- | --- | --- |
| Usage | Get | Update | Create | Delete |
| URL resource type | Array object name/array member ID | Array object name/array member ID | Array object name | Array object name/array member ID |

|  | NOTE! PUT and POST differ in: POST is only used to increase the number of array member objects that can be dynamically changed, and the URL only specifies an array name; PUT is used to update the resource object specified by an URL, and the URL specifies an object. The object ID in the PUT method is determined by the client. The object ID in the POST method is assigned by the server as the index of the newly added array object, and the ID is returned to the client via the "CreatedID" field; if the object contains any index variable, then the server will update the index variable in the object of the POST request. An interface that supports POST also supports DELETE. |
| --- | --- |

URL Rules

The following example illustrates the URL composition rules.

/LAPI/V1.0[/Channels/<ID>]/<service-name>/<resource-name>[/<child-resource-name>][/<ID>]][?<key>=<value>]

| Composition | Description |
| --- | --- |
| [ ]: | Indicates parameter (character string or unsigned long). |
| <> | Indicates video input channel resource of a device. |
| <ID> | Indicates video input channel ID (unsigned long). |
| <service-name> | Services are divided into different types, for example, system, media, network, etc. |
| <resource-name> | Includes physical resources and virtual resources. |

The following example illustrates rules for locating multi-channel resources.

/LAPI/V1.0[/Channels/<ID>]/<service-name>/<resource-name>[?ChannelID=<ID>]

| Composition | Description |
| --- | --- |
| [/Channels/<ID>] | Locates resources in the sub domain of a specified channel of the device. |
| [?ChannelID=<ID>] | Locates resources of a specified channel of the device. |

Example code 1

PUT /LAPI/V1.0/System/Time/NTP HTTP/1.1

{

"Enabled":1,

"AddressType":0,

"Address":"24.56.178.140",

"Port":123,

"SynchronizeInterval":60

}

When using PUT/POST method, the HTTP message body usually needs to contain JSON data. The request must be sent to the HTTP/HTTPS port of the device.

Example code 2

HTTP/1.1 200 Ok

{

"Response":

{

"ResponseURL": "URL",

"ResponseCode": 0,

"SubResponseCode": 101,

"ResponseString": "Succeed",

"StatusCode": 0,

“Data”: “null” or {…}

}

}

ResponseURL: Indicates the URL carried in the HTTP request. When the request uses the GET/PUT/DELETE method, the URL is the same as the URI in the request. For the POST request, the URL is the URL of a newly created object. The client can use the URL to directly obtain the new object.

ResponseCode: Indicates the system processing result. 0 means success.

ResponseString: Indicates the analysis of system processing results.

Data: Indicates the response to the request (GET), which in most cases is in JSON format. If the URLs are the same, the data format of the PUT request is the same as the data format of the GET response. The Data item in the following tables lists the data of each API. If the server failed to handle the request, the data is “null”. If the HTTP message uses the PUT/POST method, the data in the response is “null”.

Get structure Info

Please visit www.json.org for more information about JSON. JSON string values can include any Unicode characters (except “\” and control characters).

Functions

This section describes the procedure of calling multiple interfaces. For descriptions about calling interfaces of a single service, refer to Chapter 3.

Alarm Notification

Function Introduction

The upper platform can call the corresponding interfaces to obtain alarm information from devices.

Interface Overview

| Interface | Description |
| --- | --- |
| /LAPI/V1.0/System/Event/Subscription | Subscribe to/unsubscribe from alarm |
| /LAPI/V1.0/System/Event/Subscription/<ID> | Update alarm |
| /LAPI/V1.0/System/Event/Notification/Alarm | Alarm notification |

Interface Calling Procedure

图2-1 Alarm subscription

图2-2 Subscription cancellation

The procedure is described as follows:

Alarm notification

Step 1: The client subscribes to alarms from the device (server). See /LAPI/V1.0/System/Event/Subscription.

|  | NOTE! The parameters carried by this interface represent the client’s IP and port to which the device (server) pushes alarms and the duration of subscription, respectively. |
| --- | --- |

Step 2: The parameters returned by the device (server) represent the subscription identifier (ID), subscribed info (Reference), subscription start time (CurrentTime) and end time (TerminationTime), respectively. The subscription identifier (ID) is used for subsequent alarm update or subscription cancellation.

Step F3: The client periodically sends the update interface to refresh subscription till the subscription end time (TerminationTime).

Step 4: The device (server) returns the updated subscription time.

Step 5: After an alarm occurred, the device pushes the alarm to the client. See /LAPI/V1.0/System/Event/Notification/Alarm

Cancel alarm subscription

Step 1: The client cancels the alarm subscription. See /LAPI/V1.0/System/Event/Subscription/<ID>.

Step 2: The server cancels the subscription and stops pushing alarms.

Search by Image

Function Introduction

The upper platform sends an image with the search range to the device capable of image search. The device returns search results similar to the target in the received image.

Interface Overview

| Interface | Description |
| --- | --- |
| POST /LAPI/V1.0/Smart/SearchByImage/Task | Create a search-by-image task |
| PUT /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Target | Specify search-by-image parameters |
| PUT /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Control | Start a search-by-image task |
| GET /LAPI/V1.0/Smart/SearchByImage/Progress?TaskID=<ID> | Get the progress of a search-by-image task |
| GET /LAPI/V1.0/Smart/SearchByImage/Query?Task=<ID> | Get the result of a search-by-image task |
| DELETE /LAPI/V1.0/Smart/SearchByImage/Task/<ID> | Delete a search-by-image task |

Interface Calling Procedure

Search Pass-through Library

F1: Create a task.

F2: Set search criteria (image, library).

F3: Start analysis task

F4: Get task progress. Proceed to F5 when task execution is completed.

F5: Get paginated search results.

F6: Delete the task.

Search Face Library

F1: Create a task.

F2: Set search criteria (image, library).

F3: Start analysis task

F4: Get task progress. Proceed to F5 when task execution is completed.

F5: Get paginated search results.

F6: Delete the task.

Optional. You can continue using an existing task by updating search criteria (see blue steps above) if the task is executed but not deleted.

Interfaces

System

Time

/LAPI/V1.0/System/Time

| URL | /LAPI/V1.0/System/Time |
| --- | --- |
| Description | Get device time. |
| Method | GET |
| Input Data | None |
| Success Return Data | TimeCfgInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/System/Time |
| --- | --- |
| Description | Set device time. |
| Method | PUT |
| Input Data | TimeCfgInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/System/Time/SyncMode

| URL | /LAPI/V1.0/System/Time/SyncMode |
| --- | --- |
| Description | Get device’s time sync mode. |
| Method | GET |
| Input Data | None |
| Success Return Data | SyncModeInfo |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/System/Time/SyncMode |
| --- | --- |
| Description | Set device’s time sync mode. |
| Method | PUT |
| Input Data | SyncModeInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

/LAPI/V1.0/System/Time/NTP

| URL | /LAPI/V1.0/System/Time/NTP |
| --- | --- |
| Description | Get NTP server info. |
| Method | GET |
| Input Data | None |
| Success Return Data | { “Num”: , “NTPServerInfos”:[< NTPServerInfo>…] } |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/System/Time/NTP |
| --- | --- |
| Description | Set NTP server info. |
| Method | PUT |
| Input Data | { “Num”: , “NTPServerInfos”:[<NTPServerInfo>…] } |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of NTP servers. Currently only one is allowed, so the maximum value of Num is 1. | 1 |
| NTPServerInfos | M | array | NTP server list. |  |
| NTPServerInfo | M | unsigned long | NTP server info. See NTPServerInfo Json Block. |  |

/LAPI/V1.0/System/Time/DST

| URL | /LAPI/V1.0/System/Time/DST |
| --- | --- |
| Description | Get DST info. |
| Method | GET |
| Input Data | None |
| Success Return Data | DSTCfgInfo |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/System/Time/DST |
| --- | --- |
| Description | Set DST info. |
| Method | PUT |
| Input Data | DSTCfgInfo |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

Reboot

/LAPI/V1.0/System/Reboot

| URL | /LAPI/V1.0/System/Reboot |
| --- | --- |
| Description | Restart device. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

Logs

/LAPI/V1.0/System/Logs

| URL | /LAPI/V1.0/System/Logs |
| --- | --- |
| Description | Get logs. |
| Method | POST |
| Input Data | { "Num": "QueryInfos": [<QueryInfo>, <QueryInfo>,...], "Limit": , "Offset": } |
| Success Return Data | { "Total": , "Offset": “Num”:, "Logs":[<Log >, <Log >, ...] } |
| Note | Applicable to NVR and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num(Input Data) | M | unsigned long | Number of search criteria. Must not exceed 16 types. | 2 |
| QueryInfos | C | array | Search criteria info list. Optional when Num is 0. | - |
| QueryInfo | M | Json Block | Search criteria info. | See QueryInfo Json Block. |
| Limit | M | unsigned long | Number of logs to search each time. Max 200. | 100 |
| Offset | M | unsigned long | Sequence number to start search from. The sequence number starts from 0. | 0 |
| Num(Return Data) | M | unsigned long | Number of logs returned for the page | 100 |
| Total | M | unsigned long | Total number of logs | 2850 |
| Logs | C | array | Log info list. Optional when Num is 0. | - |
| Log | M | Json Block | Log info. See Log Json Block. | - |

/LAPI/V1.0/System/AlarmLogs

| URL | /LAPI/V1.0/System/AlarmLogs |
| --- | --- |
| Description | Get alarm logs. |
| Method | POST |
| Input Data | { "Num": "QueryInfos": [<QueryInfo>, <QueryInfo>,...], "Limit": , "Offset": } |
| Success Return Data | { “Total”:, “Offset”:, “Num”:, “AlarmLogs”:[<AlarmLog>, <AlarmLog>…] } |
| Note | Applicable to VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num(Input Data） | M | unsigned long | Number of search criteria. Must not exceed 16 types. | 2 |
| QueryInfos | C | array | Search criteria info list. Optional when Num is 0. | - |
| QueryInfo | M | Json Block | Search criteria info. | See QueryInfo Json Block. |
| Limit | M | unsigned long | Number of logs to search each time. Max 200. | 100 |
| Offset | M | unsigned long | Sequence number to start search from. The sequence number starts from 0. | 0 |
| Num(Return Data) | M | unsigned long | Number of alarm logs returned for the page. | 100 |
| Total | M | unsigned long | Total number of alarm logs. | 2850 |
| AlarmLogs | C | array | Alarm log info list. Optional when Num is 0. | - |
| AlarmLog | M | Json Block | Alarm log info. See AlarmLog Json Block. | - |

DeviceInfo

/LAPI/V1.0/System/DeviceInfo

| URL | /LAPI/V1.0/System/DeviceInfo |
| --- | --- |
| Description | Get device info. |
| Method | GET |
| Input Data | None |
| Success Return Data | DeviceInfo |
| Notes | It is meaningless if the ID in the return value of DeviceInfo is 0. Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/System/DeviceInfo

| URL | /LAPI/V1.0/Channels/<ID>/System/DeviceInfo |
| --- | --- |
| Description | Get info about the device connected to a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | DeviceInfo |
| Notes | Applicable to NVR. |
| Status |  |

/LAPI/V1.0/Channels/System/DeviceInfos

| URL | /LAPI/V1.0/Channels/System/DeviceInfos |
| --- | --- |
| Description | Get device info of all the channels. |
| Method | GET |
| Input Data | None |
| Success Return Data | DeviceInfoList |
| Notes | Applicable to NVR. |
| Status |  |

ChannelInfos

/LAPI/V1.0/Channels/System/ChannelDetailInfos

| URL | /LAPI/V1.0/Channels/System/ChannelDetailInfos |
| --- | --- |
| Description | Get detailed info of all channels. |
| Method | GET |
| Input Data | None |
| Success Return Data | { "Nums": , "DetailInfos": [<DetaiInfo>, <DetaiInfo>, <DetaiInfo>, …] } |
| Note | Applicable to NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Total number of video channels of the device. | 3 |
| DetailInfos | C | Array | Detailed video channel info list. Optional when Num is 0. | - |
| DetaiInfo | M | Json Block | Detailed video channel info. Optional when Nums is 0. | See DetailInfo Json Block. |

Upgrade

/LAPI/V1.0/System/Upgrade

| URL | /LAPI/V1.0/System/Upgrade |
| --- | --- |
| Description | Upgrade the firmware of a device. |
| Method | PUT |
| Input Data | { "UpgradeType": } |
| Success Return Data | { "UpgradeTaskID": } |
| Note | To perform a local upgrade, use /LAPI/V1.0/System/UploadFirmware to upload the firmware version. Use the /LAPI/V1.0/System/UpgradeStatus interface to get upgrade status. Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| UpgradeType | M | unsigned long | Upgrade type: 1: Cloud upgrade 2: By USB drive 3. Local upgrade | 3 |
| UpgradeTaskID | C | string | Upgrade task ID. Used to verify the upgrade task. Required when upgrading NVR locally. | “2b188bd214ae1e2b0d96d5b7636d3bd0@1504663113” |

/LAPI/V1.0/System/UploadFirmware?UpgradeTaskID=<UpgradeTaskID>

| URL | /LAPI/V1.0/System/UploadFirmware?UpgradeTaskID=<UpgradeTaskID> |
| --- | --- |
| Description | Upload firmware version for device upgrade. |
| Method | POST |
| Message body | Accept-Encoding: gzip, deflate Content-Length: 25665652 Content-Type: multipart/form-data; boundary=---------------------------5851473516029 -----------------------------5851473516029 Content-Disposition: form-data; name="UpgradeFile"; filename="Program.bin" Content-Type: application/octet-stream File content -----------------------------5851473516029— |
| Note | Note: The lines started with Content-Dispostion and Content-Type and the blank line are contents of the standard HTTP message body. Use the /LAPI/V1.0/System/UpgradeStatus interface to get upgrade status. Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| UpgradeTaskID | C | string | Upgrade task ID. Used to verify the upgrade task. Required when upgrading NVR locally. Returned by the /LAPI/V1.0/System/Upgrade interface. | “2b188bd214ae1e2b0d96d5b7636d3bd0@1504663113” |

/LAPI/V1.0/System/UpgradeStatus

| URL | /LAPI/V1.0/System/UpgradeStatus |
| --- | --- |
| Description | Get device upgrade status. |
| Method | GET |
| Input Data | None |
| Success Return Data | UpgradeStatusInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/System/Upgrade

| URL | /LAPI/V1.0/Channels/System/Upgrade |
| --- | --- |
| Description | Upgrade firmware of the device connected to a video channel. |
| Method | PUT |
| Input Data | { "Channels":[<ID>,<ID>,…], "UpgradeType": } |
| Success Return Data | { "UpgradeTaskID": } |
| Note | To perform a local upgrade, use /LAPI/V1.0/Channels/System/UploadFirmware?UpgradeTaskID=<UpgradeTaskID> to upgrade the firmware version. Use the /LAPI/V1.0/Channels/System/UpgradeStatuses interface to get upgrade status. Applicable to NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Channels | M | array | Channel ID list. Used only to locally upgrade a device connected to a channel. |  |
| ID | M | unsigned long | Video channel ID. | 1 |
| UpgradeType | M | unsigned long | Upgrade type: 1: Cloud upgrade 2: By USB drive 3. Local upgrade | 1 |
| UpgradeTaskID | C | string | Upgrade task ID. Required when locally upgrading IPC on NVR. | “956087319” |

/LAPI/V1.0/Channels/System/UploadFirmware?UpgradeTaskID=<UpgradeTaskID>

| URL | /LAPI/V1.0/Channels/System/UploadFirmware?UpgradeTaskID=<UpgradeTaskID> |
| --- | --- |
| Description | Upload the firmware version for the device connected to the corresponding channel. |
| Method | POST |
| Message body | Accept-Encoding: gzip, deflate Content-Length: 14231621 Content-Type: multipart/form-data; boundary=---------------------------178201537128716 -----------------------------178201537128716 Content-Disposition: form-data; name="UpgradeFile"; filename="IPC_D1201-B5022P20D1702_JPEG.zip" Content-Type: application/octet-stream File content -----------------------------178201537128716— |
| Success Return Data | None |
| Note | Note: The lines started with Content-Dispostion and Content-Type and the blank line are contents of the standard HTTP message body. Use the /LAPI/V1.0/Channels/System/UpgradeStatus interface to get upgrade status. Applicable to NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| UpgradeTaskID | C | string | Upgrade task ID. Used to verify the upgrade task. Required when locally upgrading IPC on NVR. Returned by the /LAPI/V1.0/Channels/System/Upgrade interface. | “956087319” |

/LAPI/V1.0/Channels/System/UpgradeStatuses

| URL | /LAPI/V1.0/Channels/System/UpgradeStatuses |
| --- | --- |
| Description | Get upgrade status of the device connected to a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | UpgradeStatusInfoList |
| Note | Applicable to NVR. |
| Status |  |

Json Block

DeviceInfo

| DeviceInfo Json Block | { "ID":, "DeviceName":, "DeviceType":, "DeviceCode":, "DeviceModel":, "SerialNumber":, "FirmwareVersion":, "HardwareID":, "UbootVersion": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Video channel ID. | 1 |
| DeviceName | M | string | Device name. | “NVR-B200-E4” |
| DeviceType | C | unsigned long | Device type. 0: IPC 1: NVR 2: DVR 3: VMS Applicable to NVR. | 0 |
| DeviceCode | M | unsigned long | Device ID | 12 |
| DeviceModel | M | string | Device model. Length: 1-63. | “NVR-B200-E4” |
| SerialNumber | M | string | Device serial number. Length: 0-25. | "210235C1MPF158000503" |
| FirmwareVersion | M | string | Firmware version, program version. Length: 0-255. | "R2318P05" |
| HardwareID | M | string | Hardware ID. Length: 0-255. | “A” |
| UbootVersion | M | string | UBOOT version. Length: 0-255. | “V1.4” |

TimeCfgInfo

| TimeCfgInfo Json Block | { "TimeZone": , "DeviceTime": , "DateFormat": , "HourFormat": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TimeZone | M | string | Time zone string. Range: [1,255] "GMT-12:00" "GMT-11:00" "GMT-10:00" "GMT-09:00" "GMT-08:00" "GMT-07:00" "GMT-06:00" "GMT-05:00" "GMT-04:30" "GMT-04:00" "GMT-03:30" "GMT-03:00" "GMT-02:00" "GMT-01:00" "GMT-00:00" "GMT+01:00" "GMT+02:00" "GMT+03:00 "GMT+03:30" "GMT+04:00" "GMT+04:30" "GMT+05:00" "GMT+05:30" "GMT+05:45" "GMT+06:00" "GMT+06:30" "GMT+07:00" "GMT+08:00" "GMT+09:00" "GMT+09:30" "GMT+10:00" "GMT+11:00" "GMT+12:00" "GMT+13:00" | "GMT+08:00" |
| DeviceTime | M | unsigned long | UTC time (unit: second) | 1477104949 |
| DateFormat | C | unsigned long | Date format: 0: YYYY-MM-DD 1: MM-DD-YYYY 2: DD-MM-YYYY Required for NVR. | 0 |
| HourFormat | C | unsigned long | Time format: 0: 12 hour format 1: 24 hour format Required for NVR. | 1 |

DeviceInfoList

| DeviceInfoList Json Block | { “Nums”:, “DeviceInfos”:[<DeviceInfo>] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Number of channel IDs with connected devices. | 1 |
| DeviceInfos | C | array | Device info list. Optional when Nums is 0. |  |
| DeviceInfo | C | Json Block | Device name. Optional. See DeviceInfo Json Block. |  |

DetailInfo

| DetailInfo Json Block | { "ID": , "Name": , "Status": , "IsPoEPort": "PoEStatus": , "StreamNums": , "DeviceType": , "AddressInfo": AddressInfo, "AccessProtocol ": , "OffReason": , "RemoteIndex": , "Manufacturer": , "DeviceModel": , "GBID": , "AddType": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Channel ID. | 1 |
| Name | M | string | Channel name. Range: [0, 63]. | "IP Camera 01" |
| Status | M | unsigned long | Channel status: 0: Offline 1: Online 2: Idle | 0 |
| IsPoEPort | M | unsigned long | Whether is PoE port: 0: No 1: Yes Required by NVR. | 1 |
| PoEStatus | C | unsigned long | PoE port status: 1: Normal 2: Insufficient power 4: The load exceeds the rated power. 8: No need to supply power. Absent when IsPoEPort is 0. | 1 |
| StreamNums | C | unsigned long | Number of media streams supported. Absent when Status is 2. | 3 |
| DeviceType | C | unsigned long | Type of device connected to the channel. 0: Fixed camera 1: PTZ camera 2: Fisheye camera (fisheye lens with dewarping function) 3: Wide-angle lens camera (including fisheye or wide-angle lens but without dewarping function) Required by NVR. Absent when Status is 2. | 0 |
| AddressInfo | C | Json Block | Device address info. Absent when Status is 2. See Addressinfo Json Block |  |
| AccessProtocol | C | unsigned long | Access protocol: 1: ONVIF 2: Private 3: GB/T28181 100: Custom 1 101: Custom 2 102: Custom 3 103: Custom 4 104: Custom 5 105: Custom 6 106: Custom 7 107: Custom 8 108: Custom 9 109: Custom 10 110: Custom 11 111: Custom 12 112: Custom 13 113: Custom 14 114: Custom 15 115: Custom 16 116: Custom 17 117: Custom 18 118: Custom 19 119: Custom 20 120: Custom 21 121: Custom 22 122: Custom 23 123: Custom 24 124: Custom 25 125: Custom 26 126: Custom 27 127: Custom 28 128: Custom 29 129: Custom 30 130: Custom 31 131: Custom 32 Required by NVR. Absent when Status is 2. | 1 |
| OffReason | C | unsigned long | Cause of offline device: 0: Offline (connecting) 1: Online 2: Offline (incorrect username or password) 3: Offline (network disconnection) 4: Offline (failed to set live stream) 5: Offline (failed to start live stream) 6: Offline (media stream interruption) 7: Offline (timeout) 8: Offline (IPC connected via GB/T28181 is waiting for registration packets) 9: Offline (IPC connected via GB/T28181 changed stream transport protocol) 10: Offline (PoE power error) 11: Offline (bandwidth not enough) 12: Offline (access denied for weak password) Required by NVR. Absent when Status is 2. | 1 |
| RemoteIndex | C | unsigned long | Remote channel ID (channel ID of multi-channel device) Required by NVR. Absent when Status is 2. | 1 |
| Manufacturer | C | string | Manufacturer. Range: [0, 31] Required by NVR. Absent when Status is 2. | Uniview |
| DeviceModel | C | String | Device model. Range: [0, 31] Required by NVR. Absent when Status is 2. | IPC342E-DLVIR |
| GBID | C | String | GB/T28181 resource ID. Range: [0, 31] |  |
| AddType | C | unsigned long | Adding type: 0: Add manually 1: Plug and Play Required by NVR. Absent when Status is 2. | 0 |

AddressInfo

| AddressInfo Json Block | { "AddressType": , "Address": , "Port": , "AccessAddress" : } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AddressType | M | unsigned long | IP address type. 0: IPv4 1: IPv6 | 0 |
| Address | M | string | Device IP address. Range: [0,128]. Required by NVR. | 206.5.99.17 |
| Port | M | unsigned long | Device port. Required by NVR. | 80 |
| AccessAddress | M | string | Device access address. Range: [0, 63]. Required for NVR. | http://206.5.99.17 |

SyncModeInfo

| SyncModeInfo Json Block | { "Mode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Time sync mode 0: Sync time with any server 1: Sync time with computer’s system time 2: Sync time with photo server 3: Sync time with NTP server 4: Sync time with management server (non-Onvif) 5: Sync time with management server (Onvif) | 0 |

NTPServerInfo

| NTPServerInfo Json Block | { “Enabled”: , “AddressType”: , “IPAddress”: , “DomainName”: , “Port”: , “SynchronizeInterval”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable NTP server 0: Disable 1: Enable | 0 |
| AddressType | M | unsigned long | Address type 0: IPv4 1: IPv6 (currently unavailable) 2: Domain name (applicable to NVR and VMS). | 0 |
| IPAddress | C | string | NTP server IP address. Length: [0, 64]. Required when AddressType is 0. | 0.0.0.0 |
| DomainName | C | string | NTP server domain name. Length: [0, 64]. Required when AddressType is 2. | pool.ntp.org |
| Port | C | unsigned long | NTP port. Range: [1-65535] Configuration is not supported by IPC. | 123 |
| SynchronizeInterval | M | unsigned long | Sync interval. Range for NVR and VMS: 5/10/15/30 minutes, 1/2/3/6/12h, 1 day, 1 week Range for IPC: 30-3600s All the time needs to be converted to seconds. | 10 |

DSTCfgInfo

| DSTCfgInfo Json Block | { “Enabled”: , “BeginTime”: { “Month”: , “WeekNum”: , “WeekDay”: , “Hour”: }, “EndTime”: { “Month”: , “WeekNum”: , “WeekDay”: , “Hour”: }, “DSTBias”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable DST. 0: Disable 1: Enable | 1 |
| BeginTime | M | Json Block | DST start time | - |
| Month | M | unsigned long | Start month (compliant with ISO8601 standards) 1: January 2: February 3: March 4: April 5: May 6: June 7: July 8: August 9: September 10: October 11: November 12: December | 2 |
| WeekNum | M | unsigned long | Which week of the start month 1: First week 2: Second week 3: Third week 4: Fourth week 5: Last week | 1 |
| WeekDay | M | unsigned long | Which day of the xx week of the start month (compliant with ISO8601 standards) 1: Monday 2: Tuesday 3: Wednesday 4: Thursday 5: Friday 6: Saturday 7: Sunday | 6 |
| Hour | M | unsigned long | Start time (unit: hour, 24 hour format), compliant with ISO8601 standards: 0: 00:00 1: 01:00 2: 02:00 3: 03:00 4:04:00 5: 05:00 6: 06:00 7: 07:00 8: 08:00 9: 09:00 10: 10:00 11: 11:00 12: 12:00 13: 13:00 14:14:00 15: 15:00 16: 16:00 17: 17:00 18: 18:00 19: 19:00 20: 20:00 21: 21:00 22: 22:00 23: 23:00 | 0 |
| EndTime | M | Json Block | DST end time | - |
| DSTBias | M | unsigned long | Time offset (unit: minute). Options: 30, 60, 90, 120 | 60 |

QueryInfo

| QueryInfo Json Block | { "QryType": , "QryCondition": , "QryData": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| QryType | M | unsigned long | Search criteria: 0: User name 1: Organization name 2: Device name 3: Channel name 4: Time (unit: second, UTC format) 5: Service type (equivalent to main type) 6: Operation type (equivalent to sub type) 7: Operation object 8: Alarm type 9: Alarm source name 10: Alarm level 11: Alarm status (acknowledged or not) 12: User who acknowledged the alarm 13: Time when alarm was acknowledged 14: Alarm device ID 15: Alarm channel ID 16: Alarm sub type 17: Server Only Time, Service Type, and Operation Type are applicable to NVR. On VMS, operation logs support search by username, time, service type, operation type, and operation object; alarm logs support search by time, alarm type, alarm source name, alarm level, alarm acknowledgement status/user/time, alarm device ID, alarm channel ID, alarm sub type, and server. | 4 |
| QryCondition | M | unsigned long | Logical type of search criteria. 0: Equal to 1: Greater than 2: Less than 3: Greater than or equal to 4: Less than or equal to 5: Not equal to 6: Fuzzy matching 7: IN GROUP 8: Ascending order 9: Descending order For NVR, only “equal to”, “greater than or equal to”, and “less than or equal to” are applicable. For VMS, all are applicable. | 0 |
| QryData | C | string | Rvalue of search criteria. Alarm types are represented using character strings. When searching by multiple alarm types (up to 16 allowed), separate alarm types by comma. Optional when QryCondition is 6 or 7. NVR only supports search by one type. | "DiskOffline" |

Log

| Log Json Block | { "Time": , "MainType": , "SubType": , "ID": , "LoginName": , "IP": , " DetailInfo": , "OperObject": , "Result": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Time | M | unsigned long | Log occurrence time (unit: second, UTC format) | 1476088399 |
| MainType | M | string | Log main type, See Alarm&Operate Log Type. | "alarm" |
| SubType | M | string | Log sub type, See Alarm&Operate Log Type. | "motionDetectionStart" |
| ID | C | unsigned long | Required when the logs involve a channel ID. 0 means the device itself; from 1 on it indicates a specific channel ID. When involving input/output alarm configuration and alarms, it means input/output alarm ID. For NVR, the ID is channel ID*100+alarm input ID. For example, the channel ID of the device itself is 0. 101 means the first input/output alarm ID of the first channel. For VMS, the ID is configured on the VMS. | 1 |
| LoginName | M | string | Operating user. Length range: [0, 64] | Admin |
| IP | M | string | Operating user’s IP. Length range: [0, 64] | 206.5.3.13 |
| DetailInfo | C | string | Detailed info. Currently only PoE port status is displayed. Length range: [0, 64] Required by NVR. |  |
| OperObject | C | string | Operation object. Length range: [0, 64] Required by VMS. | bsdkabik |
| Result | C | unsigned long | Operation result. Required by VMS. | 1234 |

AlarmLog

| AlarmLog Json Block | { “AlarmID”: , “AlarmType”:, “AlarmSubType”:, “AlarmLevel”:, “ServerID”:, “DevID”:, “ChlID”:, “AlarmSrc”:, “AlarmTime”:, “AlarmChecked”:, “AlarmCheckUser”:, “AlarmCheckTime”:, “AlarmCheckDesc”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AlarmID | M | unsigned long | Alarm log ID. |  |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "alarm" |
| AlarmSubType | O | string | Alarm sub type. See Alarm&Operate Log Type. | "motionDetectionStart" |
| AlarmLevel | M | unsigned long | Alarm level. 0: Critical 1: Major 2: Minor 3: Warning 4: Prompt | 0 |
| ServerID | M | unsigned long | Server ID. | 3 |
| DevID | M | unsigned long | Device ID. | 1 |
| ChlID | M | unsigned long | Channel ID. | 831 |
| AlarmSrc | M | string | Alarm source info. Length range: [1, 128]. | “Camera1” |
| AlarmTime | M | unsigned long | Alarm occurrence time (unit: second, UTC format) |  |
| AlarmChecked | M | boolean | Whether alarm is acknowledged. 0: No 1: Yes | 1 |
| AlarmCheckUser | M | string | User who acknowledged alarm. Length range: [0, 64]. | ”admin” |
| AlarmCheckTime | M | unsigned long | Alarm acknowledgement time (unit: second, UTC format) | 1476088399 |
| AlarmCheckDesc | M | string | Remarks given when acknowledging an alarm. Length range: [0, 516] |  |

Alarm&Operate Log Type

To be added the attachment

AlarmInfo

| AlarmInfo Json Block | { "AlarmType": , "TimeStamp": , "AlarmSeq": , "AlarmSrcID": , "AlarmSrcType":, "AlarmSrcName": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | “MotionAlarmOn” |
| TimeStamp | M | unsigned long | Alarm time. | 1489040894 |
| AlarmSeq | C | unsigned long | Alarm sequence number. Applicable to VMS and IPC. | 327 |
| AlarmSrcID | C | unsigned long | Resource ID. Applicable to NVR and VMS. For NVR, the following rules are applicable: 1. AlarmSrcType is 0-7, AlarmSrcID indicates HDD index. 2. AlarmSrcType is 8, AlarmSrcID is video channel ID. 3. AlarmSrcType is 9, AlarmSrcID is alarm input channel ID. 4.AlarmSrcType is 10, AlarmSrcID is 0 by default and indicates the system itself. | 860 |
| AlarmSrcType | C | unsigned long | Type of alarm source: 0: Local HDD 1: Disk enclosure-1 HDD 2: Disk enclosure-2 HDD 3: RAID HDD 4: NAS HDD 5: SAN HDD 6: eSATA HDD 7: SD card 8: Video channel 9: Alarm input channel 10: System Applicable to NVR. | 2 |
| AlarmSrcName | C | string | Alarm source name. Length range: [0, 128]. Applicable to VMS. | “206.9.252.7_V_6” |

ResInfo

| ResInfo Json Block | { "Reference": , "Action":, "Num":, "ResInfos": [{ "ResType": , "ResID": }, ……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "http://192.168.0.13:80/LAPI/V1.0/System/ALarmEvent/CommonAlarm/<SubscribersID>" |
| Action | M | unsigned long | Sub event type of action to change resource: 0: Add 1: Delete 2: Edit 3: Online 4: Offline 5: Alarm notification by E-map | 0 |
| Num | M | unsigned long | Number of changed resources. | 5 |
| ResInfos | M | array | Info about resource data changes. |  |
| ResType | M | unsigned long | Resource type. 0: User resource 1: Device resource 2: Channel resource 3: User permission resource 4: Sequence resource 5: Hotspot resource 6: Hot zone resource 7: Alarm resource 8: Time template 9: System permission resource 10: Device permission resource 11: Organization resource 12: Alarm task resource 13: Primary/replica resource 0xff: Invalid resource type | 2 |
| ResID | M | unsigned long | Resource ID. | 1206 |

VMSCommon

| VMSCommon Json Block | { "OrgID": , "DevID": , "DevName": , "ChlIndex": , "Permission": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| OrgID | M | unsigned long | Organization ID. | 1 |
| DevID | M | unsigned long | Parent device ID. | 1 |
| DevName | M | string | Parent device name. Length range: [1, 256] | "123" |
| ChlIndex | M | unsigned long | Channel index. | 1 |
| Permission | M | unsigned long | Channel permission. | 214 |

UpgradeStatusInfo

| UpgradeStatusInfo Json Block | { "ID": "Status" :, "Percent": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | O | unsigned long | The value is meaningless in /LAPI/V1.0/System/UpgradeStatus interface. The default is 0. In /LAPI/V1.0/Channels/System/UpgradeStatuses, the value is the video channel ID. | 1 |
| Status | M | unsigned long | Upgrade status. Currently these statuses are supported: 0: Initializing 1: Searching 2: Downloading 3: Transferring 4: Upgrading 5: Upgrade error 6: Already the latest 7: Higher version is available 8: Upgrade completed 9: Failed to get upgrade progress 10: Version does not match 11: Cannot load multiple upgrade processes simultaneously 12: Timeout occurred while getting upgrade progress | 2 |
| Percent | C | unsigned long | Progress of the statuses. Range: [0,100]. Required when Status is 2, 3 or 4. | 50 |

UpgradeStatusInfoList

| UpgradeStatusInfoList Json Block | { "Nums":, "UpgradeStatuses":[<UpgradeStatusInfo>,<UpgradeStatusInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Number of IPC channels. | 1 |
| UpgradeStatuses | C | array | Status info list. Required when Nums is greater than 0. | - |
| UpgradeStatusInfo | C | Json Block | Status info. Required when Nums is greater than 0. | See UpgradeStatusInfo Json Block. |

PTZ

PTZCtrl

/LAPI/V1.0/Channels/<ID>/PTZ/PTZCtrl

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/PTZCtrl |
| --- | --- |
| Description | Operate PTZ of a channel. |
| Method | PUT |
| Input Data | PTZCommandInfo |
| Success Return Data | None |
| Note | For IPC, the ID is always 0. Applicable to IPC, NVR, and VMS. |
| Status |  |

Presets

/LAPI/V1.0/Channels/<ID>/PTZ/Presets

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Presets |
| --- | --- |
| Description | Get PTZ presets of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | PresetInfoList |
| Note | For IPC, the ID is always 0. Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Presets |
| --- | --- |
| Description | Add a PTZ preset for a video channel. |
| Method | POST |
| Input Data | PresetInfo |
| Success Return Data | None |
| Note | For IPC, the ID is always 0. Applicable to IPC and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/Presets/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Presets/<ID> |
| --- | --- |
| Description | Edit a PTZ preset of a video channel. |
| Method | PUT |
| Input Data | PresetInfo |
| Success Return Data | None |
| Note | For IPC, the ID of the channels is always 0. Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Presets/<ID> |
| --- | --- |
| Description | Delete a PTZ preset of a video channel. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | For IPC, the ID of the channels is always 0. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/Presets/<ID>/Goto

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Presets/<ID>/Goto |
| --- | --- |
| Description | Rotate PTZ of a channel to a preset. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | For IPC, the ID of the channels is always 0. Applicable to IPC, NVR, and VMS. |
| Status |  |

Capabilities

/LAPI/V1.0/Channels/<ID>/PTZ/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Capabilities |
| --- | --- |
| Description | Get PTZ capability of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | PTZCapabilityInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

AreaZoom

/LAPI/V1.0/Channels/<ID>/PTZ/AreaZoomIn

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/AreaZoomIn |
| --- | --- |
| Description | Perform drag-to-zoom-in to a channel. |
| Method | PUT |
| Input Data | PTZZoomAreaInfo |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/AreaZoomOut

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/AreaZoomOut |
| --- | --- |
| Description | Perform drag-to-zoom-out to a channel. |
| Method | PUT |
| Input Data | PTZZoomAreaInfo |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

Patrols

/LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID>/Start

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID>/Start |
| --- | --- |
| Description | Start a patrol route of a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of Patrols is the patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID>/Stop

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID>/Stop |
| --- | --- |
| Description | Stop a patrol route of a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of Patrols is the patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID> |
| --- | --- |
| Description | Get a patrol route of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | PatrolInfo |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID> |
| --- | --- |
| Description | Edit a patrol route of a channel. |
| Method | PUT |
| Input Data | PatrolInfo |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols/<ID> |
| --- | --- |
| Description | Delete a patrol route of a channel. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of Patrols is the patrol route ID. Applicable to IPC and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/Patrols

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols |
| --- | --- |
| Description | Get all preset patrol routes of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | PatrolInfoList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols |
| --- | --- |
| Description | Edit preset patrol routes of a channel in batches. |
| Method | PUT |
| Input Data | PatrolInfoList |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/Patrols |
| --- | --- |
| Description | Add a preset patrol route for a channel. |
| Method | POST |
| Input Data | PatrolInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/Start

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/Start |
| --- | --- |
| Description | Start a recorded patrol route of a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of RecordedPatrols is the recorded patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/Stop

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/Stop |
| --- | --- |
| Description | Stop a recorded patrol route of a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of RecordedPatrols is the recorded patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/StartRecord

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/ RecordedPatrols /<ID>/StartRecord |
| --- | --- |
| Description | Start recording a patrol route for a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of RecordedPatrols is the recorded patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/StopRecord

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>/StopRecord |
| --- | --- |
| Description | Stop recording a patrol route for a channel. |
| Method | PUT |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of RecordedPatrols is the recorded patrol route ID. Applicable to IPC, NVR, and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols/<ID> |
| --- | --- |
| Description | Delete a recorded patrol route of a channel. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | The corresponding ID of RecordedPatrols is the recorded patrol route ID. Applicable to IPC and VMS. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols

| URL | /LAPI/V1.0/Channels/<ID>/PTZ/RecordedPatrols |
| --- | --- |
| Description | Get all the recorded patrol routes of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | RecordedPatrolInfoList |
| Note | Applicable to IPC and NVR. |
| Status |  |

Json Block

PTZCommandInfo

| PTZCommandInfo Json Block | { "PTZCmd":, "Para1":, "Para2":, "Para3": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PTZCmd | M | unsigned long | PTZ operation commands: 0x0101: Stop closing aperture 0x0102: Close aperture 0x0103: Stop opening aperture 0x0104: Open aperture 0x0201: Stop focusing near 0x0202: Focus near 0x0203: Stop focusing far 0x0204: Focus far 0x0301: Stop zooming in 0x0302: Zoom in 0x0303: Stop zooming out 0x0304: Zoom out 0x0402: Up 0x0404: Down 0x0502: Right 0x0504: Left 0x0702: Up left 0x0704: Down left 0x0802: Up right 0x0804: Down right 0x0901: Stop 0x0A01: Enable wiper 0x0A02: Disable wiper 0x0B01: Enable light 0x0B02: Disable light 0x0C01: Enable heater 0x0C02: Disable heater 0x0D01: Enable IR 0x0D02: Disable IR | 1794（0x0702） |
| Para1 | C | unsigned long | PTZ command parameter 1, pan speed. Range: [1,9]. Only for use when PTZCmd is 0x402 – 0x804. When turning up (0x402) or turning down (0x404), the pan speed is 0. Optional when PTZCmd is not in the range 0x402–0x804. | 2 |
| Para2 | C | unsigned long | PTZ command parameter 2, tilt speed. Range: [1,9]. Only for use when PTZCmd is 0x402 – 0x804. When turning left (0x504) or turning right (0x502), the tilt speed is 0. Optional when PTZCmd is not in the range 0x402–0x804. | 2 |
| Para3 | C | unsigned int | PTZ command parameter 3. Currently not in use. Enter 0. | 0 |

PresetInfoList

| PresetInfoList Json Block | { "Nums": , "PresetInfos": [<PresetInfo>,<PresetInfo>,......] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Number of presets. Max 255. | 3 |
| PresetInfos | C | array | Preset list. Optional when Nums is 0. |  |
| PresetInfo | C | Json Block | Preset info. See PresetInfo Json Block. Optional when Nums is 0. |  |

PresetInfo

| PresetInfo Json Block | { "ID":, "Name": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Preset ID, which is generated by the device when the preset is added. Starts from 1. Max 255. | 1 |
| Name | M | string | Preset name. Length: [1,64]. For NVR, the preset name is the default value and is not configurable. | "Preset1" |

PTZCapabilityInfo

| PTZCapInfo Json Block | { "IsSupportPTZ": , "IsSupportRecordPatrol": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupportPTZ | M | boolean | Whether supports PTZ: 0: No 1: Yes | 0 |
| IsSupportRecordPatrol | M | boolean | Whether supports recorded patrol: 0: No 1: Yes | 1 |
| Remarks: Other parameters will not be released temporarily. |  |  |  |  |

PTZZoomAreaInfo

| PTZZoomAreaInfoList Json Block | { "CenterPointX":, "CenterPointY":, "LengthX":, "LengthY":, "Width": , "Height": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| CenterPointX | M | unsigned long | x-coordinate of the center point of the zoomed area. Must not be greater than Width. | 500 |
| CenterPointY | M | unsigned long | y-coordinate of the center point of the zoomed area. Must not be greater than Height. | 500 |
| LengthX | M | unsigned long | Length of the zoomed area | 0 |
| LengthY | M | unsigned long | Width of the zoomed area | 0 |
| Width | M | unsigned long | Window length | 1300 |
| Height | M | unsigned long | Window height | 1300 |

PatrolInfoList

| PatrolInfoList Json Block | { "Num": , "PatrolInfos": [<PatrolInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of existing patrol routes. | 4 |
| PatrolInfos | C | array | Patrol route info. |  |

PatrolInfo

| PatrolInfo Json Block | { "ID":, "Name": , "Num": , "Actions": [<Action>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Patrol route ID (starts from 0). Set by using the retrieved ID. | 0 |
| Name | C | string | Patrol route name. Length range: [1,64]. Applicable to IPC and VMS. |  |
| Num | M | unsigned long | Number of actions while operating PTZ | 2 |
| Actions | M | array | PTZ operation action info. See Action Json Block. |  |

Action

| Action Json Block | { "ID":, "Type":, "Para1: , "Para2: , "Para3: , "PresetID":, "Duration": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Starts from 0, up to (Num-1) in PatrolInfo. Applicable to NVR and VMS. | 0 |
| Type | M | unsigned long | Action type: 0x0302: Zoom to 0x0402: Turn up 0x0404: Turn down 0x0502: Turn right 0x0504: Turn left 0x0602: Go to preset 0x0702: Turn up left 0x0704: Turn down left 0x0802: Turn up right 0x0804: Turn down right 0x0901: Stop NVR and VMS only support “Go to preset”. | 0 |
| Para1 | C | unsigned long | PTZ command parameter 1, pan speed. Range: [1,9]. Only for use when PTZCmd is 0x402 – 0x804. When turning up (0x402) or turning down (0x404), the pan speed is 0. Optional when PTZCmd is not in the range 0x402–0x804. Used by preset speed. | 5 |
| Para2 | C | unsigned long | PTZ command parameter 2, tilt speed. Range: [1,9]. Only for use when PTZCmd is 0x402 – 0x804. When turning left (0x504) or turning right (0x502), the tilt speed is 0. Required when PTZCmd is 0x402 or 0x404. When PTZCmd is in the range 0x702-0x804, use Para1; Para2 data is invalid. | 5 |
| Para3 | C | unsigned long | Lens’ zoom ratio*100 Required when Type is 0x302 or 0x304. | 2 |
| PresetID | C | unsigned long | Preset ID. Optional when Type is not 0x602. | 1 |
| Duration | C | unsigned long | Action duration (unit: millisecond) when the action type is not “Preset”. 0 means keeping the action. Range: [0,1800000]. NVR and VMS only supports prevision to seconds. For NVR and VMS, the value must be set to seconds in integer and then converted to milliseconds. | 10 |

RecordedPatrolInfoList

| RecordedPatrolInfoList Json Block | { "Num": , "RecordedPatrolInfos": [<RecordedPatrolInfo>] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of recorded patrol routes. | 1 |
| RecordedPatrolInfos | C | array | Info about recorded patrol route. |  |

RecordedPatrolInfo

| RecordedPatrolInfo Json Block | { "ID": , "Name": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Recorded patrol route ID. Set by using the retrieved ID. | 1 |
| Name | C | string | Name of recorded patrol route. Applicable to VMS. | "PatrolInfo1" |

Media

Capabilities

/LAPI/V1.0/Channels/<ID>/Media/Video/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Capabilities |
| --- | --- |
| Description | Get video capability of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | VideoCapabilityInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Media/Audio/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Media/Audio/Capabilities |
| --- | --- |
| Description | Get audio capability of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | AudioCapabilityInfo |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Media/Snapshot/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Media/Snapshot/Capabilities |
| --- | --- |
| Description | Get snapshot capability of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | SnapshotCapabilities |
| Note | Applicable to IPC. |
| Status |  |

Video

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/DetailInfos

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/DetailInfos |
| --- | --- |
| Description | Get all video streams of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | VideoStreamInfoList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/DetailInfos |
| --- | --- |
| Description | Set all video streams of a video channel. |
| Method | PUT |
| Input Data | VideoStreamInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Media/Video/Mode

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Mode |
| --- | --- |
| Description | Get image collection mode. |
| Method | GET |
| Input Data | None |
| Success Return Data | VideoModeInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Mode |
| --- | --- |
| Description | Set image collection mode. |
| Method | PUT |
| Input Data | VideoModeInfo |
| Success Return Data | None |
| Note | Applicable to NVR and IPC. |
| Status |  |

Audio

/LAPI/V1.0/Media/Audio/Input

| URL | /LAPI/V1.0/Media/Audio/Input |
| --- | --- |
| Description | Get audio input parameters. |
| Method | GET |
| Input Data | None |
| Success Return Data | AudioInputCfg |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Media/Audio/Input |
| --- | --- |
| Description | Set audio input parameters. |
| Method | PUT |
| Input Data | AudioInputCfg |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Media/Audio/Output

| URL | /LAPI/V1.0/Media/Audio/Output |
| --- | --- |
| Description | Get audio output parameters. |
| Method | GET |
| Input Data | None |
| Success Return Data | AudioOutputCfg |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Media/Audio/Output |
| --- | --- |
| Description | Set audio output parameters |
| Method | PUT |
| Input Data | None |
| Success Return Data | AudioOutputCfg |
| Note | Applicable to IPC and NVR. |

LiveStreamURL

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/LiveStreamURL?TransType=<TransType>&TransProtocol=<TransProtocol>

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/LiveStreamURL?TransType=<TransType>&TransProtocol=<TransProtocol> |
| --- | --- |
| Description | Get the live video URL of a video stream of a video channel. |
| Method | GET |
| Success Return Data | { "LoginName":, "PIN":, "URL": } |
| Note | After getting the URL, use RTSP to get media streams. For IPC, the ID of the channels is always 0. Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Streams/<ID> | M | unsigned long | Video stream index number, which starts from 0. | 0 |
| TransType | O | unsigned long | Stream transport type 0: Forward 1: Direct connection Applicable to VMS. Default: Forward | 0 |
| TransProtocol | C | unsigned long | Transport protocol type 0: TCP 1: UDP Applicable to VMS. Used with TransType. | 1 |
| LoginName | C | string | Device username encrypted using private algorithm. A decryption library is provided. Required when Transtype is 1. | sdfsdf |
| PIN | C | string | Device password encrypted using private algorithm. A decryption library is provided. Required when Transtype is 1. | sdfsdfjskdsdfsdfjskdsdfsdfjs |
| URL | M | string | Live video URL of the corresponding video stream. | rtsp://192.168.0.13:554/media/video1 |

Records

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/Records?Begin=<Begin>&End=<End>&Limit=<Limit>&Offset=<Offset>&Types=<Types>&RelationOfTypes=<RelationOfTypes>&Position=<Position>

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/Records?Begin=<Begin>&End=<End>&Limit=<Limit>&Offset=<Offset>&Types=<Types>&RelationOfTypes=<RelationOfTypes>&Position=<Position> |
| --- | --- |
| Description | Get recording files of a stream of a video channel during a time period. |
| Method | GET |
| Input Data | None |
| Success Return Data | RecordList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Ranger | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Begin | M | unsigned long | Search start time (unit: second, UTC format) | 1454428800 |
| End | M | unsigned long | Search end time (unit: second, UTF format) | 1454435960 |
| Limit | C | unsigned long | Number of recordings to search. Range: [0, 100] Applicable to IPC only. | 100 |
| Offset | C | unsigned long | Specifies from which one to start searching. Applicable to IPC only. | 100 |
| Types | 0 | unsigned long | Type of recording to search. Described by bit order, from left to right are bit 0, bit 1, bit 2 … bit 31, each representing a recording type. A recording type is valid when the corresponding bit is 1. 0: Manual recording (this recording type must not be mixed with other recording types and therefore are expressed using all 0s). BIT0: Common alarm (currently not applicable to NVR) BIT1: Motion detection BIT2: Input switch alarm BIT3: Video loss BIT4: Audio detection BIT5: POS info (currently not applicable to NVR) BIT6: Scheduled recording BIT7: Face detection BIT8: Cross line detection BIT9: Intrusion detection All types of recordings will be searched when this field absent. Applicable to IPC and NVR. | Search motion detection recording: 00000000000000000000000000000100 = 4, so fill this field with 4. Search motion detection recording and face detection recording: 00000000000000000000000100000100 = 260, so fill this field with 260. |
| RelationOfTypes | C | unsigned long | Relationship between recording types: 0: AND 1: OR Optional when Types is absent or when involving one recording type only. Required when involving two or more recording types. Applicable to NVR only. | 1 |
| Position | 0 | unsigned long | Location of recording to search: 1: Local recording 2: Other device Applicable to VMS only. | 1 |

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/RecordURL？Begin=<Begin>&End=<End>&Types=<Types>&RelationOfTypes=<RelationOfTypes>&Position=<Position>&SessionID=<SessionID>&TransType=<TransType>

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/RecordURL?Begin=<Begin>&End=<End>&Types=<Types>&RelationOfTypes=<RelationOfTypes>&Position=<Position>&SessionID=<SessionID>&TransType=<TransType> |
| --- | --- |
| Description | Get the URL of certain types of recordings of a video channel during a time period. |
| Method | GET |
| Input Data | None |
| Success Return Data | { "URL": , } |
| Note | The returned URL is only used for RTSP-based playback and download. Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Begin | O | unsigned long | Recording start time (unit: second, UTC format) End time is required if there is a start time. | 1454428800 |
| End | O | unsigned long | Recording end time (unit: second, UTF format) Start time is required if there is an end time. | 1454435960 |
| Types | O | unsigned long | Type of recording to search. Described by bit order, from left to right are bit 0, bit 1, bit 2 … bit 31, each representing a recording type. A recording type is valid when the corresponding bit is 1. 0: Manual recording (this recording type must not be mixed with other recording types and therefore are expressed using all 0s). BIT0: Common alarm (currently not applicable to NVR) BIT1: Motion detection BIT2: Input switch alarm BIT3: Video loss BIT4: Audio detection BIT5: POS info (currently not applicable to NVR) BIT6: Scheduled recording BIT7: Face detection BIT8: Cross line detection BIT9: Intrusion detection All types of recordings will be searched when this field absent. | Search motion detection recording: 00000000000000000000000000000100 = 4, so fill this field with 4. Search motion detection recording and face detection recording: 00000000000000000000000100000100 = 260, so fill this field with 260. |
| RelationOfTypes | 0 | unsigned long | Relationship between recording types: 0: AND 1: OR Optional if Types is absent or when involving one recording type only. Required when involving two or more recording types. | 1 |
| Position | 0 | unsigned long | Location of recording to play: 1: Local recording 2: Other device Currently applicable to VMS only. | 1 |
| SessionID | 0 | unsigned long | It is recommended that each playback stream has one corresponding session ID. Currently applicable to VMS only. | 1 |
| TransType | 0 | unsigned long | 0: Forward 1: Direct connection | 0 |
| URL | M | string | Recording URL. Length range: [0,128]. | rtsp://206.5.3.13:554/c2/replay/ |

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/Records/DailyDistribution?Year=<Year>&Month=<Month>&Position=<Position>

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/Records/DailyDistribution?Year=<Year>&Month=<Month>&Position=<Position> |
| --- | --- |
| Description | Get the distribution of a channel's recordings of a certain month (in unit of days). |
| Method | GET |
| Input Data | None |
| Success Return Data | { "Nums": , "DailyStatuses": [<Status>, <Status>, <Status>, <Status>…] } |
| Note | Applicable to NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Year | M | unsigned long | Year. | 2016 |
| Month | M | unsigned long | Month, starts from 1. | 9 |
| Position | 0 | unsigned long | Location of recording to search: 1: Local recording 2: Other device Currently applicable to VMS only. | 1 |
| Nums | M | unsigned long | Actual number of days in the month. | 30 |
| DailyStatuses | C | Array | Daily recording status list. Optional when Nums is 0. |  |
| Status | M | unsigned long | Recording status: 0: No recording 1: Event recording 2: Normal recording Event recording has higher priority level than normal recording. | 1 |

/LAPI/V1.0/Channels/Media/Video/Streams/RecordFileSize

| URL | /LAPI/V1.0/Channels/Media/Video/Streams/RecordFileSize |
| --- | --- |
| Description | Get the size of recording files of a certain period. |
| Method | POST |
| Input Data | RecordChannelInfoList |
| Success Return Data | RecordFileList |
| Note | Applicable to NVR. |
| Status |  |

Talk

/LAPI/V1.0/Channels/<ID>/Media/Talk

| URL | /LAPI/V1.0/Channels/<ID>/Media/Talk |
| --- | --- |
| Description | Get audio URL for two-way audio with a video channel. |
| Method | GET |
| Success Return Data | { "URL": } |
| Note | After getting the URL, use RTSP to get audio streams for two-way audio. For IPC, the ID of the channels is always 0. Applicable to IPC and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| URL | M | string | Corresponding live video stream URL of an audio stream. | rtsp://192.160.1.22:554/media/video1 |

OSD

/LAPI/V1.0/Channels/<ID>/Media/OSDs/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs/Capabilities |
| --- | --- |
| Description | Get OSD capability of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | OSDCapabilities |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Media/OSDs/ContentStyle

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs/ContentStyle |
| --- | --- |
| Description | Get the OSD content style of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | OSDContentStyle |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs/ContentStyle |
| --- | --- |
| Description | Set the OSD content style of a channel. |
| Method | PUT |
| Input Data | OSDContentStyle |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Media/OSDs/Contents

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs/Contents |
| --- | --- |
| Description | Get OSD contents. |
| Method | GET |
| Input Data | None |
| Success Return Data | OSDContent |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs/Contents |
| --- | --- |
| Description | Set OSD contents. |
| Method | PUT |
| Input Data | OSDContent |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Media/OSDs

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs |
| --- | --- |
| Description | Get all OSD contents. |
| Method | GET |
| Input Data | None |
| Success Return Data | OSDList |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/OSDs |
| --- | --- |
| Description | Set all OSD contents. |
| Method | PUT |
| Input Data | OSDList |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

Snapshot

/LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/Snapshot

| URL | /LAPI/V1.0/Channels/<ID>/Media/Video/Streams/<ID>/Snapshot |
| --- | --- |
| Description | Snapshot without preview. |
| Method | GET |
| Input Data | None |
| Success Return Data | Content-Length: 123131 Content-Type: image/jpeg Accept-Ranges: bytes Image data |
| Note | Applicable to IPC and NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Channels/<ID> | M | unsigned long | Video channel ID. | 1 |
| Streams/<ID> | M | unsigned long | The ID is the stream index (snapshot resolution of IPC is determined by snapshot configuration and so the index is invalid). 0: Main stream 1: Sub stream 2: Third stream | 0 |

/LAPI/V1.0/Channels/<ID>/Alarm/SnapshotURL?Type=<Type>&Time=<Time>

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/SnapshotURL?Type=<Type>&Time=<Time> |
| --- | --- |
| Description | Get alarm snapshot URL. |
| Method | GET |
| Input Data | None |
| Success Return Data | SnapshotURLInfo |
| Note | Applicable to NVR and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Video channel ID. | 1 |
| Type | M | string | Alarm type. Currently three types available: "MotionAlarmOn": Motion detection started "VideoLossAlarmOn": Video loss started "AbnormalAudioOn": Audio detection started | "MotionAlarmOn" |
| Time | M | unsigned long | Alarm occurrence time (UTC). Number of seconds since 0’o clock, Jan. 1, 1970 | 1476088399 |

/LAPI/V1.0/IO/InputSwitches/<ID>/Alarm/SnapshotURL?Time=<Time>

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/Alarm/SnapshotURL?Time=<Time> |
| --- | --- |
| Description | Get the URL of alarm snapshots triggered by an input switch. |
| Method | GET |
| Input Data | None |
| Success Return Data | SnapshotURLInfo |
| Note | Applicable to NVR and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Alarm input ID. For NVR, the ID is channel ID*100+alarm input ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |
| Time | M | unsigned long | Alarm occurrence time (UTC). Number of seconds since 0’o clock, Jan. 1, 1970. | 1476088399 |

/LAPI/V1.0/Channels/<ID>/Alarm/Snapshot?Name=<Name>&Size=<Size>

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/Snapshot?Name=<Name>&Size=<Size> |
| --- | --- |
| Description | Get alarm snapshot. |
| Method | GET |
| Input Data | None |
| Success Return Data | Content-Length: 44 Content-type: image/jpeg Accept-Ranges: bytes Image data |
| Note | Name and Size are returned by the interface used to get alarm image URLs. Applicable to NVR and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | ID of the corresponding channel. | 1 |
| Name | M | string | Image name. Length: [1,64] | “aaa” |
| Size | M | unsigned long | Image size. Unit: Byte. | 61440 |

/LAPI/V1.0/Channels/<ID>/Media/Snapshot/Info

| URL | /LAPI/V1.0/Channels/<ID>/Media/Snapshot/Info |
| --- | --- |
| Description | Get snapshot parameters of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | SnapshotInfo |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Media/Snapshot/Info |
| --- | --- |
| Description | Set snapshot parameters of a channel. |
| Method | PUT |
| Input Data | SnapshotInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

Json Block

AudioCapabilityInfo

| AudioCapabiliInfo Json Block | { "AudioInNum": , "AudioInModeInfoList": [<AudioModeInfo>, ...], "AudioInEncodeFormatNum":, "AudioInEncodeFormatInfoList": [<EncodeFormatInfo>,...] "SerialInNum": , "SerialInModeInfoList":[<AudioModeInfo>, ...] "SerialInEncodeFormatNum": , "SerialInEncodeFormatInfoList":[<EncodeFormatInfo>, ...] "AudioOutTypeNum": , "AudioOutTypeInfoList":[…] , "SupportAudioOutGain":, "SupportAlarmAudioOutGain": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AudioInNum | M | unsigned long | Number of audio inputs supported. | 2 |
| AudioInModeInfoList | C | array | Audio input mode list | --- |
| AudioModeInfo | M | Json Block | Audio input mode. See AudioModeInfo Json Block | --- |
| AudioInEncodeFormatNum | M | unsigned long | Number of audio input encoding formats supported. | 3 |
| AudioInEncodeFormatInfoList | C | array | List of supported audio input encoding formats | --- |
| EncodeFormatInfo | M | Json Block | Info about supported encoding formats. See EncodeFormatInfo Json Block | --- |
| SerialInNum | M | unsigned long | Number of serial inputs supported. Used to connect external digital sound pick-up. | 1 |
| SerialInModeInfoList | C | array | Serial input mode list | --- |
| SerialInEncodeFormatNum | M | unsigned long | Number of encoding formats supported by serial input |  |
| SerialInEncodeFormatInfoList | M | array | List of encoding formats supported by serial input. See EncodeFormatInfo Json Block |  |
| AudioOutTypeNum | M | unsigned long | Number of audio output device types supported: 0 means audio output parameters are not configurable. | 2 |
| AudioOutTypeInfoList | C | array | Audio output device type: 0: LINE 1: MIC 2: RS485 digital sound pick-up 3: Speaker Optional when AudioOutTypeNum is 0 | [0, 1] |
| SupportAudioOutGain | C | boolean | Whether the device supports the configuration of audio gain 0: No 1: Yes When absent, it means the capability is not available. | 1 |
| SupportAlarmAudioOutGain | C | boolean | Whether the device supports the configuration of alarm output gain Present when supported by the device. (If this field is supported, it means audio output gain can be configured separately) | 1 |

AudioModeInfo

| AudioModeInfo Json Block | { "Channel":, "ModeNum":, "ModeList": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Channel | M | unsigned long | Audio channel ID | 2 |
| ModeNum | M | unsigned long | Number of audio modes | 2 |
| ModeList | C | array | Supported audio input modes. Optional when ModeNum is 0. 0: LINE 1: MIC 2: RS485 digital sound pick-up | [0, 1] |

AudioInputCfg

| AudioInputCfg Json Block | { "IsMute":, "Type":, "EncodeFormat ":, "SampleRate":, "InputGain":, "NoiseReduction": { "Enabled":, "Mode":, "Strength": } "AudioInputNum": "AudioInputList": [<AudioInput>,<AudioInput>,......], "SerialInputNum": "SerialIInputList": [<AudioInput>......], } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsMute | M | boolean | Whether is muted. 0: No 1: Yes | 0 |
| Type | M | unsigned long | Audio collection port type. 0: Audio input 1: Serial port |  |
| EncodeFormat | M | unsigned long | Audio encoding format. See EncodeFormatInfo. | 2 |
| SampleRate | M | unsigned long | Audio sampling rate. See EncodeFormatInfo. | 0 |
| InputGain | M | unsigned long | Audio gain. Range: [0,255]. | 128 |
| NoiseReduction | M | Json Block | Noise reduction info. | 1 |
| Enabled | M | unsigned long | Enable or disable noise reduction. 0: Disable 1: Enable | 0 |
| Mode | 0 | unsigned long | Reserved field. |  |
| Strength | O | unsigned long | Reserved field. | 1 |
| AudioInputNum | M | unsigned long | Number of audio inputs. |  |
| AudioInputList | C | array | Audio input list. Required when Type is 0. |  |
| AudioInput | M | Json Block | Audio input channel info See AudioInput Json Block. |  |
| SerialIInputNum | M | unsigned long | Number of serial inputs. | 1 |
| SerialInputList | C | array | Serial audio input list. Required when Type is 1. |  |

AudioInput

| AudioInput Json Block | { "ID":, "Enabled":, "Mode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Audio channel ID, starts from 1. | 1 |
| Enabled | M | unsigned long | Enable or disable audio input channel. | 1 |
| Mode | M | unsigned long | Audio input mode. 0: LINE 1: MIC 2: RS485 digital sound pick-up | 0 |

AudioOutputCfg

| AudioOutputCfg Json Block | { "Type":, "Gain":, "AlarmGain": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Audio output device type. See AudioCapabilityInfo. |  |
| Gain | C | Unsigned long | Audio output gain Range: [0,255]. Required when SupportAudioOutGain is 1. | 242 |
| AlarmGain | C | unsigned long | Alarm audio gain. Range: [0,255] Required when SupportAlarmAudioOutGain is 1. | 255 |

ChannelRecordInfo

| ChannelRecordInfo Json Block | { "ChannelID": , "StreamID": , "Begin": , "End": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ChannelID | M | unsigned long | Video channel ID of recordings to search. | 1 |
| StreamID | M | unsigned long | Stream ID of requested recordings. 0: Main stream 1: Sub stream 2: Third stream | 0 |
| Begin | M | unsigned long | Start time of recording file (unit: second, UTC format) | 1454428800 |
| End | M | unsigned long | End time of recording file (unit: second, UTC format) | 1454435960 |

EncodeFormatInfo

| EncodeFormatInfo Json Block | { "Type": , "Num": , "SampleList": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Audio encoding format 0 : MPEG1 1: G.711A 2: G.711U 3 : ADPCM 4: OGG 5: Auto 6: AAC-LC | 2 |
| Num | M | unsigned long | Number of supported sampling rates | 3 |
| SampleList | M | array | Sampling rate list 0: 8KHz 1: 16KHz 2: 22.05KHz 3: 24KHz 4: 32KHz 5: 44.1KHz 6: 48KHz 7: 96KHz | [1, 2, 6] |

OSDCapabilities

| OSDCapabilities Json Block | { "IsSupportCfg": , "SupportedOSDContentTypeNum": , "SupportedOSDContentTypeList": , "IsSupportFontSizeCfg": , "IsSupportFontColorCfg": , "MaxAreaNum": , "MaxOSDNum": , "MaxPerAreaOSDNum": , "SupportedTimeFormatNum": , "SupportedTimeFormatList": , "SupportedDateFormatNum": , "SupportedDateFormatList": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupportCfg | M | boolean | Whether is configurable: 0: No 1: Yes | 0 |
| SupportedOSDContentTypeNum | M | unsigned long | Number of OSD content types supported. | 6 |
| SupportedOSDContentTypeList | M | array | OSD that is configurable. OSD content type 0: Disable 1: Custom 2: Time and date 3: PTZ controller 4: PTZ coordinates 5: Patrol info 6: Zoom info 7: Preset info 8: Alarm info 9: Encoding info 10: Serial port OSD 11: PTZ direction info 12: Channel name 13: Test OSD 14: People counting OSD 15: Network port OSD 16: Time 17: Date 18: Sensing OSD 19: Battery OSD 20: Scrolling characters 21: Logo OSD 22: Traffic monitoring (motor vehicle) 23: Traffic monitoring (non-motor vehicle) 24: Traffic monitoring (pedestrian) 25: Number of INFOOSD types | [1,2,3,10,16,17] |
| IsSupportFontSizeCfg | M | boolean | Whether OSD size is configurable. | 0 |
| IsSupportFontColorCfg | M | boolean | Whether OSD color is configurable. | 0 |
| MaxAreaNum | M | unsigned long | Maximum number of OSD areas. | 8 |
| MaxOSDNum | M | unsigned long | Maximum number of OSDs. | 8 |
| MaxPerAreaOSDNum | M | unsigned long | Maximum number of OSDs per area. | 8 |
| SupportedTimeFormatNum | M | unsigned long | Number of time OSD formats supported. | 6 |
| SupportedTimeFormatList | M | array | Time OSD formats. h/H=12/24 Hour; tt=A.M. or P.M.; mm=Minute; ss=Second; xxx=millisecond 0: HH:mm:ss (common in China, U.S., Great Britain) 1:hh:mm:ss t.t. (common in Spanish-speaking countries other than Spain) 2:hh:mm:ss tt (common in English-speaking countries other than the UK) 3:tt hh:mm:ss (common in Chinese (Singapore)) 4:Afternoon hh:mm:ss (common in Chinese (Taiwan)) 5:HH:mm:ss.xxx (millisecond) 6:hh:mm:ss.xxx tt (millisecond) | 7 |
| SupportedDateFormatNum | M | unsigned long | Number of date OSD formats supported. | 6 |
| SupportedDateFormatList | M | unsigned long | Date OSD format. Among which, MMMM means month, for example, June; dddd means day of week, for example, Mon Chinese 0-5 0: yyyy-MM-dd 1: MM-dd-yyyy 2: yyyyMMdd 3: MMddyyyy 4: yyyyMMdd X (day of week) 5: MMddyyyy X (day of week) English 100-108 100: dd/MM/yyyy 101: MM/dd/yyyy 102: dd MMMM, yyyy 103: MMMM dd, yyyy 104: dddd, dd MMMM, yyyy 105: dddd, MMMM dd, yyyy 106: yyyy/MM/dd 107: yyyy, MMMM dd 108: dddd, yy, MM dd | [0,1,2,3,4,5] |

OSDContentStyle

| OSDContentStyle Json Block | { "FontStyle":, "FontSize":, "Color":, "DateFormat":, "TimeFormat":, "FontAlignList":, "Margin": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FontStyle | C | unsigned long | Font style. 0: Background 1: Stroke 2: Hollow 3: Normal Applicable to IPC. | 3 |
| FontSize | M | unsigned long | Font size. 0: Very large 1: Large 2: Medium 3: Small | 1 |
| Color | M | unsigned long | Color (decimal). An RGB color is calculated by combining the relative brightness of three primary colors: red, green, and blue, and the value range of each primary color is 0-255. Formula: (65536*Red + 256*Green + Blue). To set color #665432, this field should be 65536 * 102 (decimal number of 66) + 256 * 84 (decimal number of 54) + 50 (decimal number of 32) = 6706226 (actually it is decimal number of #665432) To set RGB(102, 84, 50), this field should be 65536 * 102 + 256 * 84 + 50 = 6706226. |  |
| DateFormat | M | unsigned long | Date format. See OSDCapabilities. | 7 |
| TimeFormat | M | unsigned long | Time format. See OSDCapabilities. | 2 |
| FontAlignList | C | array | Font alignment within the area. Fixed 8 areas. 0: Left alignment 1: Right alignment Applicable to IPC. | [0,1,1,0,0,0,0,0] |
| Margin | C | unsigned long | Number of characters with empty margins. 0: None 1: One-character width 2: Two-character width Applicable to IPC. | 2 |

OSDContent

| OSDContent Json Block | { "Num": , "ContentList":[<OSDContentInfo> ,< OSDContentInfo>,...... } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of OSD contents. | 8 |
| ContentList | C | array | OSD content info list. Optional when Num is 0. | - |
| OSDContentInfo | M | Json Block | OSD content info. On NVR, channel name always uses the first OSD; date&time always uses the seconds OSD; people counting (if available, by querying capabilities) uses the third OSD, and other scenes names use the subsequent OSDs. See OSDContentInfo Json Block | - |

OSDContentInfo

| OSDContentInfo Json Block | { "ID": , "Enabled": , "Num": , "ContentInfo": [{ "ContentType": , "Value": },......] "Area": { "TopLeft": { "X": , "Y": } } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | OSD ID. Range: [0,7]. | 0 |
| Enabled | M | boolean | Enable or disable OSD. 0: Disable 1: Enable | 1 |
| Num | M | unsigned long | Number of OSDs in the area, limited by MaxPerAreaOSDNum in OSDCapabilities. For NVR, each area supports only one OSD. | 2 |
| ContentInfo | M | array | OSD content info. | - |
| ContentType | M | unsigned long | OSD content type. 0: Disable 1: Custom 2: Time and date 3: PTZ controller 4: PTZ coordinates 5: Patrol info 6: Zoom info 7: Preset info 8: Alarm info 9: Encoding info 10: Serial port OSD 11: PTZ direction info 12: Channel name 13: Test OSD 14: People counting OSD 15: Network port OSD 16: Time 17: Date 18: Sensing OSD 19: Battery OSD 20: Scrolling characters 21: Logo OSD 22: Traffic monitoring (motor vehicle) 23: Traffic monitoring (non-motor vehicle) 24: Traffic monitoring (pedestrian) 25: Number of INFOOSD types | 1 |
| Value | M | string | OSD text info. | “OSD text” |
| Area | M | Json Block | OSD area info. | - |
| TopLeft | M | Json Block | Coordinates of area’s top left corner. |  |
| X | M | unsigned long | x-coordinate of OSD. Range: [0,9999] | 200 |
| Y | M | unsigned long | y-coordinate of OSD. Range: [0,9999] | 200 |

OSDList

| OSDList Json Block | { "ContentStyle": <OSDContentStyle> , "Num": , "ContentList":[<OSDContentInfo> ,< OSDContentInfo>,...... } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ContentStyle | M | Json Block | OSD style. See OSDContentStyle Json Block. | - |
| Num | M | unsigned long | Number of OSD areas. | 8 |
| ContentList | C | array | Area and the corresponding OSD content list. See OSDContentInfo Json Block. Optional when Num is 0. | - |

PicInfo

| PicInfo Json Block | { "URL":, "Name":, "Size": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| URL | M | string | URL of alarm snapshots. Length range: [1,128]. | http://206.5.3.32:80/LAPI/V1.0/Channels/1/Alarm/Snapshot |
| Name | M | string | Image name. Length: [1,64] | ubs_19_0_0_1_0_1_409_2795 |
| Size | M | unsigned long | Image size. Unit: Byte. | 64164 |

RecordList

| RecordList Json Block | { "Total": , "Offset": , "Nums": , "RecordInfos": [<RecordInfo>, [<RecordInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Total | C | unsigned long | Total number of recordings, returned for paginated search only. Applicable to IPC. | 100 |
| Offset | C | unsigned long | Specifies from where to start searching. Limit and Offset are used to limit the maximum number of records returned. These two parameters are either used simultaneously or be null simultaneously. Null: means up to 100 records will be returned. Applicable to IPC. | 10 |
| Nums | M | unsigned long | Number of recording files searched. | 4 |
| RecordInfos | C | Array | Recording info list. Optional when Nums is 0. |  |
| RecordInfo | M | Json Block | See RecordInfo Json Block. |  |

RecordInfo

| RecordInfo Json Block | { “FileName”:“Abc123”, “RecordType”: 4, “Begin”: , “End”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FileName | M | string | Recording file name. Range: [0, 64]. Not applicable to IPC. | Abc123 |
| RecordType | M | unsigned long | Type of recording to search, handled by bit order, from left to right are bit 0, bit 1, bit 2 … bit 31, each representing a recording type, and will be converted into the Ulong type after being assigned a value: 0: Manual recording. 1: Common alarm (currently not applicable to NVR) 2: Motion detection 3. Input switch (alarm input) 4. Video loss 5. Audio detection 6. POS info (currently not applicable to NVR) 7: Scheduled recording 8: Face detection 9: Cross line detection 10: Intrusion detection | Search motion detection recording: 00000000000000000000000000000100 = 4, so fill this field with 4. Search motion detection recording and face detection recording: 00000000000000000000000100000100 = 260, so fill this field with 260. |
| Begin | M | unsigned long | Start time of recording file (unit: second, UTC format) | 1454428800 |
| End | M | unsigned long | End time of recording file (unit: second, UTC format) | 1454435960 |

RecordChannelInfoList

| RecordChannelInfoList Json Block | { "Num": , "ChannelInfoList": [<ChannelRecordInfo>, [<ChannelRecordInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of video input channels. | 3 |
| ChannelInfoList | C | array | Video channel info list. Required when Num is greater than 0. | - |
| ChannelRecordInfo | C | Json Block | Video channel info. Required when Num is greater than 0. | See ChannelRecordInfoJson Block. |

RecordFileList

| RecordFileList Json Block | { "Num": , “RecordFileInfoList”: [<RecordFileInfo >, [<RecordFileInfo >, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of recording files searched. | 4 |
| RecordFileInfos | C | array | Recording file info list. Required when Num is greater than 0. | - |
| RecordFileInfo | C | Json Block | Recording file info. Required when Num is greater than 0. | See RecordFileInfo Json Block. |

RecordFileInfo

| RecordFileInfo Json Block | { “ChannelID”: , “StreamID”: , “Begin”: , “End”: , “FileSizeForRecord”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ChannelID | M | unsigned long | Video channel ID of recordings to search. | 1 |
| StreamID | M | unsigned long | Stream ID of requested recording: 0: Main stream 1: Sub stream 2: Third stream | 0 |
| Begin | M | unsigned long | Start time of recording file (unit: second, UTC format) | 1454428800 |
| End | M | unsigned long | End time of recording file (unit: second, UTC format) | 1454435960 |
| FileSizeForRecord | M | unsigned long | Recording file size (Unit: KB). | 1024 |

ResolutionCapability

| ResolutionCapability Json Block | { “Width”: , “Height”: , “MinBitRate”: , “MaxBitRate”: , “DefaultBitRate”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Image width. | 0 |
| Height | M | unsigned long | Image height. | 1 |
| MinBitRate | M | unsigned long | Minimum bit rate. | 128 |
| MaxBitRate | M | unsigned long | Maximum bit rate. | 16384 |
| DefaultBitRate | C | unsigned long | Default bit rate. Applicable to NVR. | 2048 |

SnapshotCapabilities

| SnapshotCapabilities Json Block | { “SupportSnapshotCfg”:, "PictureMaxSize": , “ResolutionNum”: , “ResolutionCapabilityList”:[<SnapshotResolutionCapability>, <SnapshotResolutionCapability>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportSnapshotCfg | M | boolean | Whether the device supports snapshot configuration 0: No 1: Yes | 1 |
| PictureMaxSize | M | unsigned long | Maximum image size of snapshot. | 800 |
| ResolutionNum | M | unsigned long | Number of resolutions supported. | 4 |
| ResolutionCapabilityList | C | array | Info list of supported snapshot resolutions. Optional when ResolutionNum is 0. |  |
| SnapshotResolutionCapability | M | Json Block | Snapshot resolution. See SnapshotResolutionCapability Json Block. |  |

SnapshotResolutionCapability

| SnapshotResolutionCapability Json Block | { “Width”: , “Height”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Image width | 1920 |
| Height | M | unsigned long | Image height | 1680 |

StreamCapability

| StreamCapability Json Block | { “ID”: , “ResolutionNum”: , “ResolutionCapabilityList”:[<ResolutionCapability>, <ResolutionCapability>…] "MaxFrameRate":, "MaxMJPEGFrameRate": “SmartEncode”:<SmartEncodeInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Stream index 0: Main stream 1: Sub stream 2: Third stream | 3 |
| ResolutionNum | M | unsigned long | Number of resolutions supported. | 4 |
| ResolutionCapabilityList | M | array | Supported resolutions. Resolutions: Main stream >= Sub stream >= Third stream |  |
| ResolutionCapability | M | Json Block | Resolutions See ResolutionCapability Json Block. |  |
| MaxFrameRate | C | unsigned long | Maximum frame rate at the current bit rate. Applicable to IPC. | 25 |
| MaxMJPEGFrameRate | C | unsigned long | Maximum MJPEG frame rate at the current bit rate. Applicable to IPC. | 5 |
| SmartEncode | M | Json Block | Extended image encoding modes. See SmartEncodeInfo Json Block. |  |

SmartEncodeInfo

| SmartEncodeInfo Json Block | { “H264SmartEncodeModeNum”: , “H264SmartEncodeModeList”: , “H265SmartEncodeModeNum”: , “H265SmartEncodeModeList”: , } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| H264SmartEncodeModeNum | M | unsigned long | Number of H.264 smart encoding modes supported. | 2 |
| H264SmartEncodeModeList | M | array | H.264 extended image encoding modes. 0: off (disabled) 1: Basic (basic mode) 2: Advanced (advanced mode) | [1,2] |
| H265SmartEncodeModeNum | M | unsigned long | Number of H.265 smart encoding modes supported. | 2 |
| H265SmartEncodeModeList | M | array | H.265 extended image encoding modes. 0: off (disabled) 1: Basic (basic mode) 2: Advanced (advanced mode) | [1,2] |

SnapshotURLInfo

| SnapshotURLInfo Json Block | { "Num":, "PicInfos":[<PicInfo>,......] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of URLs. | 2 |
| PicInfos | C | array | Alarm snapshot info. Optional when Num is 0. | - |
| PicInfo | M | Json block | Alarm snapshot info. | See PicInfo Json Block |

SnapshotResolutionCapability

| SnapshotResolutionCapability Json Block | { “Width”: , “Height”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Image width | 1920 |
| Height | M | unsigned long | Image height | 1680 |

SnapshotCapabilities

| SnapshotCapabilities Json Block | { “SupportSnapshotCfg”:, "PictureMaxSize": , “ResolutionNum”: , “ResolutionCapabilityList”:[<SnapshotResolutionCapability>, <SnapshotResolutionCapability>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportSnapshotCfg | M | boolean | Whether the device supports snapshot configuration 0: No 1: Yes | 1 |
| PictureMaxSize | M | unsigned long | Maximum image size of snapshot. | 800 |
| ResolutionNum | M | unsigned long | Number of resolutions supported. | 4 |
| ResolutionCapabilityList | C | array | Info list of supported snapshot resolutions. Optional when ResolutionNum is 0. |  |
| SnapshotResolutionCapability | M | Json Block | Snapshot resolution. See ResolutionCapability Json Block. |  |

SnapshotInfo

| SnapshotInfo Json Block | { “Enabled”:, “Resolution”:{ “Width”:, “Height”: }, “PictureMaxSize”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | Boolean | Enable or disable snapshot: 0: Disable 1: Enable | 0 |
| Resolution | M | Json block | Snapshot resolution |  |
| Width | M | unsigned long | Image width of snapshot. Unit: Pixel. For snapshot resolutions available, see snapshot capability. | 1920 |
| Height | M | unsigned long | Image height of snapshot. Unit: Pixel. For snapshot resolutions available, see snapshot capability. | 1080 |
| PictureMaxSize | M | unsigned long | Maximum image size. Unit: KB The value of this parameter cannot be greater than the maximum value of image size in snapshot capability. | 10 |

VideoCapabilityInfo

| VideoCapability Info Json Block | { “IsSupportCfg”: , “IsSupportSmoothLevel”: , “IsSupportImageFormat”: , “EncodeFormatNum“: , “EncodeFormatList”: , “MinIFrameInterval”: , “MaxIFrameInterval”: , “StreamCapabilityNum”: , “StreamCapabilityList”:[<StreamCapability>, <StreamCapability> …] “VideoModeNum”: , “VideoModeInfoList”: [<VideoModeInfo>, <VideoModeInfo> … ] "GOPTypeNum":, "GOPTypeList ":[] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupportCfg | M | boolean | Whether is configurable: 0: No 1: Yes | 1 |
| IsSupportSmoothLevel | M | boolean | Whether supports stream smoothness settings. 0: No 1: Yes | 0 |
| IsSupportImageFormat | M | boolean | Whether image format is configurable: 0: No 1: Yes | 0 |
| EncodeFormatNum | M | unsigned long | Number of supported video encoding formats. |  |
| EncodeFormatList | C | array | Supported video encoding formats. 0: MJPEG 1: H.264 2: H.265 3: JPEG | [1,2] |
| MinIFrameInterval | M | unsigned long | Minimum I-frame interval. | 5 |
| MaxIFrameInterval | M | unsigned long | Maximum I-frame interval. | 250 |
| StreamCapabilityNum | M | unsigned long | Number of video streams supported. |  |
| StreamCapabilityList | C | array | List of supported stream capabilities. |  |
| StreamCapability | M | Json Block | Stream capability info. See StreamCapability Json Block. |  |
| VideoModeNum | M | unsigned long | Number of supported video modes. | 3 |
| VideoModeInfoList | C | Json Block | Video mode list. |  |
| VideoModeInfo | M | Json Block | Video mode info. See VideoModeInfo Json Block. |  |
| GOPTypeNum | M | unsigned long | Number of supported GOP types. |  |
| GOPTypeList | C | array | List of GOP types supported. 0: IP 1: IBP 2: IBBP 3: I Optional when GOPTypeNum is 0. | [0, 3] |

VideoModeInfo

| VideoModeInfo Json Block | { “Resolution”: { “Width”: , “Height”: }, “FrameRate”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Resolution | M | Json Block | Resolution. |  |
| Width | M | unsigned long | Image width. | 1920 |
| Height | M | unsigned long | Image height. | 1080 |
| FrameRate | M | unsigned long | Maximum frame rate in the current mode. | 25 |

VideoStreamInfoList

| VideoStreamInfoList Json Block | { "Num":, "VideoStreamInfos":[<VideoStreamInfo>,<VideoStreamInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of video streams | 1 |
| VideoStreamInfos | C | array | Video stream info list. Optional when Num is 0. | - |
| VideoStreamInfo | M | Json Block | Info about a video stream. See VideoStreamInfo Json Block. | - |

VideoStreamInfo

| VideoStreamInfo Json Block | { "ID":, "MainStreamType":, "Enabled":, "VideoEncodeInfo":<VideoEncodeInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Video stream index number, starts from 0. 0: Main stream 1: Sub stream 2: Third stream | 0 |
| MainStreamType | C | unsigned long | 0: Main stream of scheduled recording 1: Main stream of event recording Valid when ID is 0. Applicable to NVR only. | 0 |
| Enabled | M | boolean | Enable or disable encoding for the video stream. 0: Disable 1: Enable | 1 |
| VideoEncodeInfo | M | Json Block | Info about video encoding. See VideoEncodeInfo Json Block | - |

VideoEncodeInfo

| VideoEncodeInfo Json Block | { "EncodeFormat":, "Resolution": { "Width":, "Height": }, "BitRate":, "BitRateType":, "FrameRate":, "GOPType": "IFrameInterval":, "ImageQuality":, "SmoothLevel":, "SVCMode":, "SmartEncodeMode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| EncodeFormat | M | unsigned long | Video encoding format. 0: MJPEG 1: H.264 2: H.265 | 2 |
| Resolution | M | Json Block | Resolution. |  |
| Width | M | unsigned long | Image width. | 1920 |
| Height | M | unsigned long | Image height. | 1080 |
| BitRate | M | unsigned long | Bit rate. | 4096 |
| BitRateType | M | unsigned long | Bit rate type. 0: CBR 1: VBR | 0 |
| FrameRate | M | unsigned long | Frame rate | 25 |
| GOPType | C | unsigned long | GOP mode. 0: IP 1: IBP 2: IBBP 3: I Applicable to IPC. | 0 |
| IFrameInterval | M | unsigned long | I frame interval. The range depends on capability. | 50 |
| ImageQuality | C | unsigned long | Image quality. Range: [0 9]. 9 is the highest. Valid only when BitrateType is VBR. Optional when invalid. | 9 |
| SmoothLevel | M | unsigned long | Stream smoothness level. Range: [1,9]. 1 is the lowest. | 5 |
| SVCMode | C | boolean | Enable or disable SVC: 0: Disable 1: Enable Applicable to IPC. | 0 |
| SmartEncodeMode | M | unsigned long | Smart encoding format. 0: off (disabled) 1: Basic (basic mode) 2: Advanced (advanced mode) | 2 |

Image

Capabilities

/LAPI/V1.0/Channels/<ID>/Image/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Image/Capabilities |
| --- | --- |
| Description | Get image parameters of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | ImageCapabilities |
| Note | Applicable to IPC and NVR. |
| Status |  |

Enhance

/LAPI/V1.0/Channels/<ID>/Image/Enhance

| URL | /LAPI/V1.0/Channels/<ID>/Image/Enhance |
| --- | --- |
| Description | Get image enhancement parameters of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | ImageEnhanceInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Image/Enhance |
| --- | --- |
| Description | Set image enhancement parameters of a channel. |
| Method | PUT |
| Input Data | ImageEnhanceInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

Focus

/LAPI/V1.0/Channels/<ID>/Image/FocusInfo

| URL | /LAPI/V1.0/Channels/<ID>/Image/FocusInfo |
| --- | --- |
| Description | Get focus parameters of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | FocusInfo |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Image/FocusInfo |
| --- | --- |
| Description | Set focus parameters of a video channel. |
| Method | PUT |
| Input Data | FocusInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

LampCtrl

/LAPI/V1.1/Channels/<ID>/Image/LampCtrl

| URL | /LAPI/V1.1/Channels/<ID>/Image/LampCtrl |
| --- | --- |
| Description | Get illuminator parameters of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | LampCtrlInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.1/Channels/<ID>/Image/LampCtrl |
| --- | --- |
| Description | Set illuminator parameters of a video channel. |
| Method | PUT |
| Input Data | LampCtrlInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

Exposure

/LAPI/V1.0/Channels/<ID>/Image/Advanced/Exposure

| URL | /LAPI/V1.0/Channels/<ID>/Image/Advanced/Exposure |
| --- | --- |
| Description | Get image exposure parameters. |
| Method | GET |
| Input Data | None |
| Success Return Data | ExposureInfo |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Image/Advanced/Exposure |
| --- | --- |
| Description | Set image exposure parameters. |
| Method | PUT |
| Input Data | ExposureInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

Illumination

/LAPI/V1.0/Channels/<ID>/Image/Illumination

| URL | /LAPI/V1.0/Channels/<ID>/Image/Illumination |
| --- | --- |
| Description | Get image illumination level. |
| Method | GET |
| Input Data | None |
| Success Return Data | IlluminationInfo |
| Note | Applicable to IPC. |
| Status |  |

Reset

/LAPI/V1.0/Channels/<ID>/Image/ImageParamReset

| URL | /LAPI/V1.0/Channels/<ID>/Image/ImageParamReset |
| --- | --- |
| Description | Set default image parameter values. |
| Method | PUT |
| Input Data | ImageParamResetInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

Json Block

DayNightCapabilitiesInfo

| DayNightCapabilitiesInfo Json Block | { “SupportDayNightCfg”:, “DayNightModeNum”:, “DayNightModeList”:, “SupportSensibilityCfg”:, “SupportSwitchingCfg”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportDayNightCfg | M | boolean | Whether day/night mode is configurable: 0: No 1: Yes | 1 |
| DayNightModeNum | M | unsigned long | Number of day/night mode types available. |  |
| DayNightModeList | C | array | Day/night mode types: 0: Auto 1: Day--color 2: Night--black & white 3: Trigger day to night Optional when DayLightModeNum is 0. | [0,1,2] |
| SupportSensibilityCfg | M | boolean | Whether day/night mode sensitivity is configurable: 0: No 1: Yes | 1 |
| SupportSwitchingCfg | M | boolean | Whether day/night mode switching time is configurable: 0: No 1: Yes | 1 |

DayNightInfo

| DayNightInfo Json Block | { “Mode”:, “Sensitivity”:, “Time”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | C | unsigned long | Day/night mode type. See DayNightCapabilitiesInfo. Required when supported by image capability. | 2 |
| Sensitivity | C | unsigned long | Day/night mode sensitivity. Valid when day/night mode is “auto”. Range: [0, 9]. Required when supported by image capability. | 4 |
| Time | C | unsigned long | Day/night mode switching time. Valid when day/night mode is “auto”. Range: [3, 120]. Unit: Second. Required when supported by image capability. | 3 |

EnhanceCapabilitiesInfo

| EnhanceCapabilitiesInfo Json Block | { “ImageRotationModeNum”: , “ImageRotationModeList”: , “IsSupportSharpness”:, “IsSupport2DNoiseReduce”:, “IsSupport3DNoiseReduce”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ImageRotationModeNum | M | unsigned long | Number of image rotation modes supported. | 3 |
| ImageRotationModeList | C | array | Info list of supported image rotation modes: 0: Normal 1: Flip vertically 2: Flip horizontally 3: Rotate 180° clockwise 4: Rotate 90° clockwise 5: Rotate 90° counterclockwise Optional when ImageRotationModeNum is 0. | [1,3,5] |
| IsSupportSharpness | M | boolean | Whether sharpness is supported 0: No 1: Yes | 1 |
| IsSupport2DNoiseReduce | M | boolean | Whether 2D noise reduction is supported. 0: No 1: Yes | 0 |
| IsSupport3DNoiseReduce | M | boolean | Whether 3D noise reduction is supported. 0: No 1: Yes Applicable to IPC and VMS. | 0 |

ExposureCapabilitiesInfo

| ExposureCapabilitiesInfo Json Block | { "SupportCompensation":, "ExposureModeNum":, "ExposureModeList":, "SupportIrisCfg":, "IrisRangeNum":, "IrisRangeList":, "MinGain":, "MaxGain":, "SupportHLCCfg":, "SupportMeteringCfg":, "MeteringModeNum":, "MeteringModeList":, "Shutter": <ShutterCapabilitiesInfo>, "WideDynamic"<WideDynamicCapabilitiesInfo>, "DayNight": <DayNightCapabilitiesInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCompensationCfg | M | boolean | Whether exposure compensation is configurable. 0: No 1: Yes | 1 |
| ExposureModeNum | M | unsigned long | Number of exposure modes available. |  |
| ExposureModeList | C | array | Exposure modes: 0: Automatic exposure 1: Custom exposure 2: Shutter priority 3: Iris priority 4: Gain priority 5: Indoor 50Hz 6: Indoor 60Hz 7: Manual exposure 8: Low motion blur Optional when ExposureModeNum is 0. | [0,1,3,4,6,7] |
| SupportIrisCfg | M | boolean | Whether iris is configurable. 0: No 1: Yes | 1 |
| IrisRangeNum | M | unsigned long | Number of iris values available. |  |
| IrisRangeList | C | array | Iris values: 160, 200, 240, 280, 340, 400, 480, 560, 680, 800, 960, 1100, 1400, 1600, 2200 Optional when IrisRangeNum is 0. | [160, 200, 240] |
| MinGain | M | unsigned long | Minimum gain. | 1 |
| MaxGain | M | unsigned long | Maximum gain. | 100 |
| SupportHLCCfg | M | boolean | Whether HLC sensitivity is configurable. 0: No 1: Yes | 1 |
| SupportMeteringCfg | M | boolean | Whether metering control is configurable. 0: No 1: Yes | 1 |
| MeteringModeNum | M | unsigned long | Number of metering control modes available. |  |
| MeteringModeList | C | array | Metering control modes: 0: Center-weighted average metering 1: Evaluative metering 2: HLC 3: Vehicle metering 4: Face metering 5: Spot metering Optional when MeteringModeNum is 0. | [0,1,2,3,4,5] |
| Shutter | M | Json block | Shutter See ShutterCapabilitiesInfo Json Block. |  |
| WideDynamic | M | Json block | WDR See WideDynamicCapabilitiesInfo Json Block. |  |
| DayNight | M | Json block | Day/night mode See DayNightCapabilitiesInfo Json Block. |  |

ExposureInfo

| ExposureInfo Json Block | { "Mode": , "CompensationLevel":, "HLCSensitivity":, "IrisInfo": <IrisInfo>, "ShutterInfo": <ShutterInfo>, "GainInfo": <GainInfo>, "WideDynamic": <WideDynamicInfo>, "Metering": <MeteringInfo>, "DayNight": <DayNightInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Exposure mode. See ExposureCapabilitiesInfo | 0 |
| CompensationLevel | C | unsigned long | Exposure compensation level. Valid when exposure mode is not "Manual". Range: [-100,100]. Required when supported by image capability. | 0 |
| HLCSensitivity | C | unsigned long | HLC sensitivity Valid when the scene is Road HLC or Park HLC. Range: [1, 9]. Required when supported by image capability. |  |
| IrisInfo | C | Json Block | Iris info. See IrisInfo. Required when supported by image capability. |  |
| ShutterInfo | C | Json Block | Shutter info. See ShutterInfo. Required when supported by image capability. |  |
| GainInfo | M | Json Block | Gain info. See GainInfo. |  |
| WideDynamic | C | Json Block | WDR info See WideDynamicInfo Required when supported by image capability. |  |
| Metering | C | Json Block | Metering info See MeteringInfo. Valid when the scene is not Road HLC and Park HLC. Required when supported by image capability. |  |
| DayNight | C | Json Block | Day/night mode info. See DayNightInfo Required when supported by image capability. |  |

FocusCapabilitiesInfo

| FocusCapabilitiesInfo Json Block | { “SupportFocusCfg”:, “FocusModeNum”:, “FocusModeList”:, “FocusSceneNum”:, “FocusSceneList”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportFocusCfg | M | boolean | Whether focus parameters are configurable: 0: No 1: Yes | 1 |
| FocusModeNum | M | unsigned long | Number of focus modes available. |  |
| FocusModeList | C | array | List of focus modes available. 0: Auto focus 1: Manual focus 2: One-key focus 3: One-key focus (IR) 4: One-key focus (locked) One-click focus: The device triggers automatic focus once when rotating, zooming or calling a preset. One-click focus (IR): This mode achieves optimal effects when IR is turned on in a low-light environment (e.g., during night-time or in a dark room). One-click focus (lock): Automatic focus is performed once after the camera finishes zooming. Other actions cannot trigger focus. Optional when FocusModeNum is 0. | [1,2,3,4] |
| FocusSceneNum | M | unsigned long | Number of focus scenes available |  |
| FocusSceneList | C | array | Focus scenes available: 0: Normal 1: Ultra-long distance 2: Indoor The Normal mode is applicable to general scenes such as road, parks, etc. The Ultra-long distance mode is applicable to long distant road scenes, for example, when the camera mounting height is greater than 30m. The Indoor mode is applicable to indoor scenes. Optional when FocusSceneNum is 0. | [1,2] |

FocusInfo

| FocusInfo Json Block | { "Mode": , "Scene": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Focus mode. See FocusInfoCapabilitiesInfo. | 1 |
| Scene | C | unsigned long | Focus scene. See FocusInfoCapabilitiesInfo. Required when supported by image capability. | 5 |

GainInfo

| GainInfo Json Block | { “Gain”:, “MinGain”:, “MaxGain”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Gain | C | unsigned long | Gain (unit: db). Valid in manual exposure mode. Required when supported by image capability. See image capability for the enumerations and range. | 0 |
| MinGain | C | unsigned long | Minimum gain. Valid in custom exposure mode. Must not be greater than the maximum gain. Required when supported by image capability. See image capability for the range. | 0 |
| MaxGain | C | unsigned long | Maximum gain. Valid in custom exposure mode. Must not be less than the minimum gain. Required when supported by image capability. See image capability for the range. | 30 |

ImageCapabilities

| ImageCapabilities Json Block | { "IsSupportCfg":, "SupportCorrectCfg":, "SupportIlluminationCfg", "SupportEPTZ":, "Enhance": <EnhanceCapabilitiesInfo>, "Focus":<FocusCapabilitiesInfo>, "LampCtrl":< LampCtrlCapabilitiesInfo>, "Exposure": <ExposureCapabilitiesInfo>, "WhiteBalance":<WhiteBalanceCapabilitiesInfo>, "Scene":<SceneCapabilitiesInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupportCfg | M | boolean | Whether the device supports image parameter configuration: 0: No 1: Yes | 1 |
| SupportCorrectCfg | M | boolean | Whether the device supports image correction. 0: No 1: Yes Applicable to IPC only. | 1 |
| SupportIlluminationCfg | M | boolean | Whether the device supports obtaining illumination level. 0: No 1: Yes Applicable to IPC only. | 1 |
| SupportEPTZ | C | boolean | Whether the device supports ePTZ auto tracking. 0: No 1: Yes | 1 |
| Enhance | M | Json block | Image enhancement capability parameter | See EnhanceCapabilitiesInfo Json Block. |
| Focus | M | Json block | Focus capability parameter Applicable to IPC only. | See FocusCapabilitiesInfo Json Block. |
| LampCtrl | M | Json block | Illuminator capability parameter Applicable to IPC only. | See LampCtrlCapabilitiesInfo Json Block. |
| Exposure | M | Json block | Exposure capability parameter Applicable to IPC only. | See ExposureCapabilitiesInfo Json Block. |
| WhiteBalance | M | Json block | White balance capability parameter Applicable to IPC only. | See WhiteBalanceCapabilitiesInfo Json Block. |
| Scene | M | Json block | Scene capability parameter Applicable to IPC only. | See SceneCapabilitiesInfo Json Block. |

IrisInfo

| IrisInfo Json Block | { “Iris”:, “MinIris”:, “MaxIris”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Iris | C | unsigned long | Iris Optional in iris priority or manual exposure mode. Required when supported by image capability. See image capability for the enumerations. | 160 |
| MinIris | C | unsigned long | Minimum iris. Valid in custom exposure mode. Must not be greater than the maximum iris. See iris capability for the enumerations. Required when supported by image capability. | 160 |
| MaxIris | C | unsigned long | Maximum iris. Valid in custom exposure mode. Must not be less than the minimum iris. See iris capability for the enumerations. Required when supported by image capability. | 200 |

IlluminationInfo

| IlluminationInfo Json Block | { “Value”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Value | C | unsigned long | Illumination level value, normalized to the range [0 - 255]. The higher the level, the higher the overall brightness of the scene. Required when supported by image capability. | 1 |

ImageParamResetInfo

| ImageParamResetInfo Json Block | { “Type”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Restore default parameter values 0: Video parameters 1: Image parameters | 1 |

ImageEnhanceInfo

| ImageEnhanceInfo Json Block | { “Brightness”: , “Contrast”:, “Saturation”:, “Sharpness”:, “ImageRotation”:, “2DNoiseReduce”:, “3DNoiseReduce”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Brightness | M | unsigned long | Brightness. Range: [0,255] | 128 |
| Contrast | M | unsigned long | Contrast. Range: [0,255]. | 128 |
| Saturation | M | unsigned long | Saturation. Range: [0,255]. | 128 |
| Sharpness | C | unsigned long | Sharpness. Range: [0,255]. Required when supported by image capability. | 128 |
| ImageRotation | C | unsigned long | Image rotation modes. Required when supported by image capability. | 1 |
| 2DNoiseReduce | C | unsigned long | 2D NR. Range: [0,255]. Required when supported by image capability. | 128 |
| 3DNoiseReduce | C | unsigned long | 3D noise reduction. Required when supported by image capability. | 128 |

LampInfo

| LampInfo Json Block | { “LampType”: , “LampCtrlModeListNum”: , “LampCtrlModeList”: , “SupportNearLampCfg”:, “SupportMiddleLampCfg”:, “SupportFarLampCfg”: , “SupportSuperFarLampCfg”:, “SupportLaserAngleCfg”:, “LaserAngleMin”: , “LaserAngleMax”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LampType | M | unsigned long | Illuminator types: 0: No illuminator 1: White light illuminator 2: IR illuminator 3: Laser 4: Housing IR illuminator | 1 |
| LampCtrlModeListNum | M | unsigned long | Number of control modes available. |  |
| LampCtrlModeList | C | array | Control modes available: 0: Global mode 1: Overexposure restrain mode 2: Preset-road mode 3: Manual mode 4: Preset-park mode 5: Auto sensitivity control mode 6: Indoor mode 7: Manual-always on mode 8: Nighttime video detection mode Global mode and Overexposure restrain mode: Illuminator on/off is linked to day/night mode, and illumination intensity is calculated by AE algorithm. Preset-road mode or Preset-park mode: Turning illuminator on or off is linked to day/night mode, and the intensity of illumination is calculated based on the current zoom ratio and PT angle in combination with preset table file. Manual mode: Linked to day/night mode, and illumination intensity is determined by values configured in the user interface. Manual-always on mode: Not linked with day/night mode, and illuminator is always on. Nighttime video detection mode: Linked with day/night mode and smart virtual iris status. When day/night mode is “nigh” and smart virtual iris is “on”, the illuminator is on; otherwise, the illuminator is off. Illumination intensity level is set on the GUI. Optional when LampCtrlModeListNum is 0. | 0 |
| SupportNearLampCfg | M | boolean | Whether near-range illuminator is configurable: 0: No 1: Yes | 0 |
| SupportMiddleLampCfg | M | boolean | Whether medium-range illuminator is configurable: 0: No 1: Yes | 0 |
| SupportFarLampCfg | M | boolean | Whether long-range illuminator is configurable: 0: No 1: Yes | 0 |
| SupportSuperFarLampCfg | M | boolean | Whether ultra-long-range illuminator is configurable: 0: No 1: Yes | 1 |
| SupportLaserAngleCfg | M | boolean | Whether laser angle is configurable: 0: No 1: Yes | 0 |
| LaserAngleMin | C | unsigned long | Minimum laser angle. Optional when SupportLaserAngleCfg is 0. |  |
| LaserAngleMax | C | unsigned long | Maximum laser angle. Optional when SupportLaserAngleCfg is 0. |  |

LampCtrlCapabilitiesInfo

| LampCtrlCapabilitiesInfo Json Block | { “LampNum”: “LampInfos”: [<LampInfo>,<LampInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LampNum | M | unsigned long | Number of illuminator types available. 0 means illuminator is not supported. |  |
| LampInfos | C | array | Illuminator info list. Optional when LampNum is 0. |  |
| LampInfo | M | Json Block | Illuminator info. | See LampInfo Json Block. |

LampCtrlInfo

| LampCtrlInfo Json Block | { "Enable":, "Type": , "Mode": , "Num": , "LampList":[<LampCtrlInfoList>,<LampCtrlInfoList>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enable | M | boolean | Enable or disable illuminator: 0: Disable 1: Enable | 1 |
| Type | M | unsigned long | Illuminator type. See LampCtrlCapabilitiesInfo ). | 1 |
| Mode | M | unsigned long | Illuminator control mode. See LampCtrlCapabilitiesInfo ). | 1 |
| Num | M | unsigned long | Number of illuminator groups. | 1 |
| LampList | C | array | Illuminator parameter list. Optional when Num is 0. |  |
| LampCtrlInfoList | M | Json Block | Illuminator parameter. See LampCtrlInfoList Json. |  |

LampCtrlInfoList

| LampCtrlInfoList Json Block | { "Type": , "Num": , "PropertyList":[<PropertyCtrlInfoList>,<PropertyCtrlInfoList>,……]" } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Illuminator type. See LampCtrlCapabilitiesInfo | 0 |
| Num | M | unsigned long | Number of illuminator groups available. | 1 |
| PropertyList | C | array | Illuminator attribute list. Optional when ListNum is 0. |  |
| PropertyCtrlInfo | M | Json Block | Illuminator attribute parameter. See PropertyCtrlInfoList Json. |  |

MeteringInfo

| MeteringInfo Json Block | { "Mode": , “RefBrightness”: “HoldTime”: "Area": { "TopLeft": { "X":, "Y": }, " BottomRight": { "X":, "Y": } } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | C | unsigned long | Metering control mode. Valid when not in manual exposure mode. See ExposureCapabilitiesInfo for the enumerations Required when supported by image capability. | 1 |
| RefBrightness | C | unsigned long | Brightness of face. Valid in face metering mode. Range: [0, 100]. | 1 |
| HoldTime | C | unsigned long | Minimum duration. Valid in face metering mode. Unit: minute. Range: [0, 60]. | 1 |
| MeteringArea | C | Json block | Metering area. Valid in evaluative metering mode or spot metering mode. |  |
| TopLeft | M | Json block | Coordinates of top left corner. | (25, 25) |
| X | M | unsigned long | x-coordinate of top left corner(proportion). Evaluative metering range: [0100]. | 25 |
| Y | M | unsigned long | y-coordinate of top left corner(proportion): Evaluative metering range: [0, 100] | 25 |
| BottomRight | M | Json block | Coordinates of bottom right corner. | (75, 75) |
| X | M | unsigned long | x-coordinate of bottom right corner(proportion): Evaluative metering range: [0, 100] Must not be less than the x-coordinate of top left corner. | 75 |
| Y | M | unsigned long | y-coordinate of bottom right corner(proportion): Evaluative metering range: [0, 100] Must not be less than the y-coordinate of top left corner. | 75 |

PropertyCtrlInfoList

| PropertyCtrlInfoList Json Block | { "Index": , "Property": , "Level" } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Index | M | unsigned long | Index of the illuminator group. Range: [0-3]. | 0 |
| Property | M | unsigned long | Illumination distance: 0: Near-range illuminator 1: Medium-range illuminator 2: Long-range illuminator 3: Ultra-long-range illuminator 4: Laser | 0 |
| Level | M | unsigned long | Range for near-range, medium-range, long-range, and ultra-long-range illuminator: [0-1000]. A higher level means higher illumination intensity. For laser range see illuminator capability. A smaller laser angle means higher illumination intensity. | 5 |

SceneCapabilitiesInfo

| SceneCapabilitiesInfo Json Block | { "SupportSceneCfg": , "SceneTemplateNum":, "SceneTypeNum":, "SceneTypeList":, "TriggerNum":, "EnvironmentTypeNum":, "EnvironmentTypeList": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportSceneCfg | M | boolean | Whether scene is configurable: 0: No 1: Yes | 1 |
| SceneTemplateNum | C | unsigned long | Number of scene templates available. Optional when scene is not configurable. | 1 |
| SceneTypeNum | C | unsigned long | Number of scene types available. Optional when scene is not configurable. | 1 |
| SceneTypeList | C | array | Scene types: 0: Custom 1: Indoor 2: Common 3: Vehicle license plate 4: Starlight 5: Test 6: Photographic 7: HLC 8: WDR 9: Standard 10: Bright 11: Vivid 12: Park HLC 13: Face 14: Clarity 15: Mixed traffic Optional when SceneTemplateNum is 0. | [2, 1, 5, 7, 12, 4, 13, 0] |
| TriggerNum | C | unsigned long | Number of triggers available. Optional when scene is not configurable. | 4 |
| EnvironmentTypeNum | C | unsigned long | Number of environment types. Optional when scene is not configurable. | 1 |
| EnvironmentTypeList | C | array | Environment types: 0: Ambient illumination 1: PTZ tilt angle Optional when EnvironmentTypeNum is 0. | [0] |

ShutterCapabilitiesInfo

| ShutterCapabilities Json Block | { “SupportShutterTimeCfg”: , “ShtterTimeUnit”: , “ShutterTimeRangeNum”: , “ShutterTimeRangeList”: , “SupportSlowShutterCfg”:, “SlowShutterTimeRangeNum”:, “SlowShutterTimeRangeList”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportShutterTimeCfg | M | boolean | Whether shutter time is configurable: 0: No 1: Yes | 1 |
| ShtterTimeUnit | M | unsigned long | Shuttle time unit: 0: Microsecond 1: Second | 1 |
| ShutterTimeRangeNum | M | unsigned long | Number of shutter time values available. |  |
| ShutterTimeRangeList | C | array | Shutter time values: 0: Automatic 1: 1/1 2: 1/2 3: 1/3 4: 1/4 6: 1/6 8: 1/8 10: 1/10 12: 1/12 15: 1/15 20: 1/20 25: 1/25 30: 1/30 50: 1/50 60: 1/60 100: 1/100 120: 1/120 150: 1/150 180: 1/180 200: 1/200 250: 1/250 500: 1/500 1000: 1/1000 2000: 1/2000 4000: 1/4000 8000: 1/8000 50000: 1/50000 100000: 1/100000 Optional when ShutterTimeRangeNum is 0. | [0,4,8,10] |
| SupportSlowShutterCfg | M | boolean | Whether slow shutter is configurable: 0: No 1: Yes | 1 |
| SlowShutterTimeRangeNum | M | unsigned long | Number of slow shutter time values available. |  |
| SlowShutterTimeRangeList | C | array | Slow shutter time values: 0: Automatic 1: 1/1 2: 1/2 3: 1/3 4: 1/4 6: 1/6 8: 1/8 10: 1/10 12: 1/12 15: 1/15 20: 1/20 25: 1/25 Optional when SlowShutterTimeRangeNum is 0. | [0,4,8,10] |

ShutterInfo

| ShutterInfo Json Block | { “Shutter”:, “MinShutter”:, “MaxShutter”:, “IsEnableSlowShutter”:, “SlowestShutter”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Shutter | C | unsigned long | Shutter time. Valid in iris priority mode, manual exposure mode, or low motion blur mode. Required when supported by image capability. See image capability for the enumerations and units. | 0 |
| MinShutter | C | unsigned long | Minimum shutter time. Valid in custom exposure mode. Must not be greater than the maximum shutter time. See shutter capability for the enumerations. Required when supported by image capability. | 0 |
| MaxShutter | C | unsigned long | Maximum shutter time. Valid in custom exposure mode. Must not be less than the minimum shutter time. See shutter capability for the enumerations. Required when supported by image capability. | 30 |
| IsEnableSlowShutter | C | boolean | Enable or disable slow shutter. 0: Disable 1: Enable Valid when not in iris priority mode. Required when supported by image capability. | 1 |
| SlowestShutter | C | unsigned long | Slowest shutter. Valid when slow shutter is enabled. Required when supported by image capability. See image capability for the enumerations. | 20 |

WhiteBalanceCapabilitiesInfo

| WhiteBalanceCapabilitiesInfo Json Block | { “WhiteBalanceModeNum”:, “WhiteBalanceModeList” } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| WhiteBalanceModeNum | M | unsigned long | Number of white balance modes available. | 4 |
| WhiteBalanceModeList | M | array | Info list of supported white balance modes: 0: Auto 1: Fine-tune 2: Indoor 3: Outdoor 4: Auto mercury lamp 5: Night fine-tune 6: Auto sodium lamp 7: Locked 8: Adaptive night fine-tune 9: Auto 2 | [0,1,2,3] |

WideDynamicCapabilitiesInfo

| WideDynamicCapabilitiesInfo Json Block | { “SupportWideDynamicCfg”:, “WideDynamicModeNum”:, “WideDynamicModeList”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportWideDynamicCfg | M | boolean | Whether WDR is configurable: 0: No 1: Yes | 1 |
| WideDynamicModeNum | M | unsigned long | Number of WDR modes available. |  |
| WideDynamicModeList | C | array | WDR modes: 0: Disable 1: Enable 2: Auto Optional when WideDynamicModeNum is 0. | [0,1] |

WideDynamicInfo

| WideDynamicInfo Json Block | { “Mode”:, “Level”:, “OpenSensitivity”:, “CloseSensitivity”:, “AntiFlicker”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | WDR mode. See WideDynamicCapabilitiesInfo | 1 |
| Level | M | unsigned long | WDR level. Valid when WDR is enabled and the exposure mode is one of the following: Automatic, Custom, Shutter priority, Indoor 50Hz, Indoor 60Hz, Low motion blur. Range: [1, 9]. | 1 |
| OpenSensitivity | M | unsigned long | Sensitivity for enabling WDR. Valid when WDR mode is Auto. Range: [1, 9]. | 1 |
| CloseSensitivity | M | unsigned long | Sensitivity for disabling WDR. Valid when WDR mode is Auto. Range: [1, 9]. | 1 |
| AntiFlicker | M | Boolean | Anti-flicker (suppress WDR stripes) 0: Disable 1: Enable When enabled, this feature can eliminate stripes in images. | 1 |

Storage

Capabilities

/LAPI/V1.0/Storage/Capabilities

| URL | /LAPI/V1.0/Storage/Capabilities |
| --- | --- |
| Description | Get device storage capability. |
| Method | GET |
| Input Data | None |
| Success Return Data | StorageCapabilities |
| Notes | Applicable to NVR. |
| Status |  |

Containers

/LAPI/V1.0/Storage/Containers/DetailInfos

| URL | /LAPI/V1.0/Storage/Containers/DetailInfos |
| --- | --- |
| Description | Get storage container info list. |
| Method | GET |
| Input Data | None |
| Success Return Data | ContainerInfoList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/Storage/Containers/DetailInfos |
| --- | --- |
| Description | Set all storage container info. |
| Method | PUT |
| Input Data | { "Num":, "ContainerInfoList": [{ "ID": , "Type": , "Property": , "UsageType": , "GroupID": }, { "ID": , "Type": , "Property": , "UsageType": , "GroupID": }, ……] } |
| Success Return Data | None |
| Note | Applicable to NVR and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of storage containers to modify. | 3 |
| ContainerInfoList | M | array | Storage container info list. | - |
| ID | M | unsigned long | Storage container ID. Read-only. | 1 |
| Type | M | unsigned long | Storage container type. Read-only. 0: Local HDD 1: Disk enclosure-1 HDD 2: Disk enclosure-2 HDD 3: RAID 4: NAS 5: SAN 6: eSATA 7: SD | 0 |
| Property | M | unsigned long | Storage container property. 0: Read/write 1: Read-only 2: Redundant. Valid only when the device supports redundancy. Applicable to NVR. | 0 |
| GroupID | 0 | unsigned long | Disk group ID. Applicable to NVR. | 1 |
| UsageType | 0 | unsigned long | Usage. 0: Recording/snapshot 1: Backup (valid only for NAS and eSATA) Absent when the storage container is SD card. Applicable to NVR. | 0 |

Schedule

/LAPI/V1.0/Channels/<ID>/Storage/Schedule/Record

| URL | /LAPI/V1.0/Channels/<ID>/Storage/Schedule/Record |
| --- | --- |
| Description | Get recording schedule of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | RecordScheduleInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Storage/Schedule/Record |
| --- | --- |
| Description | Set recording schedule of a video channel. |
| Method | PUT |
| Input Data | RecordScheduleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Storage/Schedule/Picture

| URL | /LAPI/V1.0/Channels/<ID>/Storage/Schedule/Picture |
| --- | --- |
| Description | Get snapshot schedule of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | PictureScheduleInfo |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Storage/Schedule/Picture |
| --- | --- |
| Description | Set snapshot schedule of a video channel. |
| Method | PUT |
| Input Data | PictureScheduleInfo |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

Json Block

StorageCapabilities

| StorageCapabilities Json Block | { "IsSupportRedundancy": , "IsSupporteSATA": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupportRedundancy | M | boolean | Whether redundancy is supported. 0: No 1: Yes | 1 |
| IsSupporteSATA | M | boolean | Whether eSATA is supported. 0: No 1: Yes | 0 |

ContainerInfoList

| ContainerInfoList Json Block | { "LocalHDDNum" : , "LocalHDDList": [<ContainerInfo>, <ContainerInfo>,……], "SDNum" : , "SDList": [<ContainerInfo>, <ContainerInfo>,……], "ArrayNum" : , "ArrayList":[<ContainerInfo>, <ContainerInfo>,……], "ExtendCabinet1HDDNum" : , "ExtendCabinet1HDDList": [<ContainerInfo>, <ContainerInfo>,……], "ExtendCabinet2HDDNum" : , "ExtendCabinetHDDList": [<ContainerInfo>, <ContainerInfo>,……], "NASNum" : , "NASList": [<ExContainerInfo>, <ExContainerInfo>,……], "eSATANum" : , "eSATAList": [<ExContainerInfo>, <ExContainerInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LocalHDDNum | M | unsigned long | Number of local HDDs. | 4 |
| LocalHDDList | C | array | Local HDD info list. Optional when LocalHDDNum is 0. | - |
| SDNum | M | unsigned long | Number of SD cards. Reserved. Not applicable to NVR and VMS. | 0 |
| SDList | C | array | SD card info list. Optional when SDNum is 0. | - |
| ArrayNum | M | unsigned long | Number of RAIDs. | 1 |
| ArrayList | C | array | Array info list. Optional when ArrayNum is 0. | - |
| ExtendCabinet1HDDNum | M | unsigned long | Number of HDDs in storage expansion enclosure -1. | 2 |
| ExtendCabinet1HDDList | C | array | Expansion disk enclosure-1 info list. Optional when ExtendCabinetHDD1Num is 0. | - |
| ExtendCabinet2HDDNum | M | unsigned long | Number of HDDs in storage expansion enclosure -2. | 2 |
| ExtendCabinet2HDDList | C | array | Expansion disk enclosure-2 info list. Optional when ExtendCabinet2HDDNum is 0. | - |
| ContainerInfo | C | Json Block | Container info. See ContainerInfo Json Block. | - |
| NASNum | M | unsigned long | Number of NAS. Applicable to NVR. | 2 |
| NASList | C | array | NAS info list. Optional when NASNum is 0. | - |
| eSATANum | M | unsigned long | Number of eSATA disks Applicable to NVR. | 2 |
| eSATAList | C | array | eSATA info list. Optional when eSATANum is 0. |  |
| ExContainerInfo | M | Json Block | Extended info about storage container. See ExContainerInfo Json Block. | - |

ContainerInfo

| ContainerInfo Json Block | { "ID": , "RemainCapacity": , "TotalCapacity": , "Manufacturer": , "Status": , "Property": , "FormatProgress": , "GroupID": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Storage container ID. | 1 |
| RemainCapacity | M | unsigned long | Remaining space of storage container (MB). | 1907730 |
| TotalCapacity | M | unsigned long | Total capacity of storage container (MB). | 1907730 |
| Manufacturer | 0 | string | Manufacturer name. Length: [1,64]. | er000VX000-9YW1CV12 |
| Status | M | unsigned long | Storage container status. 0: No HDD/idle 1: Not formatted 2: Formatting 3: Healthy 4: Sleep 5: Abnormal 6: Switching 7: Unmounted (for NAS HDDs, applicable to NVR) | 3 |
| Property | C | unsigned long | Disk property. 0: Read/write 1: Read-only 2: Redundant Optional when Status is 0. Applicable to NVR. | 0 |
| FormatProgress | 0 | unsigned long | Formatting progress. Percentage. Required when Status is 2. Applicable to NVR. | 0 |
| GroupID | O | unsigned long | Disk group ID. | 1 |

ExContainerInfo

| ExContainerInfo Json Block | { "ID": , "RemainCapacity": , "TotalCapacity": , "AddressType": , "IPAddress": , "Path": , "UsageType": , "Status": , "Property": , "FormatProgress": , "GroupID": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Storage container ID. | 1 |
| RemainCapacity | M | unsigned long | Remaining space of storage disk (MB). | 1907730 |
| TotalCapacity | M | unsigned long | Total capacity of storage disk (MB). | 1907730 |
| Status | M | unsigned long | Storage disk status. 0: No HDD/idle 1: Not formatted 2: Formatting 3: Healthy 4: Sleep 5: Abnormal 6: Switching 7: Unmounted (for NAS HDDs, not applicable to VMS) | 3 |
| Property | M | unsigned long | Disk property: 0: Read/write 1: Read-only 2: Redundant | 0 |
| FormatProgress | C | unsigned long | Progress. Percentage. Required when Status is 2. | 0 |
| AddressType | C | unsigned long | IP address type. 0: IPv4 (default) 1: IPv6 2: Domain name 3: IPv4 and IPv6 Currently only IPv4 is supported. Required when the storage type is NAS. | 0 |
| IPAddress | C | string | Server IP address. Length: [0,64]. Required when the storage type is NAS. | “192.168.100.30” |
| Path | C | string | Storage path on NAS server. Length: [1,256]. | “/volume2/123456” |
| UsageType | M | unsigned long | Usage. 0: Recording/snapshot 1: Backup | 0 |
| GroupID | C | unsigned long | Disk group ID. The default is 1. | 1 |

RecordScheduleInfo

| RecordScheduleInfo Json Block | { "Enabled": , "IsRedundantStorage": , "RecordRule": { "PreRecordTime": , "PostRecordTime": , }, "WeekPlan":<WeekPlanInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable schedule: 0: Disable 1: Enable | 1 |
| IsRedundantStorage | C | unsigned long | Enable or disable redundant recording. 0: Disable 1: Enable. When enabled, an extra copy of recordings will be stored on the redundant HDD. Absent when the device does not support redundant recording storage. Not applicable to IPC. | 1 |
| RecordRule | M | Json Block | Video recording rules. | - |
| PreRecordTime | M | unsigned long | Length of pre-alarm recording. Unit: Second. Range: 0, 5, 10, 20, 30, 60 | 5 |
| PostRecordTime | M | unsigned long | Length of post-alarm recording. Unit: Second. Range: 5, 10, 30, 60, 120, 300, 600 | 10 |
| WeekPlan | M | Json Block | Scheduled time list. | - |
| WeekPlanInfo | M | Json Block | Detailed schedule info. See Json Block. | - |

PictureScheduleInfo

| PictureScheduleInfo Json Block | { "Enabled": , "IsRedundantStorage": , "WeekPlan":<WeekPlanInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable schedule: 0: Disable 1: Enable | 1 |
| IsRedundantStorage | C | unsigned long | Enable or disable redundant snapshot 0: Disable 1: Enable. When enabled, an extra copy of snapshots will be stored on the redundant HDD. Absent when the device does not support redundant recording storage. Applicable to NVR. | 1 |
| WeekPlan | M | Json Block | Scheduled time list | - |
| WeekPlanInfo | M | Json Block | Detailed arming schedule info. See WeekPlanInfo Json Block. | - |

Alarm

Capabilities

/LAPI/V1.0/Channels/<ID>/Alarm/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/Capabilities |
| --- | --- |
| Description | Get alarm capability of a channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | AlarmCapabilities |
| Note | Applicable to IPC and NVR. |
| Status |  |

MotionDetection

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/AreaType

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/AreaType |
| --- | --- |
| Description | Get the motion detection area type of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | MotionDetectionAreaType |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/AreaType |
| --- | --- |
| Description | Set the motion detection area type of a video channel. |
| Method | PUT |
| Input Data | MotionDetectionAreaType |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle |
| --- | --- |
| Description | Get all motion detection rectangular areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | MotionDetectionRectangleAreaInfoList |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle |
| --- | --- |
| Description | Set all motion detection rectangular areas of a video channel. |
| Method | PUT |
| Input Data | MotionDetectionRectangleAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle/<ID> |
| --- | --- |
| Description | Get a motion detection rectangular area of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | MotionDetectionRectangleAreaInfo |
| Note | Applicable to IPC. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Rectangle/<ID> |
| --- | --- |
| Description | Set a motion detection rectangular area of a video channel. |
| Method | PUT |
| Input Data | MotionDetectionRectangleAreaInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Grid

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Grid |
| --- | --- |
| Description | Get all motion detection grid areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | MotionDetectionGridAreaInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/Areas/Grid |
| --- | --- |
| Description | Set all motion detection grid areas of a video channel. |
| Method | PUT |
| Input Data | MotionDetectionGridAreaInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of motion detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of motion detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of motion detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/MotionDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of motion detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

VideoLoss

/LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/Rule |
| --- | --- |
| Description | Get video loss alarm rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | VideoLossRuleInfo |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/Rule |
| --- | --- |
| Description | Set video loss alarm rules of a video channel. |
| Method | PUT |
| Input Data | VideoLossRuleInfo |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/LinkageActions |
| --- | --- |
| Description | Get linkage actions of video loss alarm. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/LinkageActions |
| --- | --- |
| Description | Set linkage actions of video loss alarm. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/WeekPlan |
| --- | --- |
| Description | Get arming scheduling of video loss alarm. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/VideoLoss/WeekPlan |
| --- | --- |
| Description | Set arming schedule of video loss alarm. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to NVR. |
| Status |  |

TamperDetection

/LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/Rule |
| --- | --- |
| Description | Get tampering detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | TamperDetectionRuleInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/Rule |
| --- | --- |
| Description | Set tampering detection rules of a video channel. |
| Method | PUT |
| Input Data | TamperDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of tampering detection alarm. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of tampering detection alarm. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Linkage actions of tampering detection alarm do not support snapshot and pop-up alarm window. Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of tampering detection alarm. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/TamperDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of tampering detection alarm. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

AudioDetect

/LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/Rule |
| --- | --- |
| Description | Get audio detection rules. |
| Method | GET |
| Input Data | None |
| Success Return Data | AudioDetectInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/Rule |
| --- | --- |
| Description | Set audio detection rules. |
| Method | PUT |
| Input Data | AudioDetectInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of audio detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of audio detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of audio detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/AudioDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of audio detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

HumanShapeDetection

/LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/Rule |
| --- | --- |
| Description | Get human body detection rules. |
| Method | GET |
| Input Data | None |
| Success Return Data | HumanShapeDetectionRuleInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/Rule |
| --- | --- |
| Description | Set human body detection rules. |
| Method | PUT |
| Input Data | HumanShapeDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.1/Channels/<ID>/Alarm/HumanShapeDetection/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/Areas |
| --- | --- |
| Description | Get all rectangular detection areas of human body detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | HumanShapeDetectAreaInfoList |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/Areas |
| --- | --- |
| Description | Set all rectangular detection areas of human body detection. |
| Method | PUT |
| Input Data | HumanShapeDetectAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of human body detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of human body detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of human body detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Alarm/HumanShapeDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of human body detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

Json Block

AlarmCapabilities

| AlarmCapabilities Json Block | { "MotionDetection": { "IsSupportCfg": , "SupportedTypeNum":, "SupportedTypeList":[...] , "RectangleAreaNum ": , "BlockWidth": , "BlockHeight": }, "TamperDetection": { "IsSupportCfg": , "TamperRECTAreaNum": }, "AudioDetection": { "SupportCfg": } "HumanShapeDetection": { "SupportCfg": , "RectangleAreaNum": , "LinkagePlanCfg": { "SupportAudioLinkagePlan":, "SupportLightLinkagePlan": } } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MotionDetection | M | Json Block | Motion detection capability info. | - |
| IsSupportCfg | M | boolean | Whether is configurable: 0: No 1: Yes | 1 |
| SupportedTypeNum | C | unsigned long | Number of motion detection types supported. Applicable to IPC. | 2 |
| SupportedTypeList | C | array | Supported motion detection types 0: Rectangular area 1: Grid area Applicable to IPC. | [0, 1] |
| RectangleAreaNum | C | unsigned long | Number of rectangular detection areas supported. Optional when rectangular area is not supported. Applicable to IPC. |  |
| BlockWidth | M | unsigned long | Maximum value of grid width. | 22 |
| BlockHeight | M | unsigned long | Maximum value of grid height. | 18 |
| TamperDetection | M | Json Block | Tampering detection capability info. | - |
| IsSupportCfg | M | boolean | Whether the connected device supports configuration: 0: No 1: Yes | 1 |
| TamperRECTAreaNum | M | unsigned long | Number of tampering detection areas that can be drawn. Currently fixed to 1. | 1 |
| AudioDetection | M | Json Block | Audio detection capability info. |  |
| SupportCfg | M | boolean | Whether is configurable: 0: No 1: Yes | 1 |
| HumanShapeDetection | M | Json Block | Human body detection capability info. | - |
| SupportCfg | M | boolean | Whether is configurable: 0: No 1: Yes | 1 |
| RectangleAreaNum | M | unsigned long | Number of human body detection areas that can be drawn. Currently fixed to 1. | 1 |
| LinkagePlanCfg | C | Json Block | Arming schedule of linkage actions. Required when supported. |  |
| SupportAudioLinkagePlan | M | boolean | Whether audio linkage schedule is supported. 0: No 1: Yes | 1 |
| SupportLightLinkagePlan | M | boolean | Whether light linkage schedule is supported. 0: No 1: Yes | 1 |

HumanShapeDetectionRuleInfo

| HumanShapDetectionRuleInfo Json Blcok | { "Enabled":, "DetectDisplayEnabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable human body detection 0: Disable 1: Enable | 1 |
| DetectDisplayEnabled | C | boolean | Enable or disable bounding box of human body detection 0: Disable 1: Enable | 1 |

HumanShapeDetectAreaInfoList

| HumanShapeDetectAreaInfoList Json Block | { "Num": , "PolygonInfoList":[<HumanShapeDetectPolygonInfo>, ……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of detection areas. | 1 |
| PolygonInfoList | C | array | Detection area info list. Optional when Num is 0. | - |
| HumanShapeDetecPolygonInfo | M | Json Block | Detection area info. See HumanShapeDetectPolygonInfo Json Block | - |

HumanShapeDetectPolygonInfo

| HumanShapeDetectPolygonInfo Json Block | { "ID": , "Enabled": , "DisplayEnabled":, "Sensitivity": , "TargetSize": , "PointNum": , "PointList": [ { "X": , "Y": }, … ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Detection area ID, starts from 0. Use the /LAPI/V1.1/Channels/<ID>/Alarm/Capabilities interface to obtain the maximum number allowed. | 1 |
| Enabled | M | boolean | Enable or disable detection area: 0: Disable 1: Enable | 1 |
| DisplayEnabled | M | boolean | Enable or disable detection area display 0: Disable 1: Enable Indicates whether to show the detection box. | 1 |
| Sensitivity | M | unsigned long | Motion detection sensitivity. The greater the value, the higher the sensitivity. Range: [1,100] | 50 |
| TargetSize | M | unsigned long | Object size. Range: [1,100] | 1 |
| PointNum | M | unsigned long | Number of vertices of the polygon detection area. Range: [0,6] | 4 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Unit: ‱. Range: [0,9999] | 0 |
| Y | M | unsigned long | y-coordinate. Unit: ‱. Range: [0,9999] | 0 |

LinkageActionList

| LinkageActionList Json Block | { "Num": , "Actions":[<LinkageActionInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of linkage actions. | 8 |
| Actions | C | array | Linkage action info list. Optional when Num is 0. | - |
| LinkageActionInfo | M | Json Block | Linkage action. See LinkageActionInfo Json Block. | - |

LinkageActionInfo

| LinkageActionInfo Json Block | { "Enabled": , "ActID": , "ActParam": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | C | boolean | Enable or disable linkage action. Required when ActID is greater than or equal to 28. 0: Disable 1: Enable (For compatibility, any value that is less than 28, even when present, is invalid) | 1 |
| ActID | M | unsigned long | See the table below for ActIDs. | 5 |
| ActParam | C | Json Block | See the table below for the correspondence relationship between ActID and ActParam. |  |

| ActID | Description | IPC ActParam | NVR ActParam |
| --- | --- | --- | --- |
| 0 | NVR preview (applicable to NVR) | Not Support | ChannelActParamInfo |
| 1 | Buzzer (not available to IPC) | Not Support | EnabledActParamInfo |
| 2 | E-mail (applicable to NVR) | Not Support | EnabledActParamInfo |
| 3 | Recording | Not Support | ChannelActParamInfo |
| 4 | Go to PTZ preset | PresetActParamInfo | PresetActParamInfo |
| 5 | Output switch | OutputSwitchActParamInfo | OutputSwitchActParamInfo |
| 6 | Snapshot | Null | ChannelActParamInfo |
| 7 | Pop-up alarm window (applicable to NVR) | Not Support | EnabledActParamInfo |
| 8 | Central recording | Null | Not Support |
| 9 | Start local recording | Null | Not Support |
| 10 | Stop local recording | Null | Not Support |
| 11 | Snapshot and then upload via FTP | Null | Not Support |
| 12 | Snapshot and then send via e-mail | Null | Not Support |
| 13 | Snapshot and then upload via email and FTP | Null | Not Support |
| 14 | Smart snapshot and upload | Null | Not Support |
| 15 | Upload close-up image of face | Null | Not Support |
| 16 | Alarm notification | Null | Not Support |
| 28 | Sound alarm | AudioLinkagePlanInfo | AudioLinkagePlanInfo |
| 29 | Light alarm | LightLinkagePlanInfo | LightLinkagePlanInfo |

AudioLinkagePlanInfo

| AudioLinkagePlanInfo Json Block | { "TimeMode": , "AudioCustomInfo": <AudioCustomInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TimeMode | M | unsigned long | Type of linkage time period. 0: Custom time period | 0 |
| AudioCustomInfo | C | Json Block | Detailed arming schedule info when the linkage action is sound alarm. Required when in PUT messages and TimeMode is 0, or when in GET messages. See AudioCustomInfo Json Block. |  |

AudioCustomInfo

| AudioCustomInfo Json Block | { "Num": , "Days": [<AudioCustomDayPlanInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of scheduled days IPC: Fixed to 7 (seven days a week). | 7 |
| Days | M | array | Arming schedule of every day in the week. |  |
| AudioCustomDayPlanInfo | M | Json Block | Detailed info about arming schedule of every day. See AudioCustomDayPlanInfo Json Block. |  |

AudioCustomDayPlanInfo

| AudioCustomDayPlanInfo Json Block | { "ID": , "Num": , "TimeSectionInfos": [<AudioTimeSectionInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of days 1: Monday 2: Tuesday 3: Wednesday 4: Thursday 5: Friday 6: Saturday 7: Sunday | 1 |
| Num | M | unsigned long | Number of time periods per day. IPC: Fixed to 4. | 4 |
| TimeSectionInfos | M | array | Time periods of arming schedule. Time periods on the same day must not overlap. |  |
| AudioTimeSectionInfo | M | Json Block | Detailed audio arming schedule. See AudioTimeSectionInfo. |  |

AudioTimeSectionInfo

| AudioTimeSectionInfo Json Block | { "Enabled": , "Begin": , "End": , "AudioActParamInfo":<AudioActParamInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Enable or disable arming time periods. 0: Disable 1: Enable | 0 |
| Begin | M | string | Start time. Format: hh:mm:ss Length range: [0,31] | 00:00:00 |
| End | M | string | End time. Format: hh:mm:ss Length range: [0,31] The end time must be later than the start time. | 23:59:59 |
| AudioActParamInfo | M | Json Block | Sound alarm configuration. See AudioActParamInfo Json Block. |  |

LightLinkagePlanInfo

| LightLinkagePlanInfo Json Block | { "TimeMode": , "LightCustomInfo": <LightCustomInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TimeMode | M | unsigned long | Type of linkage time period. 0: Custom time period | 0 |
| LightCustomInfo | C | Json Block | Detailed arming schedule info when the linkage action is light alarm. Required when in PUT messages and TimeMode is 0, or when in GET messages. See LightCustomInfo Json Block. |  |

LightCustomInfo

| LightCustomInfo Json Block | { "LightActParamInfo":<LightActParamInfo>, "LightWeekPlanInfo":<LightWeekPlanInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LightActParamInfo | M | Json Block | Light alarm configuration. See LightActParamInfo Json Block. |  |
| LightWeekPlanInfo | M | Json Block | Detailed info about light arming schedule. See LightWeekPlanInfo Json Block |  |

LightWeekPlanInfo

| LightWeekPlanInfo Json Block | { "Num": , "Days": [<LightDayPlanInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of scheduled days IPC: Fixed to 7 (seven days a week). | 7 |
| Days | M | array | Arming schedule of every day in the week. |  |
| LightDayPlanInfo | M | Json Block | Detailed info about daily arming schedule. See LightDayPlanInfo Json Block |  |

LightDayPlanInfo

| LightDayPlanInfo Json Block | { "ID": , "Num": , "TimeSectionInfos": [<LightTimeSectionInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of days 1: Monday 2: Tuesday 3: Wednesday 4: Thursday 5: Friday 6: Saturday 7: Sunday | 1 |
| Num | M | unsigned long | Number of time periods per day IPC: Fixed to 4. | 4 |
| TimeSectionInfos | M | array | Time periods of arming schedule. Time periods on the same day must not overlap. |  |
| LightTimeSectionInfo | M | Json Block | Detailed light arming schedule. See LightTimeSectionInfo. |  |

LightTimeSectionInfo

| LightTimeSectionInfo Json Block | { "Enabled": , "Begin": , "End": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Enable or disable arming time periods. 0: Disable 1: Enable | 0 |
| Begin | M | string | Start time. Format: hh:mm:ss Length range: [0,31] | 00:00:00 |
| End | M | string | End time. Format: hh:mm:ss Length range: [0,31] The end time must be later than the start time. | 23:59:59 |

EnabledActParamInfo

| EnabledActParamInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | 1: Linkage action enabled 0: Linkage action disabled | 1 |

ChannelActParamInfo

| ChannelActParamInfo Json Block | { "Num":, "IDs": [<ID>,<ID>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of video channels. |  |
| IDs | C | array | Video channel IDs. |  |
| ID | C | unsigned long | Video channel ID | 1 |

PresetActParamInfo

| PresetActParamInfo Json Block | { "Num":, "ChannelPresetList": [<ChannelPreset>, <ChannelPreset>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of linkage actions. | 2 |
| ChannelPresetList | C | array | Info list of linked presets. Optional when Num is 0. |  |
| ChannelPreset | C | Json block | Channel and preset ID. | See ChannelPreset Json block |

ChannelPreset

| ChannelPreset Json Block | { "ChannelID":, "PresetID": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ChannelID | C | unsigned long | Video channel ID. Optional for IPC. | 1 |
| PresetID | M | unsigned long | Preset ID. | 1 |

OutputSwitchActParamInfo

| OutputSwitchActParamInfo Json Block | { "Num":, "IDs":[<ID>,<ID>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of linked output switches. | M |
| IDs | C | array | List of linked output switches. | C |
| ID | C | unsigned long | Output switch channel ID. For NVR, the ID is video channel ID*100 + alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |

DayPlanInfo

| DayPlanInfo Json Block | { "ID": , "Num": , "TimeSectionInfos": [<TimeSectionInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of days 1: Monday 2: Tuesday 3: Wednesday 4: Thursday 5: Friday 6: Saturday 7: Sunday 8: Holiday Remarks: 8 is applicable to NVR and VMS. | 1 |
| Num | M | unsigned long | Number of time periods per day. NVR: Max 8. IPC: Max 4. | 8 |
| TimeSectionInfos | C | array | Time periods of arming schedule. Time periods on the same day must not overlap. Optional when Num is 0. |  |
| TimeSectionInfo | C | Json Block | Detailed arming schedule. See TimeSectionInfo Json Block. Optional when Num is 0. |  |

WeekPlanInfo

| WeekPlanInfo Json Block | { "Enabled": , "Num": , "Days": [<DayPlanInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | C | boolean | Enable or disable arming schedule. 0: Disable 1: Enable Applicable to IPC only. Default: Enable. | 1 |
| Num | M | unsigned long | Number of scheduled days. NVR: Max 8 (7 days of a week plus holiday). IPC: Max 7 (7 days of a week). | 8 |
| Days | M | array | Arming schedule of every day in the week. |  |
| DayPlanInfo | M | Json Block | Detailed info about daily arming schedule. See DayPlanInfo Json Block |  |

TimeSectionInfo

| TimeSectionInfo Json Block | { "Begin": , "End": , "ArmingType": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Begin | M | string | Start time. Format: hh:mm:ss Length range: [0,31]. NVR does not support ss (seconds). | 00:00:00 |
| End | M | string | End time. Format: hh:mm:ss Length range: [0,31]. NVR does not support ss (seconds). | 24:00:00 |
| ArmingType | C | unsigned long | Arming type. 0: Normal 1: Motion detection 2: Alarm 3: Motion detection and alarm 4: Motion detection or alarm 5: No schedule 10: Event Recording and snapshot support all; other linkage actions support Normal only. For IPC this field is optional because IPC only supports Normal (default setting). Not applicable to IPC. |  |

VideoLossRuleInfo

| VideoLossRuleInfo Json Blcok | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable 0: Disable 1: Enable | 1 |

TamperDetectionRuleInfo

| TamperDetectionRuleInfo Json Blcok | { "Enabled": , "Sensitivity": , "Duration": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 50 |
| Duration | C | unsigned long | Duration (unit: second). Range: [0, 10]. Applicable to IPC. | 1 |

MotionDetectionAreaType

| MotionDetectionAreaType Json Block | { "Type": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Area type. Currently two types: 0: Rectangular area 1: Grid area | 1 |

MotionDetectionRectangleAreaInfo

| MotionDetectionRectangleAreaInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "Duration": , "TargetSize": , "Area": { "TopLeft": { "X": 0, "Y": 0 }, "BottomRight": { "X": 10, "Y": 10 } } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Rectangle area ID, starts from 0. Use the /LAPI/V1.0/Channels/<ID>/Alarm/Capabilities interface to get the maximum number of areas allowed. | 1 |
| Enabled | M | boolean | Enable or disable area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Motion detection sensitivity. The greater the value, the higher the sensitivity. Range: [1,100]. | 50 |
| Duration | C | unsigned long | Duration. Range: [1,100]. Applicable to IPC. | 5 |
| TargetSize | C | unsigned long | Object size. Range: [1,100]. Applicable to IPC. | 1 |
| RectangleArea | M | Json Block | Rectangular area info. | - |
| TopLeft | M | Json Block | Coordinates of the rectangular area’s top left corner. | - |
| X | M | unsigned long | x-coordinate. Range: [0,9999]. | 0 |
| Y | M | unsigned long | y-coordinate. Range: [0,9999]. | 0 |
| BottomRight | M | Json Block | Coordinates of the rectangular area’s bottom right corner. | - |
| X | M | unsigned long | x-coordinate. Range: [0,9999]. | 10 |
| Y | M | unsigned long | y-coordinate. Range: [0,9999]. | 10 |

MotionDetectionRectangleAreaInfoList

| MotionDetectionRectangleAreaInfoList Json Block | { "Num": , "RectangleAreas": [ <MotionDetectionRectangleAreaInfo>, ……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of rectangular areas. | 1 |
| RectangleAreas | C | array | Rectangular area info list. Optional when Num is 0. | - |
| MotionDetectionRectangleAreaInfo | M | Json Block | Rectangular area info. See MotionDetectionRectangleAreaInfo Json Block. | - |

MotionDetectionGridAreaInfo

| MotionDetectionGridAreaInfo Json Block | { "Enabled": , "Sensitivity": , "Grid": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Motion detection sensitivity. The greater the value, the higher the sensitivity. Range: [1,100]. | 50 |
| Grid | M | string | Detection area. Length range: [0, 256] 1. The entire screen is divided into 22x18 grids, starting from the upper left corner to the lower right corner (from left to right, top to bottom), a total of 396 grids, numbered sequentially. Then the 8-bit number (maximum 255) represented by binary bits indicates the selection status of the motion detection area of 8 grids: 1 means “selected”, 0 means “not selected” (from low order to high order). This converts all grids into 50 arrays of 8-bit numbers. 2. The array is first compressed by the PackBits algorithm, and then encoded with Base64 to obtain the compressed string. | "DWAAAYAABj/5mP/mAAAY/gAgP/4A//77//vv/++//74ABvgBG+AAb4ABvh/2+H/YAf8A" |

AudioDetectInfo

| AudioDetectInfo Json Block | { "Enabled": , "DetectType": , "DiffValue": , "Threshold": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable audio detection: 0: Disable 1: Enable | 1 |
| DetectType | M | unsigned long | Audio detection type: 0: Sudden rise 1: Sudden fall 2: Sudden change 3: Threshold | 0 |
| DiffValue | C | unsigned long | Difference value, which is used when DetectType is “Sudden Rise”, “Sudden Fall”, “Sudden Change”. Range: [0,400]. An alarm is triggered when the value of sudden rise, sudden fall, or sudden change exceeds the threshold. | 100 |
| Threshold | C | unsigned long | Threshold. Valid when audio detection type is “Threshold”. Range: [0,400]. An alarm is triggered when the sound volume exceeds the threshold. | 100 |

IO

InputSwitch

/LAPI/V1.0/IO/InputSwitches/BasicInfos?[DevID=<DevID>]&[OrgID=<OrgID>]

| URL | /LAPI/V1.0/IO/InputSwitches/BasicInfos?[DevID=<DevID>]&[OrgID=<OrgID>] |
| --- | --- |
| Description | Get detailed info list of multiple alarm input channels. |
| Method | GET |
| Input Data | None |
| Success Return Data | InputSwitchBasicInfoList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DevID | 0 | unsigned long | Device ID. Null means searching all devices. Applicable to VMS. | 21 |
| OrgID | 0 | unsigned long | Organization ID. Null means searching all organizations. Applicable to VMS. | 55 |

/LAPI/V1.0/IO/InputSwitches/<ID>/BasicInfos

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/BasicInfos |
| --- | --- |
| Description | Get an alarm input channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | InputSwitchBasicInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/BasicInfos |
| --- | --- |
| Description | Set an alarm input channel. |
| Method | PUT |
| Input Data | InputSwitchBasicInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Alarm input channel ID. For NVR, the ID is channel ID*100+alarm input ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 108 |

/LAPI/V1.0/IO/InputSwitches/<ID>/LinkageActions

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/LinkageActions |
| --- | --- |
| Description | Get linkage actions of an input switch. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/LinkageActions |
| --- | --- |
| Description | Set linkage actions of an input switch. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/IO/InputSwitches/<ID>/WeekPlan

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/WeekPlan |
| --- | --- |
| Description | Get arming schedule of input switch. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/IO/InputSwitches/<ID>/WeekPlan |
| --- | --- |
| Description | Set arming schedule of input switch. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

OutputSwitch

/LAPI/V1.0/IO/OutputSwitches/BasicInfos?[DevID=<DevID>]&[OrgID=<OrgID>]

| URL | /LAPI/V1.0/IO/OutputSwitches/BasicInfos?[DevID=<DevID>]&[OrgID=<OrgID>] |
| --- | --- |
| Description | Get detailed info list of multiple alarm output channels. |
| Method | GET |
| Input Data | None |
| Success Return Data | OutputSwitchBasicInfoList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DevID | 0 | unsigned long | Device ID. Null means searching all devices. Applicable to VMS. | 21 |
| OrgID | 0 | unsigned long | Organization ID. Null means searching all organizations. Applicable to VMS. | 55 |

/LAPI/V1.0/IO/OutputSwitches/<ID>/BasicInfos

| URL | /LAPI/V1.0/IO/OutputSwitches/<ID>/BasicInfos |
| --- | --- |
| Description | Get info about alarm output channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | OutputSwitchBasicInfo |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

| URL | /LAPI/V1.0/IO/OutputSwitches/<ID>/BasicInfos |
| --- | --- |
| Description | Set info about alarm output channel. |
| Method | PUT |
| Input Data | OutputSwitchBasicInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Alarm output channel ID. | 99 |

/LAPI/V1.0/IO/OutputSwitches/<ID>/WeekPlan

| URL | /LAPI/V1.0/IO/OutputSwitches/<ID>/WeekPlan |
| --- | --- |
| Description | Get arming schedule of a device’s output switch. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/IO/OutputSwitches/<ID>/WeekPlan |
| --- | --- |
| Description | Set arming schedule of a device’s output switch. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/IO/OutputSwitches/ManualAlarm

| URL | /LAPI/V1.0/IO/OutputSwitches/ManualAlarm |
| --- | --- |
| Description | Manually control output switch alarm. |
| Method | PUT |
| Input Data | SetOutputSwitchManualAlarmList |
| Success Return Data | None |
| Note | Applicable to NVR and VMS. |
| Status |  |

/LAPI/V1.0/IO/OutputSwitches/AlarmStatus

| URL | /LAPI/V1.0/IO/OutputSwitches/AlarmStatus |
| --- | --- |
| Description | Get alarm status of output switch. |
| Method | GET |
| Input Data | None |
| Success Return Data | OutputSwitchAlarmStatusList |
| Note | Applicable to IPC, NVR, and VMS. |
| Status |  |

Json Block

InputSwitchBasciInfo

| InputSwitchDetailInfo Json Block | { "ID": , "Name": , "Status": , "GBID": , “VMSCommon”: , "RunMode": , "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Alarm input channel ID. For NVR, the ID is the video channel ID*100+alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |
| Name | C | string | Channel name. Length: [1, 64]. Currently not applicable to NVR. | "IP Camera 01" |
| Status | C | unsigned long | Channel status 0: Offline 1: Online Applicable to VMS | 0 |
| GBID | C | string | GB/T28181 resource ID Length range: [0, 20]. Applicable to VMS and IPC. | "34000000002000000010" |
| VMSCommon | C | Json Block | Common VMS information | See VMSCommon Json Block. |
| RunMode | M | unsigned long | Alarm type 1: Normally open 2: Normally closed | 1 |
| Enabled | M | boolean | Enable or disable alarm 0: Disable 1: Enable | 1 |

InputSwitchBasicInfoList

| InputSwitchBasicInfoList Json Block | { "Num": , "BasicInfos":[<InputSwitchBasicInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of alarm input channels. | 4 |
| BasicInfos | C | array | Detailed info list of alarm input channels. Optional when Num is 0. | - |
| InputSwitchBasicInfo | M | Json Block | Detailed info about alarm input channel. See InputSwitchBasicInfo Json Block. | - |

OutputSwitchBasicInfo

| OutputSwitchBasicInfo Json Block | { "ID": , "Name": , "Status": , “VMSCommon”: , "RunMode": , "Duration": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Output switch channel ID. For NVR, the ID is video channel ID*100+alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |
| Name | C | string | Channel name. Length: [1, 64]. Currently not applicable to NVR. | "O_AlarmOutput_2" |
| Status | C | unsigned long | Channel status. 0: Offline 1: Online Applicable to VMS. | 0 |
| VMSCommon | C | Json Block | Common VMS information | See VMSCommon Json Block |
| RunMode | M | unsigned long | Alarm type. 1: Normally open 2: Normally closed | 1 |
| Duration | M | unsigned long | Duration. Range: NVR, VMS and ordinary IPC: [5, 3600]. Unit: second. LPR camera: [500, 3600000]. Unit: millisecond. Smart camera: [1, 3600]. Unit: second. All the ranges need to be converted to time values in unit of millisecond. | 6 |

OutputSwitchBasicInfoList

| OutputSwitchBasicInfoList Json Block | { "Num": , "BasicInfos":[<OutputSwitchBasicInfo>,……] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of alarm output channels. | 4 |
| BasicInfos | C | array | Detailed info list of alarm output channels. Optional when Num is 0. | - |
| OutputSwitchBasicInfo | M | Json Block | Detailed info about alarm output channel. See OutputSwitchBasicInfo Json Block。 | - |

SetOutputSwitchManualAlarmList

| SetOutputSwitchManualAlarmList Json Block | { "Num":, "IDs": [<ID>, …], "AlarmAction": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of output switch channels whose alarm status has changed. | 4 |
| IDs | C | array | Output switch ID list. Optional when Num is 0. | - |
| ID | M | unsigned long | Output switch ID. For NVR, the value is video channel ID*100 + alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |
| AlarmAction | M | unsigned long | Alarm action of output switch. 0: Close alarm 1: Trigger alarm | 1 |

OutputSwitchStatus

| OutputSwitchStatus Json Block | { "ID": , "Status": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | ID is channel ID*100+alarm output ID. The channel ID of the device itself is 0. For example, the first output switch ID on IPC is 0*100+1=1. | 1 |
| Status | M | unsigned long | Output switch status. 0: Close, closed circuit 1: Open, open circuit | 0 |

OutputSwitchStatusList

| OutputSwitchStatusList Json Block | { "Nums":, "Statuses":[<OutputSwitchStatus>, <OutputSwitchStatus>...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Number of output switches. | 1 |
| Statuses | C | array | Output switch list. Optional when Nums is 0. | - |
| OutputSwitchStatus | M | Json Block | Output switch info. See OutputSwitchStatus Json Block. | - |

OutputSwitchAlarmStatusList

| OutputSwitchAlarmStatusList Json Block | { "Num": , "AlarmStatusList": [<AlarmStatusInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of alarm output switch channels | 1 |
| AlarmStatusList | C | array | List of alarm status of output switch. Optional when Num is 0. | - |
| AlarmStatusInfo | C | Json Block | Status of output switch alarm. See AlarmStatusInfo Json Block | - |

AlarmStatusInfo

| AlarmStatusInfo Json Block | { "ID": , "AlarmStatus": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Output switch ID. For NVR, the ID is video channel ID*100 + alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |
| AlarmStatus | M | unsigned long | Alarm status of output switch. 0: No alarm 1: Alarm | 1 |

Smart

Capabilities

/LAPI/V1.0/Channels/<ID>/Smart/Capabilities

| URL | /LAPI/V1.0/Channels/<ID>/Smart/Capabilities |
| --- | --- |
| Description | Get channel’s smart capability. |
| Method | GET |
| Input Data | None |
| Success Return Data | SmartCapInfo |
| Note | Applicable to IPC. |

CrossLineDetection

/LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Rule |
| --- | --- |
| Description | Get cross line detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | CrossLineDetectionRuleInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Rule |
| --- | --- |
| Description | Set cross line detection rules of a video channel. |
| Method | PUT |
| Input Data | CrossLineDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas |
| --- | --- |
| Description | Get all cross line detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | CrossLineDetectionAreaInfoList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas |
| --- | --- |
| Description | Set all cross line detection areas of a video channel. |
| Method | PUT |
| Input Data | CrossLineDetectionAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas/<ID> |
| --- | --- |
| Description | Get a cross line detection area of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | CrossLineDetectionLineInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/Areas/<ID> |
| --- | --- |
| Description | Set a cross line detection area of a video channel. |
| Method | PUT |
| Input Data | CrossLineDetectionLineInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of cross line detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of cross line detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of cross line detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/CrossLineDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of cross line detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

IntrusionDetection

/LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Rule |
| --- | --- |
| Description | Get intrusion detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | IntrusionDetectionRuleInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Rule |
| --- | --- |
| Description | Set intrusion detection rules of a video channel. |
| Method | PUT |
| Input Data | IntrusionDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas |
| --- | --- |
| Description | Get all intrusion detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | IntrusionDetectionAreaInfoList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas |
| --- | --- |
| Description | Set all intrusion detection areas of a video channel. |
| Method | PUT |
| Input Data | IntrusionDetectionAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas/<ID> |
| --- | --- |
| Description | Get area info of intrusion detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | IntrusionDetectionPolygonInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/Areas/<ID> |
| --- | --- |
| Description | Set area info of intrusion detection. |
| Method | PUT |
| Input Data | IntrusionDetectionPolygonInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of intrusion detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of intrusion detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

/LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of intrusion detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |
| Status |  |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/IntrusionDetection/WeekPlan |
| --- | --- |
| Description | Set arming scheduling of intrusion detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |
| Status |  |

LeaveZone

/LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Rule |
| --- | --- |
| Description | Get leave area detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | LeaveZoneRuleInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Rule |
| --- | --- |
| Description | Set leave area detection rules of a video channel. |
| Method | PUT |
| Input Data | LeaveZoneRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas |
| --- | --- |
| Description | Get all leave area detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | LeaveZoneAreaInfoList |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas |
| --- | --- |
| Description | Set all leave area detection areas of a video channel. |
| Method | PUT |
| Input Data | LeaveZoneAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas/<ID> |
| --- | --- |
| Description | Get area info of leave area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | ZonePolygonInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/Areas/<ID> |
| --- | --- |
| Description | Set area info of leave area detection. |
| Method | PUT |
| Input Data | ZonePolygonInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/LinkageActions |
| --- | --- |
| Description | Get linkage actions of leave area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/LinkageActions |
| --- | --- |
| Description | Set linkage actions of leave area detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/WeekPlan |
| --- | --- |
| Description | Get arming schedule of leave area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/LeaveZone/WeekPlan |
| --- | --- |
| Description | Set arming schedule of leave area detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

AccessZone

/LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Rule |
| --- | --- |
| Description | Get enter area detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | AccessZoneRuleInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Rule |
| --- | --- |
| Description | Set enter area detection rules of a video channel. |
| Method | PUT |
| Input Data | AccessZoneRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas |
| --- | --- |
| Description | Get all enter area detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | AccessZoneAreaInfoList |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas |
| --- | --- |
| Description | Set all enter area detection areas of a video channel. |
| Method | PUT |
| Input Data | AccessZoneAreaInfoList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas/<ID> |
| --- | --- |
| Description | Get area info of enter area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | ZonePolygonInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/Areas/<ID> |
| --- | --- |
| Description | Set area info of enter area detection. |
| Method | PUT |
| Input Data | ZonePolygonInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/AccessZone/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/LinkageActions |
| --- | --- |
| Description | Get linkage actions of enter area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/LinkageActions |
| --- | --- |
| Description | Set linkage actions of enter area detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

/LAPI/V1.0/Channels/<ID>/Smart/AccessZone/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/WeekPlan |
| --- | --- |
| Description | Get arming schedule of enter area detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC and NVR. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AccessZone/WeekPlan |
| --- | --- |
| Description | Set arming schedule of enter area detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC and NVR. |

FaceDetection

/LAPI/V1.0/Channels/<ID>/Smart/FaceEnable

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceEnable |
| --- | --- |
| Description | Get enabled/disabled status of face related smart functions (face and body) of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | FaceEnableInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceEnable |
| --- | --- |
| Description | Enable/disable face related smart functions (face and body) of a video channel. |
| Method | PUT |
| Input Data | FaceEnableInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Rule |
| --- | --- |
| Description | Get face detection rules of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | FaceDetectionRuleInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Rule |
| --- | --- |
| Description | Set face detection rules of a video channel. |
| Method | PUT |
| Input Data | FaceDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections |
| --- | --- |
| Description | Get all face detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | FaceDetectionAreaList |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections |
| --- | --- |
| Description | Set all face detection areas of a video channel. |
| Method | PUT |
| Input Data | FaceDetectionAreaList |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections/<ID>

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections/<ID> |
| --- | --- |
| Description | Get face detection areas of a video channel. |
| Method | GET |
| Input Data | None |
| Success Return Data | FaceDetectionPolygonInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/Areas/Detections/<ID> |
| --- | --- |
| Description | Set face detection areas of a video channel. |
| Method | PUT |
| Input Data | FaceDetectionPolygonInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of face detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of face detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/WeekPlans

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/WeekPlans |
| --- | --- |
| Description | Get arming schedule of face detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/WeekPlans |
| --- | --- |
| Description | Set arming schedule of face detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/FaceCount/Reset

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/FaceCount/Reset |
| --- | --- |
| Description | Get configuration of face counting reset. |
| Method | GET |
| Input Data | None |
| Success Return Data | CountResetInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/FaceDetection/FaceCount/Reset |
| --- | --- |
| Description | Set configuration of face counting reset |
| Method | PUT |
| Input Data | CountResetInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

HybridDetection

/LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Rule

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Rule |
| --- | --- |
| Description | Get mixed traffic detection rules. |
| Method | GET |
| Input Data | None |
| Success Return Data | HybridDetectionRuleInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Rule |
| --- | --- |
| Description | Set mixed traffic detection rules. |
| Method | PUT |
| Input Data | HybridDetectionRuleInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Areas

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Areas |
| --- | --- |
| Description | Get areas of mixed traffic detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | HybridDetectionAreaCfg |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/Areas |
| --- | --- |
| Description | Set areas of mixed traffic detection. |
| Method | PUT |
| Input Data | HybridDetectionAreaCfg |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/LinkageActions

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/LinkageActions |
| --- | --- |
| Description | Get linkage actions of mixed traffic detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | LinkageActionList |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/LinkageActions |
| --- | --- |
| Description | Set linkage actions of mixed traffic detection. |
| Method | PUT |
| Input Data | LinkageActionList |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/WeekPlan

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/WeekPlan |
| --- | --- |
| Description | Get arming schedule of mixed traffic detection. |
| Method | GET |
| Input Data | None |
| Success Return Data | WeekPlanInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/HybridDetection/WeekPlan |
| --- | --- |
| Description | Set arming schedule of mixed traffic detection. |
| Method | PUT |
| Input Data | WeekPlanInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

AttributeCollect

/LAPI/V1.0/Channels/<ID>/Smart/AttributeCollect

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AttributeCollect |
| --- | --- |
| Description | Get attribute collection info. |
| Method | GET |
| Input Data | None |
| Success Return Data | AttributeCollectInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AttributeCollect |
| --- | --- |
| Description | Set attribute collection info. |
| Method | PUT |
| Input Data | AttributeCollectInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

/LAPI/V1.0/Channels/<ID>/Smart/AttributeParam

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AttributeParam |
| --- | --- |
| Description | Get attribute parameters (province and first letter of license plate, and pupillary distance). |
| Method | GET |
| Input Data | None |
| Success Return Data | AttributeParamInfo |
| Note | Applicable to IPC. |

| URL | /LAPI/V1.0/Channels/<ID>/Smart/AttributeParam |
| --- | --- |
| Description | Set attribute parameters (province and first letter of license plate, and pupillary distance). |
| Method | PUT |
| Input Data | AttributeParamInfo |
| Success Return Data | None |
| Note | Applicable to IPC. |

Json Block

SmartCapInfo

| SmartCapInfo Json Block | { "IntrusionDetection": <IntrusionDetectionCapInfo>, "CrossLineDetection": <CrossLineDetectionCapInfo>, "AccessZone": <AccessZoneCapInfo>, "LeaveZone": <LeaveZoneCapInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IntrusionDetection | M | Json Block | Intrusion detection capability. See IntrusionDetectionCapInfo Json Block. | -- |
| CrossLineDetection | M | Json Block | Cross line detection capability. See CrossLineDetectionCapInfo Json Block. | -- |
| AccessZone | M | Json Block | Enter area detection capability. See AccessZoneCapInfo Block. | - |
| LeaveZone | M | Json Block | Leave area detection capability. See LeaveZoneCapInfo Block. |  |

IntrusionDetectionCapInfo

| IntrusionDetectionCapInfo Json Block | { "SupportCfg":, "Mode":, "LinkagePlanCfg": { "SupportAudioLinkagePlan":, "SupportLightLinkagePlan": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCfg | M | boolean | Whether intrusion detection is supported. 0: No 1: Yes | 0 |
| Mode | 0 | unsigned long | Smart mode: 0: Ordinary 1: Deep Learning | 0 |
| LinkagePlanCfg | C | Json Block | Arming schedule of linkage actions. Required when supported. |  |
| SupportAudioLinkagePlan | M | boolean | Whether audio linkage schedule is supported. 0: No 1: Yes | 1 |
| SupportLightLinkagePlan | M | boolean | Whether light linkage schedule is supported. 0: No 1: Yes | 1 |

CrossLineDetectionCapInfo

| CrossLineDetectionCapInfo Json Block | { "SupportCfg": , "Mode":, "LinkagePlanCfg": { "SupportAudioLinkagePlan":, "SupportLightLinkagePlan": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCfg | M | boolean | Whether cross line detection is supported: 0: No 1: Yes | 0 |
| Mode | 0 | unsigned long | Smart mode: 0: Ordinary 1: Deep Learning | 0 |
| LinkagePlanCfg | C | Json Block | Arming schedule of linkage actions. Required when supported. |  |
| SupportAudioLinkagePlan | M | boolean | Whether audio linkage schedule is supported. 0: No 1: Yes | 1 |
| SupportLightLinkagePlan | M | boolean | Whether light linkage schedule is supported. 0: No 1: Yes | 1 |

AccessZoneCapInfo

| AccessZoneCapInfo Json Block | { "SupportCfg":, "Mode":, "LinkagePlanCfg": { "SupportAudioLinkagePlan":, "SupportLightLinkagePlan": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCfg | M | boolean | Whether enter area detection is supported. 0: No 1: Yes | 0 |
| Mode | 0 | unsigned long | Smart mode: 0: Ordinary 1: Deep Learning | 0 |
| LinkagePlanCfg | C | Json Block | Arming schedule of linkage actions. Required when supported. |  |
| SupportAudioLinkagePlan | M | boolean | Whether audio linkage schedule is supported. 0: No 1: Yes | 1 |
| SupportLightLinkagePlan | M | boolean | Whether light linkage schedule is supported. 0: No 1: Yes | 1 |

LeaveZoneCapInfo

| LeaveZoneCapInfo Json Block | { "SupportCfg":, "Mode": , "LinkagePlanCfg": { "SupportAudioLinkagePlan":, "SupportLightLinkagePlan": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCfg | M | boolean | Whether leave area detection is supported. 0: No 1: Yes | 0 |
| Mode | 0 | unsigned long | Smart mode: 0: Ordinary 1: Deep Learning | 0 |
| LinkagePlanCfg | C | Json Block | Arming schedule of linkage actions. Required when supported. |  |
| SupportAudioLinkagePlan | M | boolean | Whether audio linkage schedule is supported. 0: No 1: Yes | 1 |
| SupportLightLinkagePlan | M | boolean | Whether light linkage schedule is supported. 0: No 1: Yes | 1 |

CrossLineDetectionAreaInfoList

| CrossLineDetectionAreaInfoList Json Block | { "Num": , "LineInfoList": [<CrossLineDetectionLineInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Maximum number of areas that can be drawn. The default is 4 for cross line detection. | 4 |
| LineInfoList | C | array | Info list of detection area parameters. Optional when Num is 0. | -- |
| CrossLineDetectionLineInfo | M | Json Block | Detection area parameter info. See CrossLineDetectionLineInfo Json Block. | -- |

CrossLineDetectionLineInfo

| CrossLineDetectionLineInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "Direction": , "StartPoint": { "X": , "Y": }, "EndPoint": { "X": , "Y": } "Priority":, "Num":, "DetectTargetList":[<DetectTargetInfo>,<DetectTargetInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of detection area, starts from 0. Up to 4 areas are allowed. | 1 |
| Enabled | M | boolean | Enable or disable detection area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| Direction | M | unsigned long | Direction 0: A < - > B, bidirectional between A and B 1: B -> A, from B to A 2: A -> B, from A to B | 1 |
| StartPoint | M | Json Block | Coordinates of start point of detection line. |  |
| EndPoint | M | Json Block | Coordinates of end point of detection line. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |
| Priority | 0 | unsigned long | Priority level High: 0 Medium: 1 Low: 2 | 0 |
| Num | C | unsigned long | Number of detection objects. Required when object parameters are present. | 3 |
| DetectTargetList | C | array | Detection object list. Required when object parameters are present. |  |
| DetectTargetInfo | M | Json Block | Detection object info. See DetectTargetInfo Json Block. |  |

DetectTargetInfo

| DetectTargetInfo Json Block | { "Enabled":, "Type":, "ObjectMaxSize":<ObjectSize>, "ObjectMinSize":<ObjectSize> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Enable or disable detection 0: Disable 1: Enable | 0 |
| Type | M | unsigned long | Detection type Motor vehicle: 0 Non-motor vehicle: 1 Pedestrian: 2 | 0 |
| ObjectMaxSize | M | Json Block | Maximum object size. See ObjectSize Json Block. | -- |
| ObjectMinSize | M | Json Block | Minimum object size. See ObjectSize Json Block. | -- |

ObjectSize

| ObjectSize Json Block | { "Width":, "Height": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Object width. Range: [0,9999]. | 5000 |
| Height | M | unsigned long | Object height. Range: [0,9999]. | 5000 |

IntrusionDetectionAreaInfoList

| IntrusionDetectionAreaInfoList Json Block | { "Num": , "PolygonInfoList": [<IntrusionDetectionPolygonInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Maximum number of areas that can be drawn. The default is 4 for intrusion detection. | 4 |
| PolygonInfoList | C | array | Info list of detection area parameters. Optional when Num is 0. | -- |
| IntrusionDetectionPolygonInfo | M | Json Block | Detection area parameter info. See IntrusionDetectionPolygonInfo Json Block. | -- |

IntrusionDetectionPolygonInfo

| IntrusionDetectionPolygonInfoJson Block | { "ID": , "Enabled": , "Sensitivity": , "Percentage": , "TimeThreshold": , "PointNum": , "PointList": [{ "X": , "Y": },…] "Priority":, "Num":, "DetectTargetList":[<DetectTargetInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of detection area, starts from 0. Up to 4 areas are allowed. | 1 |
| Enabled | M | boolean | Enable or disable detection area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| Percentage | M | unsigned long | Percentage. Range: [1,100] | 1 |
| TimeThreshold | M | unsigned long | Time threshold. Range: [1, 10] | 1 |
| PointNum | M | unsigned long | Number of vertices of the polygon detection area. Range: [0,6] | 4 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |
| Priority | C | unsigned long | Priority level High: 0 Medium: 1 Low: 2 | 0 |
| Num | C | unsigned long | Number of detection object types. | 3 |
| DetectTargetList | C | array | Detection object list. Required when object parameters are present. |  |
| DetectTargetInfo | M | Json Block | Detection object info. See DetectTargetInfo Json Block. |  |

LeaveZoneRuleInfo

| LeaveZoneRuleInfooJson Blcok | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable leave area detection. 0: Disable 1: Enable | 1 |

LeaveZoneAreaInfoList

| LeaveZoneAreaInfoList Json Block | { "Num": , "PolygonInfoList": [<ZonePolygonInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Maximum number of areas that can be drawn. The default is 4 for leave area detection. | 4 |
| PolygonInfoList | C | array | Detection area parameter list. Optional when Num is 0. |  |
| ZonePolygonInfo | M | Json Block | Detection area parameter info. See ZonePolygonInfo Json Block. |  |

IntrusionDetectionRuleInfo

| IntrusionDetectionRuleInfo JsonBlcok | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable intrusion detection. 0: Disable 1: Enable | 1 |

ZonePolygonInfo

| ZonePolygonInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "PointNum": , "PointList": [ { "X": , "Y": }, … ], "Priority":, "Num":, "DetectTargetList":[<DetectTargetInfo>,<DetectTargetInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of detection area, starts from 1, and the maximum is the value of Nums in Areas. | 1 |
| Enabled | M | boolean | Enable or disable detection area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| PointNum | M | unsigned long | Number of vertices of the polygon detection area. | 4 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |
| Priority | 0 | unsigned long | Priority level High: 0 Medium: 1 Low: 2 | 0 |
| Num | M | unsigned long | Number of detection objects. | 3 |
| DetectTargetList | C | array | Detection object list. Required when object parameters are present. |  |
| DetectTargetInfo | M | Json Block | Detection object info. See DetectTargetInfo Json Block. |  |

AccessZoneRuleInfo

| AccessZoneRuleInfo Json Blcok | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable enter area detection. 0: Disable 1: Enable | 1 |

AccessZoneAreaInfoList

| AccessZoneAreaInfoList Json Block | { "Num": , "PolygonInfoList": [<ZonePolygonInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Maximum number of areas that can be drawn. The default is 4 for enter area detection. | 4 |
| PolygonInfoList | C | array | Detection area parameter list. Optional when Num is 0. |  |
| ZonePolygonInfo | M | Json Block | Detection area parameter info. See ZonePolygonInfo Json Block. |  |

HybridDetectionRuleInfo

| HybridDetectionRuleInfo Json Block | { "Enabled":, "ObjectCount":<ObjectCountInfo>, "Num":, "DetectTargetList":[{ "Enabled":, "Type":, "ObjectMaxSize": { "Width":, "Height": }, "ObjectMinSize":{ "Width":, "Height": } },…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable mixed traffic detection. 0: Disable 1: Enable | 1 |
| Num | M | unsigned long | Number of detection object types. | 1 |
| ObjectCount | M | Json Block | Object counting information. | See ObjectCountInfo Json Block |
| DetectTargetList | C | array | Detection object info. Optional when the Num is 0. | -- |
| Enabled | M | boolean | Enable or disable detection. 0: Disable 1: Enable | 0 |
| Type | M | unsigned long | Detection type. 0: Motor vehicle 1: Non-motor vehicle 2: Pedestrian |  |
| ObjectMaxSize | M | Json Block | Max object size. | -- |
| Width | M | unsigned long | Object width. Range: [0,9999]. | 5000 |
| Height | M | unsigned long | Object height. Range: [0,9999]. | 5000 |
| ObjectMinSize | M | Json Block | Minimum object size. | -- |
| Width | M | unsigned long | Object width. Range: [0,9999]. | 5000 |
| Height | M | unsigned long | Object height. Range: [0,9999]. | 5000 |

HybridDetectionAreaCfg

| HybridDetectionAreaCfg Json Block | { "IsEnableEntireImage":, "Sensitivity":, "PointNum":, "PointList": [{ "X": , "Y": },…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsEnableEntireImage | M | boolean | Enable or disable full screen detection (detect entire image). 0: Full screen detection is disabled (detection of specified areas) 1: Full screen detection is enabled | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| PointNum | M | unsigned long | Number of vertices of the detection area. | 4 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when the number of vertices is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,9999] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,9999] | 5000 |

AttributeCollectInfo

| AttributeCollectInfo Json Block | { "AttributeAll":, "FaceAttribute": { "IsEnableGender": , "IsEnableAge": , "IsEnableGlasses": , "IsEnableMaskFlag": , "IsEnableTemp": }, "PedestrianAttribute": { "IsEnableGender": , "IsEnableAge": , "IsEnableMaskFlag": , "IsEnableUpperColor": , "IsEnableLowerColor": , "IsEnableUpClothesTexture": , "IsEnableUpperClothingStyle": , "IsEnableLowerClothingStyle": , "IsEnableShoes": , "IsEnableBodyAngle": , "IsEnableBagInfo": , "IsEnableHairStyle": , "IsEnableDirection": }, "NonMotorVehicleAttribute": { "IsEnableGender": , "IsEnableAge": , "IsEnableUpperColor": , "IsEnableUpperClothingStyle": , "IsEnableNonVehicleType": , "IsEnableDirection": , "IsEnableSpeed": , "IsEnablePlateNumber": , "IsEnablePlateType": , "IsEnablePlateColor": }, "VehicleAttribute": { "IsEnableLogo": , "IsEnableVehicleType": , "IsEnableVehicleColor": , "IsEnablePlateNumber": , "IsEnablePlateType": , "IsEnablePlateColor": , "IsEnableDirection": , "IsEnableSpeed": , "IsEnableSunVisor": , "IsEnableSeatBelt": , "IsEnableDriverMobile": , "IsEnableAim": , "IsEnablePendant": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AttributeAll | 0 | boolean | Whether collect all attributes. 0: Not all attributes 1: All attributes | 0 |
| FaceAttribute | M | Json Block | Face attribute enablement/disablement info | -- |
| IsEnableGender | M | boolean | Enable or disable gender detection. 0: Disable 1: Enable | 0 |
| IsEnableAge | M | boolean | Enable or disable age detection. 0: Disable 1: Enable | 0 |
| IsEnableGlasses | M | boolean | Enable or disable glasses detection. 0: Disable 1: Enable | 0 |
| IsEnableMarkFlag | M | Boolean | Enable or disable mask detection. 0: Disable 1: Enable | 0 |
| IsEnableTemp | M | Boolean | Enable or disable temperature detection. 0: Disable 1: Enable | 0 |
| PedestrianAttribute | M | Json Block | Pedestrian attribute enablement/disablement info | -- |
| IsEnableGender | M | boolean | Enable or disable gender detection. 0: Disable 1: Enable | 0 |
| IsEnableAge | M | boolean | Enable or disable age detection. 0: Disable 1: Enable | 0 |
| IsEnableMarkFlag | M | boolean | Enable or disable mask detection. 0: Disable 1: Enable | 0 |
| IsEnableUpperColor | M | boolean | Enable or disable coat color detection. 0: Disable 1: Enable | 0 |
| IsEnableLowerColor | M | boolean | Enable or disable trousers color detection. 0: Disable 1: Enable | 0 |
| IsEnableUpClothesTexture | M | boolean | Enable or disable coat texture detection. 0: Disable 1: Enable | 0 |
| IsEnableUpperClothingStyle | M | boolean | Enable or disable coat style detection. 0: Disable 1: Enable | 0 |
| IsEnableLowerClothingStyle | M | boolean | Enable or disable trousers style detection. 0: Disable 1: Enable | 0 |
| IsEnableShoes | M | boolean | Enable or disable shoes detection. 0: Disable 1: Enable | 0 |
| IsEnableBodyAngle | M | boolean | Enable or disable body angle detection. 0: Disable 1: Enable | 0 |
| IsEnableBagInfo | M | boolean | Enable or disable bag detection. 0: Disable 1: Enable | 0 |
| IsEnableHairStyle | M | boolean | Enable or disable hairstyle detection. 0: Disable 1: Enable | 0 |
| IsEnableDirection | M | boolean | Enable or disable movement direction detection. 0: Disable 1: Enable | 0 |
| NonMotorVehicleAttributeInfo | M | Json Block | Non-motor vehicle attribute enablement/disablement info | -- |
| IsEnableGender | M | boolean | Enable or disable gender detection. 0: Disable 1: Enable | 0 |
| IsEnableAge | M | boolean | Enable or disable age detection. 0: Disable 1: Enable | 0 |
| IsEnableUpperColor | M | boolean | Enable or disable coat color detection. 0: Disable 1: Enable | 0 |
| IsEnableUpperClothingStyle | M | boolean | Enable or disable coat style detection. 0: Disable 1: Enable | 0 |
| IsEnableNonVehicleType | M | boolean | Enable or disable non-motor vehicle type detection. 0: Disable 1: Enable | 0 |
| IsEnableDirection | M | boolean | Enable or disable movement direction detection. 0: Disable 1: Enable | 0 |
| IsEnableSpeed | M | boolean | Enable or disable movement speed detection. 0: Disable 1: Enable | 0 |
| VehicleAttribute | M | Json Block | Motor vehicle attribute enablement/disablement info | -- |
| IsEnableVehicleType | M | boolean | Enable or disable vehicle type detection. 0: Disable 1: Enable | 0 |
| IsEnableVehicleColor | M | boolean | Enable or disable vehicle color detection. 0: Disable 1: Enable | 0 |
| IsEnableLogo | M | boolean | Enable or disable vehicle brand detection. 0: Disable 1: Enable | 0 |
| IsEnableDirection | M | boolean | Enable or disable driving direction detection. 0: Disable 1: Enable | 0 |
| IsEnableSpeed | M | boolean | Enable or disable driving speed detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateNumber | M | boolean | Enable or disable plate number detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateType | M | boolean | Enable or disable plate type detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateColor | M | boolean | Enable or disable plate color detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateNumber | M | boolean | Enable or disable plate number detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateType | M | boolean | Enable or disable plate type detection. 0: Disable 1: Enable | 0 |
| IsEnablePlateColor | M | boolean | Enable or disable plate color detection. 0: Disable 1: Enable | 0 |
| IsEnableSunVisor | M | boolean | Enable or disable sun visor detection. 0: Disable 1: Enable | 0 |
| IsEnablePendant | M | boolean | Enable or disable pendant detection. 0: Disable 1: Enable | 0 |
| IsEnableAim | M | boolean | Enable or disable inspection sticker detection. 0: Disable 1: Enable | 0 |
| IsEnableSeatBelt | M | boolean | Enable or disable seat belt detection. 0: Disable 1: Enable | 0 |
| IsEnableDriverMobile | M | boolean | Enable or disable driver’s phone usage detection. 0: Disable 1: Enable | 0 |

AttributeParamInfo

| AttributeParamInfo Json Block | { “PupilDistance”:, “PlateParam”:{ “PlateProvince”:, “PlateLetter”: } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PupilDistance | M | unsigned long | Pupillary distance Range: 40-250. Default: 60. Unit: px | 60 |
| PlateParam | M | Json Block | Plate parameter | -- |
| PlateProvince | M | unsigned long | Default province of plate 0: Undefined 1~43 | 1 |
| PlateLetter | M | unsigned long | Default letter of plate 'A'-'H' > 0 -7 'J'-'N' > 8 - 12 'P'-'Z' > 13 - 23 'O'> 24 | 1 |

AudioActParamInfo

| AudioActParamInfo Json Block | { "AudioFileID": , “WarnCount”： } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AudioFileID | M | unsigned long | Audio file ID. Used to identify different audio files. | 1 |
| WarnCount | M | unsigned long | How many times to play audio files repeatedly. Range: [1,9] | 2 |

AudioFileInfo

| AudioFileInfo Json Block | { "ID": , "Status": , "InstallType":, "FileName": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Audio file ID. Used to identify different audio files. | 1 |
| Status | M | unsigned long | Audio file status. 0: Not in use 1: In use | 0 |
| InstallType | M | unsigned long | Installation type: 0: Built-in 1: External Built-in: Internal alarm audio file of the device. External: Audio file imported by user. | 1 |
| FileName | M | string | Audio file name Max 256 bytes. Required when getting info about alarm audio files | “AudioFile.pcm” |

CalibrationLineInfo

| CalibrationLineInfo Json Block | { “Left”: { “X”: , “Y”: }, “Right”: { “X”: , “Y”: }, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Left | M | Json Block | Coordinates of the left point of the calibration line | -- |
| Right | M | Json Block | Coordinates of the right point of the calibration line | -- |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |

CalibrationPointsInfo

| CalibrationPoints Json Block | { "DomeID":, "SceneNum":, "SceneCalibrationPointList":[<SceneCalibrationPointInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DomeID | M | unsigned long | Dome camera ID. | 1 |
| SceneNum | M | unsigned long | Number of scenes. | 1 |
| SceneCalibrationPointInfo | M | array | Info about all scene calibration points. | - |

CalibrationPointInfo

| CalibrationPointInfo Json Block | { "BoxPoint":<BoxPointInfo>, "DomePoint":<DomePointInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| BoxPoint | M | Json Block | Calibration point of box camera. | - |
| DomePoint | M | Json Block | Calibration point of dome camera. | - |

CaptureInfo

| CaptureInfo Json Block | { "CaptureNums":, "CaptureType":, "Interval": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| CaptureNums | M | unsigned long | Number of snapshots. Range: [3,6] | 4 |
| CaptureType | M | array | Type of captured image: 1. Panoramic 2: Close-up 4: Features | [1, 2, 1, 2] |
| Interval | M | array | Time interval of image capture (unit: second) | [120, 120, 120] |

ChannelPreset

| ChannelPreset Json Block | { "ChannelID":, "PresetID" } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ChannelID | C | unsigned long | Video channel ID Optional for IPC. | 1 |
| PresetID | M | unsigned long | Preset ID | 1 |

CompareInfo

| CompareInfo Json Block | { "Similarity":, "MemberInfo": <MemberInfo>, “FaceAttr”:<FaceAttr>, “PersonAttr”:<PersonAttr>, "SnapshotImage": <SnapshotImage > } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Similarity | C | unsigned long | Matching degree. Required for match alarms. | 80 |
| MemberInfo | C | JsonBlock | Person info. See MemberInfo JsonBlock. Required for match alarms. | -- |
| FaceAttr | C | Json Block | Face attributes. See FaceAttr Json Block. Applicable to VMS when querying structured data. | -- |
| PersonAttr | C | Json Block | Person attributes. See PersonAttr Json Block. Applicable to VMS when querying structured data. | -- |
| SnapshotImage | M | JsonBlock | Snapshot image. See SnapshotImage JsonBlock. | -- |

CompassInfo

| CompassInfo Json Block | { "Angle": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Angle | M | float | Angle between the electronic compass and true north (unit: degree). Two decimal places. | 50.12 |

CrossLineDetectionCapInfo

| CrossLineDetectionCapInfo Json Block | { "SupportCfg": , "Mode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SupportCfg | M | boolean | Whether cross line detection is supported: 0: No 1: Yes | 0 |
| Mode | C | unsigned long | Smart mode: 0: Ordinary 1: Deep Learning | 0 |

CrossLineDetectionInfo

| CrossLineDetectionInfo Json Block | { "Rule":<RuleInfo>, "Areas": { "Nums": , "LineInfos": [<LineInfo>,…] }, "LinkageActions":<LinkageActionInfoList>, "WeekPlan":<WeekPlanInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Rule | M | Json Block | Cross line detection rules. See RuleInfoJsonBlcok. | -- |
| RuleInfo | M | Json Block | Detailed cross line detection rules. | -- |
| Areas | M | Json Block | Info about cross line detection areas. See AreaInfo Json Block. | -- |
| LinkageActions | M | Json Block | Linkage actions of cross line detection. See LinkageActionInfoList Json Block. | -- |
| WeekPlan | M | Json Block | Arming scheduling of cross line detection. See WeekPlanInfo Json Block. | -- |

CrossLineDetectionRuleInfo

| CrossLineDetectionRuleInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable cross line detection. 0: Disable 1: Enable | 1 |

CrossLineDetectionLineInfo

| CrossLineDetectionLineInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "Direction": , "StartPoint": { "X": , "Y": }, "EndPoint": { "X": , "Y": } "Priority":, "Num":, "DetectTargetList":[<DetectTargetInfo>,<DetectTargetInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of detection area, starts from 0. Up to 4 areas are allowed. | 1 |
| Enabled | M | boolean | Enable or disable detection area. 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| Direction | M | unsigned long | Direction 0: A < - > B, bidirectional between A and B 1: B -> A, from B to A 2: A -> B, from A to B | 1 |
| StartPoint | M | Json Block | Coordinates of start point of detection line. |  |
| EndPoint | M | Json Block | Coordinates of end point of detection line. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |
| Priority | C | unsigned long | Priority level High: 0 Medium: 1 Low: 2 | 0 |
| Num | C | unsigned long | Number of detection objects. Required when object parameters are present. | 3 |
| DetectTargetList | C | array | Detection object list. Required when object parameters are present. |  |
| DetectTargetInfo | M | Json Block | Detection object info. See DetectTargetInfo Json Block. |  |

CrossLineDetectionAreaInfoList

| CrossLineDetectionAreaInfoList Json Block | { "Num": , "LineInfoList": [<CrossLineDetectionLineInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Maximum number of areas that can be drawn. The default is 4 for cross line detection. | 4 |
| LineInfoList | C | array | Info list of detection area parameters. Optional when Num is 0. | -- |
| CrossLineDetectionLineInfo | M | Json Block | Detection area parameter info. See CrossLineDetectionLineInfo Json Block. | -- |

CrossLineDetectionInfoList

| CrossLineDetectionInfolist Json Block | { “Total”:, "Num": , "CrossLineDetectionInfoList":[<PicDataInfo>, …] } |
| --- | --- |

| Param | Ranger | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Total | M | unsigned long | Total number of cross line detections. | 1000 |
| Num | M | unsigned long | Number of cross line detections currently returned. | 2 |
| CrossLineDetectionInfoList | C | array | Image results of cross line detection. Optional when Num is 0. See PicDataInfoJson Block. | - |

CtrlFaceInfo

| CtrlFaceInfo Json Block | { "ID":, "Timestamp":, "CapSrc":, "FeatureNum": , "FeatureList":[{ "FeatureVersion":, "Feature": },...], "PanoImage":<FileInfo>, "FaceImage":<FileInfo>, "FaceArea": { "LeftTopX":, "LeftTopY":, "RightBottomX":, "RightBottomY": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Record ID | 0 |
| Timestamp | O | unsigned long | Collection time (unit: second, UTC format) Optional when absent. |  |
| CapSrc | M | unsigned long | Source of collection 1. Face info collected by face recognition terminal. 2. Access card info collected by card reader. 3. ID card info collected by card reader. 4. Pass-through info collected by gate For FaceInfo, choose 1. |  |
| FeatureNum | 0 | unsigned long | Number of semi-structured features. Range: [0, 2]. Optional when there is no semi-structured feature. |  |
| FeatureList | 0 | array | List of semi-structured features. Optional when there is no semi-structured feature. |  |
| FeatureVersion | 0 | string | Version number of the algorithm that extracts semi-structured face features, for example, "ISFRFR259.2.0”. Range: [0, 20]. |  |
| Feature | 0 | string | Base64 encoding. Extracted face features. Used to assist the backend server to perform face comparison. Currently 512Bytes before encryption. |  |
| PanoImage | 0 | Json Block | Panoramic image (original image). Optional. Note: For PTS, the file size range is [0,1M]. |  |
| FaceImage | 0 | Json Block | Close-up image of face. Optional. Note: For PTS, the file size range is [0,256K]. |  |
| FaceArea | M | Json Block | Coordinates of face area in the panoramic image Position of the face in the panoramic image. Coordinates: (0-10000). The top left and bottom right points of the rectangle: “138,315,282,684” |  |
| LeftTopX | M | unsigned long | x-coordinate of upper-left corner |  |
| LeftTopY | M | unsigned long | y-coordinate of upper-left corner |  |
| RightBottomX | M | unsigned long | x-coordinate of lower-right corner |  |
| RightBottomY | M | unsigned long | y-coordinate of lower-right corner |  |

CtrlCardInfo

| CtrlCardInfo Json Block | { "ID":, "Timestamp":, "CapSrc":, "CardType":, "CardID": , "CardStatus": , "Name":, "Gender":, "Ethnicity":, "Birthday":, "ResidentialAddress":, "IdentityNo":, "IssuingAuthority":, "IssuingDate":, "ValidDateStart":, "ValidDateEnd":, "IDImage":<FileInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Record ID |  |
| Timestamp | O | unsigned long | Collection time (unit: second, UTF format) Optional when absent. |  |
| CapSrc | M | unsigned long | Source of collection 1. Face info collected by face recognition terminal. 2. Access card info collected by card reader. 3. ID card info collected by card reader. 4. Pass-through info collected by gate For CardInfo choose 2 or 3. |  |
| CardType | M | unsigned long | 0: ID card 1: Access card |  |
| CardID | C | string | Field of access card: Physical card number. Max. 18 digits. |  |
| CardStatus | C | unsigned long | Field of access card. Card status: 1: Valid 0: Invalid |  |
| Name | C | string | ID card field: Name. Range: [1,63]. |  |
| Gender | C | unsigned long | Field of ID card. 0: Unknown gender 1: Male 2: Female 9: Unspecified gender |  |
| Ethnicity | C | unsigned long | Field of ID card. Refer to GB/T 3304 for the names of nationalities of China in romanization with codes. |  |
| Birthday | C | string | Field of ID card: Date of birth. Format: YYYYMMDD. |  |
| ResidentialAddress | C | string | Field of ID card: Address. |  |
| IdentityNo | C | string | Field of ID card: ID card number. Max. 18 bits. |  |
| IssuingAuthority | C | string | Field of ID card: Issuing authority, generally in format of XX Province XX City XX District (County) Public Security Bureau. |  |
| IssuingDate | C | string | Field of ID card. Date of issuance. Format: YYYYMMDD. |  |
| ValidDateStart | C | string | Field of ID card. Start time of validity period of ID card. Format: YYYYMMDD. |  |
| ValidDateEnd | C | string | Field of ID card. End time of validity period of ID card. Format: YYYYMMDD. |  |
| IDImage | C | Json Block | Field of ID card: ID card photo. Note: For PTS, the file size range is [0,32K]. |  |

CtrlGateInfo

| CtrlGateInfo Json Block | { "ID":, "Timestamp":, "CapSrc":, "InPersonCnt":, "OutPersonCnt": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Record ID |  |
| Timestamp | M | unsigned long | Collection time (unit: second, UTC format) Optional when absent. |  |
| CapSrc | M | unsigned long | Source of collection 1. Face info collected by face recognition terminal. 2. Access card info collected by card reader. 3. ID card info collected by card reader. 4. Pass-through info collected by gate For GateInfo, choose 4. |  |
| InPersonCnt | M | unsigned long | Number of people that have entered |  |
| OutPersonCnt | M | unsigned long | Number of people that have left |  |

CtrlLibMatchInfo

| CtrlLibMatchInfo Json Block | { "ID":, "LibID":, "LibType":, "MatchStatus":, "MatchPersonID":, "MatchFaceID":, "MatchPersonInfo":{ “PersonCode”:, "PersonName":, "Gender":, "CardID":, "IdentityNo": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Record ID |  |
| LibID | M | unsigned long | Library ID |  |
| LibType | O | unsigned long | Library type. Optional. |  |
| MatchStatus | M | unsigned long | Matching status. 1: Authentication succeeded. 2: Authentication failed (comparison failed). 3: Authentication failed (comparison succeeded, not within monitoring period). 41: Library image collected successfully 42: Failed to collect library image |  |
| MatchPersonID | M | unsigned long | Matching person ID |  |
| MatchFaceID | M | unsigned long | Matching face ID |  |
| MatchPersonInfo | 0 | Json Block | Matching person info. Optional. |  |
| PersonCode | 0 | string | Person ID. Can be student ID or employee ID. Range: [1, 15] | "3060911012" |
| PersonName | 0 | string | Member name. Range: [1,63]. |  |
| Gender | 0 | unsigned long | Gender. 0: Unknown 1: Male 2: Female 9: Unspecified |  |
| CardID | 0 | string | Access card number. Empty if not available. |  |
| IdentityNo | 0 | string | ID card number. Empty if not available. |  |

CountResetInfo

| CountResetInfo Json Block | { "ResetEnabled":, "TimingResetEnabled":, "ResetTime": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ResetEnabled | M | boolean | Whether to reset counter immediately. 0: No. 1: Yes | 0 |
| TimingResetEnabled | M | boolean | Whether timed reset is enabled. 0: Disable 1: Enable | 0 |
| ResetTime | C | string | Scheduled reset time. Required when timed reset is enabled. Reset counter at a set time of the day (24-hour format). Format: xx:xx:xx. | “23:30:59” |

CustomInfo

| CustomInfo Json Block | { "ID":, "Value": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | ID of customer attribute name. Starts from 0. | 1 |
| Value | M | string | Custom attribute value. Range: [1,63]. | "Custom attribute value" |

CustomAttrInfo

| CustomAttrInfo Json Block | { "ID":, "Enabled":, "Name": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | ID of customer attribute name. Starts from 1. | 1 |
| Enabled | M | boolean | Enable or disable custom attribute 0: Disable 1: Enable | 1 |
| Name | M | string | Custom attribute name. Range: [1,63]. | "Custom attribute title" |

CustomAttrList

| CustomAttrList Json Block | { "Num":, "CustomAttrList":[<CustomAttrInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of custom attributes. | 5 |
| CustomAttrList | C | Array | Custom attribute list. This field will not be returned when Num is 0. | - |
| CustomAttrInfo | C | Json Block | Custom attribute info. See CustomAttrInfo JsonBlock. | - |

DatabaseInfo

| DatabaseInfo Json Block | { "ID":, "Name":, "IsMonitored":, “MemberNum”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Library ID. Required when using the GET method. | 3 |
| Name | M | string | Library name. Length range: [1,63] | “List” |
| IsMonitored | C | boolean | Whether monitoring is enabled. Required when using the GET method. 0: No 1: Yes | 0 |
| MemberNum | C | unsigned long | Total number of members in the face library. Required when using the GET method. | 100 |

DatabaseList

| DatabaseList Json Block | { "DatabaseNum":, "DatabaseList":[ <DatabaseInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DatabaseNum | M | unsigned long | Number of libraries. | 3 |
| DatabaseList | C | array | Library list. Optional when DatabaseNum is 0. | - |
| DatabaseInfo | M | Json Block | Library info. See DatabaseInfo Json Block. | - |

DatabaseImportInfo

| DatabaseImportInfo Json Block | { "ID":, "ImportedNum": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Face library ID. | 1 |
| ImportedNum | M | unsigned long | Number of imported members | 1000 |

DayPlanInfo

| DayPlanInfo Json Block | { "ID":, "Num":, "TimeSectionInfos": [<TimeSectionInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of days 1: Monday 2: Tuesday 3: Wednesday 4: Thursday 5: Friday 6: Saturday 7: Sunday; 8: Holiday; | 1 |
| Num | M | unsigned long | Number of time periods per day NVR: Max 8 IPC: Max 4 | 8 |
| TimeSectionInfos | M | array | Arming schedule info. Time periods on the same day must not overlap. |  |
| TimeSectionInfo | M | Json Block | Detailed arming schedule. See TimeSectionInfo. |  |

DetectionSceneInfo

| DetectionSceneInfo Json Block | { "SceneType": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SceneType | M | unsigned long | Scene of smart detection. 0: Indoor scene 1: Outdoor scene | 0 |

DetectTargetInfo

| DetectTargetInfo Json Block | { "Enabled":, "Type":, "ObjectMaxSize":<ObjectSize>, "ObjectMinSize":<ObjectSize> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Enable or disable detection 0: Disable 1: Enable | 0 |
| Type | M | unsigned long | Detection type Motor vehicle: 0 Non-motor vehicle: 1 Pedestrian: 2 | 0 |
| ObjectMaxSize | M | Json Block | Maximum object size. See ObjectSize Json Block. | -- |
| ObjectMinSize | M | Json Block | Minimum object size. See ObjectSize Json Block. | -- |

FaceAngleFilterInfo

| FaceAngleFilterInfo Json Block | { "Enabled":, "RollAngleLeft":, "RollAngleRight":, "YawAngleLeft":, "YawAngleRight":, "PitchAngleUp":, "PitchAngleDown": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Whether face angle filtering is enabled. 0: Disabled (default) 1: Enabled | 0 |
| RollAngleLeft | M | unsigned short | Left roll angle, rotated left around Z-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle a left-facing face can be detected. | 40 |
| RollAngleRight | M | unsigned short | Right roll angle, rotated right around Z-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle a right-facing face can be detected. | 40 |
| YawAngleLeft | M | unsigned short | Left yaw angle, rotated left around Y-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle a left-facing face can be detected. | 40 |
| YawAngleRight | M | unsigned short | Right yaw angle, rotated right around Y-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle a right-facing face can be detected. | 40 |
| PitchAngleUp | M | unsigned short | Positive pitch angle, rotated upward around X-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle an upward-facing face can be detected. | 40 |
| PitchAngleDown | M | unsigned short | Negative pitch angle, rotated downward around X-axis of the right-handed coordinate system. Unit: Degree Range: [0,90] 0°: The face is horizontal and front-facing 90°: The maximum angle an downward-facing face can be detected. | 40 |

FaceAttr

| Faceattr Json Block | { "Gender":, "AgeRange":, "EthicCode":, "GlassFlag":, "GlassesStyle":, "GlassesColor":, "HairStyle":, "HairColor":, "FaceStyle":, "SkinColor":, "EyebrowStyle":, "WrinklePouch":, "NoseStyle":, "Beard ":, "LipStyle":, "MustacheStyle":, "MaskFlag":, "MaskColor":, "HatFlag":, "HatStyle":, "HatColor":, "Scarf":, "ScarfColor":, "CoatColor":, "AcneStain":, "FreckleBirthmark":, "ScarDimple":, "FacialFeature":, "Temperature": } |
| --- | --- |

| Param | Requirement | Type | Description |
| --- | --- | --- | --- |
| Gender | C | unsigned long | Gender. Optional when detection is disabled. 0: Unknown 1: Male 2: Female |
| AgeRange | C | unsigned long | Age range. Optional when detection is disabled. 0: Unknown 1: Child 2: Juvenile 3: Youth 4: Middle-aged 5: Senior |
| GlassFlag | C | unsigned long | Whether glasses are detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| GlassesStyle | C | unsigned long | Glasses style. Optional when GlassFlag is 0. 0: Unknown 1: Common glasses 2: Sunglasses 99: Other |
| HairStyle | C | unsigned long | Hairstyle. Optional when detection is disabled. See definitions of person info elements. |
| HairColor | C | unsigned long | Hair color. Optional when detection is disabled. See definitions of person info elements. |
| FaceStyle | C | unsigned long | Face style. Optional when detection is disabled. See definitions of person info elements. |
| SkinColor | C | unsigned long | Skin color. Optional when detection is disabled. See definitions of person info elements. |
| EyebrowStyle | C | unsigned long | Eyebrow style. Optional when detection is disabled. See definitions of person info elements. |
| WrinklePouch | C | unsigned long | Wrinkle and pouch. Optional when detection is disabled. See definitions of person info elements. |
| NoseStyle | C | unsigned long | Nose style. Optional when detection is disabled. See definitions of person info elements. |
| Beard | C | unsigned long | Beard. Optional when detection is disabled. See definitions of person info elements. |
| LipStyle | C | unsigned long | Lip style. Optional when detection is disabled. See definitions of person info elements. |
| MustacheStyle | C | unsigned long | Mustache style. Optional when detection is disabled. See definitions of person info elements. |
| MaskFlag | C | unsigned long | Whether mask is detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| MaskColor | C | unsigned long | Mask color. Optional when detection is disabled. See definitions of person info elements. |
| HatFlag | C | unsigned long | Whether hat is detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| HatStyle | C | unsigned long | Hat style. Optional when detection is disabled. See definitions of person info elements. |
| HatColor | C | unsigned long | Hat color. Optional when detection is disabled. See definitions of person info elements. |
| Scarf | C | unsigned long | Whether scarf is detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| ScarfColor | C | unsigned long | Scarf color. Optional when detection is disabled. See definitions of person info elements. |
| CoatColor | C | unsigned long | Coat color. Optional when detection is disabled. See definitions of person info elements. |
| AcneStain | C | unsigned long | Acne and stain. Optional when detection is disabled. See definitions of person info elements. |
| FreckleBirthmark | C | unsigned long | Freckle and birthmark. Optional when detection is disabled. See definitions of person info elements. |
| ScarDimple | C | unsigned long | Scar and dimple. Optional when detection is disabled. See definitions of person info elements. |
| FacialFeature | C | unsigned long | Facial features. Optional when detection is disabled. See definitions of person info elements. |
| Temperature | C | float | Body temperature. Optional when detection is disabled. Unit: Celsius degree Precision: 2 decimal places |

FaceAttributeAnalysisInfo

| FaceAttributeAnalysisInfo Json Block | { "Enabled":, "SexEnabled":, "AgeEnabled":, "GlassEnabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable face attribute analysis. 0: Disable 1: Enable When enabled, face attributes can be pushed. | 0 |
| SexEnabled | C | boolean | Enable or disable gender detection. 0: Disable 1: Enable Applicable to IPC. | 1 |
| AgeEnabled | C | boolean | Enable or disable age detection. 0: Disable 1: Enable Applicable to IPC. | 1 |
| GlassEnabled | C | boolean | Enable or disable glasses detection. 0: Disable 1: Enable Applicable to IPC. | 1 |

FaceCalibrationSizeInfo

| FaceCalibrationSizeInfo Json Block | { "WidthMin":, "HeightMin":, "WidthMid":, "HeightMid":, "WidthMax":, "HeightMax": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| WidthMin | M | unsigned long | Minimum face width that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 625 |
| HeightMin | C | unsigned long | Minimum face height that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 1111 |
| WidthMid | C | unsigned long | Moderate face width that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 1093 |
| HeightMid | C | unsigned long | Moderate face height that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 1944 |
| WidthMax | M | unsigned long | Maximum face width that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 1562 |
| HeightMax | C | unsigned long | Maximum face height that can be detected. Range: [104,2604]. Unit: ‱. The displayed image is converted to the unit of pixels based on the actual image size. | 2777 |

FaceCountInfo

| FaceCountInfo Json Block | { "Enabled":, "Direction": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable people/face counting 0: Disable 1: Enable | 0 |
| Direction | C | unsigned long | People/face counting direction: 0: In 1: Out Applicable to IPC. | 1 |

FaceDetectionCalPolygonInfo

| FaceDetectionCalPolygonInfo Json Block | { "ID":, "PointNum":, "Points": [{ "X":, "Y": },…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of calibrated area, starts from 0, and the maximum is the value of Nums in Calibrations. | 1 |
| PointNum | M | unsigned long | Number of vertices of the detection area. The value must be 4 because currently the detection area can only be a rectangle. | 4 |
| Points | C | array | Coordinates of vertices of the detection area. For coordinates of the upper-left corner (X1,Y1) and the lower-right corner (X2,Y2), X1<X2, Y1<Y2. | - |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |

FaceDetectionCapInfo

| FaceDetectionCapInfo Json Block | { "IsSupport":, "QualityAnalysisIsSupport":, "FaceQualityAnalysisType":, "AttributeAnalysisIsSupport":, "AttributeAnalysisSkills":, "FeatureIsSupport":, "RecognitionIsSupport":, "FaceRecognitionType":, "FaceLivingAnalysisIsSupport":, "PupilDistanceSizeNum":, "PupilDistanceSizeList":[<PupilDistanceCapInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupport | M | boolean | Whether smart face detection is supported: 0: No 1: Yes | 0 |
| QualityAnalysisIsSupport | C | boolean | Whether the device supports face selection (pushing images). 0: No 1: Yes | 1 |
| FaceQualityAnalysisType | C | unsigned long | Face selection type. 0: Selection by the front end 1: Selection by the center | 0 |
| AttributeAnalysisIsSupport | C | boolean | Whether the device supports face attribute analysis (push). 0: No 1: Yes | 1 |
| AttributeAnalysisSkills | C | array | Configured face attribute analysis capability. Optional when FaceAttributeAnalysisSupported is 0. 0: Age indication bit 1: Gender indication bit 2: Reserved 3: Glasses indication bit 4: Mask indication bit 5: Beard indication bit 6: Hat indication bit 7: Hairstyle indication bit 8: Expression indication bit 9: Facial attractiveness indication bit | [0, 1, 4] |
| FeatureIsSupport | C | boolean | Whether the device supports binary feature extraction (push). 0: No 1: Yes | 1 |
| RecognitionIsSupport | C | boolean | Whether the device supports face recognition. 0: No 1: Yes | 1 |
| FaceRecognitionType | C | unsigned long | Face recognition type. 0: Face recognition by the front end. 1: Face recognition by the center. | 1 |
| FaceLivingAnalysisIsSupport | C | boolean | Whether the device supports liveness detection. 0: No 1: Yes | 1 |
| PupilDistanceSizeNum | C | unsigned long | Number of pupillary distance ranges supported. | 1 |
| PupilDistanceSizeList | C | Json Block | Pupillary distance range list. See PupilDistanceCapInfo JsonBlock. |  |

FaceDetectionAreaList

| FaceDetectionAreaList Json Block | { "EntireImageEnabled": , "Num": , "Polygons": [<FaceDetectionPolygonInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| EntireImageEnabled | C | boolean | Enable or disable full screen detection (detect entire image). 0: Full screen detection is disabled (detection of specified areas) 1: Full screen detection is enabled Optional when returned results include capabilities other than smart face detection (IsSupport). | 1 |
| Num | M | unsigned long | Maximum number of detection areas. The default is 1. | 1 |
| Polygons | C | array | Detection area parameter list. Optional when Num is 0. | -- |
| FaceDetectionPolygonInfo | M | Json Block | Detection area parameter info. See FaceDetectionPolygonInfo Json Block. | -- |

FaceDetectionPolygonInfo

| FaceDetectionPolygonInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "FaceCalibrationSize": <FaceCalibrationSizeInfo>, "PupilDistanceMin":, "PointNum": , "Points": [{ "X": , "Y": },…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Index of detection area, starts from 0, and the maximum is the value of Nums in Detections. Optional when the detection area is the entire image (by setting EntireImageEnabled to 1 in FaceDetectionAreaList). ID is optional. | 1 |
| Enabled | C | boolean | Enable or disable detection area. 0: Disable 1: Enable Applicable to NVR. | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| FaceCalibrationSize | C | Json Block | Configuration item of detectable face size. See FaceCalibrationSizeInfo Json Block. Optional when returned results include smart face detection (IsSupport) only. This field is only for compatibility use. For new products use the PupilDistanceMin field. |  |
| PupilDistanceMin | C | unsigned long | Minimum pupillary distance, expressed using ‱. The range is determined by PupilDistanceSize in face capability. Required when face capability is supported. | 208 |
| PointNum | M | unsigned long | Number of vertices of the detection area. | 4 |
| Points | C | array | Coordinates of vertices of the detection area. Optional when the number of vertices is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |

FaceDetectionRuleInfo

| FaceDetectionRuleInfo Json Block | { "FaceFullcheck":<FaceFullcheckInfo>, "FaceCount":<FaceCountInfo>, "FaceSnapshot":<FaceSnapshotInfo>, "FaceQualityAnalysisEx":<FaceQualityAnalysisInfoEx>, "FaceLivingAnalysis":<FaceLivingAnalysisInfo>, "FaceAttributeAnalysis":<FaceAttributeAnalysisInfo>, "FaceRecognition":<FaceRecognitionInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FaceFullcheck | C | Json Block | Configuration item of full face detection. See FaceFullcheckInfo. Optional when returned results include capabilities other than smart face detection (IsSupport). | -- |
| FaceCount | C | Json Block | Configuration item of face counting. See FaceCountInfo. Currently not available. | -- |
| FaceSnapshot | C | Json Block | Configuration item of face snapshot. See FaceSnapshotInfo Optional when returned results include smart face detection (IsSupport) only. | -- |
| FaceQualityAnalysisEx | C | Json Block | Extended field of configuration item of face selection. See FaceQualityAnalysisInfoEx. Optional when returned results include smart face detection (IsSupport) only, or when search results include the face selection (QulityAnalysisIsSupported) field but the capability is not supported. | -- |
| FaceLivingAnalysis | C | Json Block | Configuration item of liveness detection. See FaceAttributeAnalysisInfo. Applicable to PTS. | -- |
| FaceAttributeAnalysis | C | Json Block | Configuration item of attribute analysis. See FaceAttributeAnalysisInfo. Optional when returned results include smart face detection (IsSupport) only, or when returned results include the face attribute analysis (FaceAttributeAnalysisIsSupported) field but the capability is not supported. | -- |
| FaceRecognition | C | Json Block | Configuration item of face recognition. See FaceRecognitionInfo Optional when returned results include smart face detection (IsSupport) only, or when returned results include the face recognition (FaceRecognitionIsSupported) field but the capability is not supported. Not applicable to IPC. | -- |

FaceEnableInfo

| FaceEnableInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable face related smart function (face+body). 0: Disable 1: Enable This parameter is the general switch used to enable/disable various face related smart functions. | 1 |

FaceFullcheckInfo

| FaceFullcheckInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable full face detection. 0: Disable 1: Enable | 0 |

FaceLivingAnalysisInfo

| FaceLivingAnalysisInfo Json Block | { "Enabled":, "Threshold": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Enable or disable liveness detection 0: Disable 1: Enable | 0 |
| Threshold | M | unsigned long | Liveness detection threshold. Range: [0,100] | 100 |

FacePassRecordInfo

| FacePassRecordInfo Json Block | { "RecordID": , "PassingTime": , "ChannelName": "CompareInfo": <ComapreInfo>, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| CompareInfo | M | Json Block | Comparison info. See CompareInfo json block. | - |

FaceQualityAnalysisInfoEx

| FaceQualityAnalysisInfoEx Json Block | { "Enabled":, "Mode":, "PreferenceLastTime":, “QualityNum”:, “IntervalTime”:, "QualityThreshold":, "ImageQualityThreshold":, "FaceAngleFilter":<FaceAngleFilterInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable face quality analysis: 0: Disable 1: Enable Face quality analysis is optional for face detection and is required for face recognition. | 0 |
| Mode | M | unsigned long | Face quality analysis mode. 0: Effect priority 1: Speed priority 2: Periodic selection | 0 |
| PreferenceLastTime | C | unsigned long | Duration of quick mode for face selection (seconds). Range: [1,1800]. Valid when Mode is Speed Priority. | 3 |
| QualityNum | C | unsigned long | Number of selected face pictures: [1-3] Valid when Mode is Effect Priority or Speed Priority. | 1 |
| IntervalTime | C | unsigned long | Uploading interval (millisecond) Range: [100-1800000] Valid when Mode is Periodic Selection. Applicable to IPC. | 1000 |
| QualityThreshold | M | unsigned long | Face quality threshold value. Range: [0, 100]. Applicable to PTS. | 64 |
| ImageQualityThreshold | C | unsigned long | Photo quality threshold value. Range: [0, 100]. Applicable to PTS. | 80 |
| FaceAngleFilter | C | Json Block | Face filtering angle. Valid on new face platform. See FaceAngleFilterInfo. Applicable to IPC. |  |

FaceRecognitionAlarmLog

| FaceRecognitionAlarmLog Json Block | { "ID": , "Timestamp": , "Type": , “ChannelID”:, "AlarmSrc": , "CompareInfo": <ComapreInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Face recognition alarm record ID. | 1 |
| Timestamp | M | unsigned long | Time (UTC format) |  |
| Type | M | unsigned long | Face library type: 0: Blocklist 1: Allowlist | 0 |
| ChannelID | C | unsigned long | Channel ID. Required when using the POST method. | 1 |
| AlarmSrc | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | "Camera" |
| CompareInfo | M | Json Block | Comparison info. | -- |

FaceRecognitionInfo

| FaceRecognitionInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable face recognition 0: Disable 1: Enable | 0 |

FaceSnapShotInfo

| FaceSnapShotInfo Json Block | { "Mode":, "Num":, “MarkFace”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Face snapshot mode (triggered by smart analysis or alarm) 1: Triggered by smart analysis 2: Triggered by an input switch signal 3: Triggered by smart analysis and an input switch signal | 0 |
| Num | M | unsigned long | Number of captured face snapshots. Range: [1,30]. | 2 |
| MarkFace | M | boolean | Face bounding box. For the original snapshot image, the face will have a bounding box in the image. 0: Turn off face bounding box. 1: Enable face bounding box. | 1 |

FeatureInfo

| FeatureInfo Json Block | { "Age":, "Gender":, "WithGlasses": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Age | M | unsigned long | Age. |  |
| Gender | M | unsigned long | Gender. |  |
| WithGlasses | M | boolean | Whether glasses is detected. | 1 |

FaceSnapshotInfo

| FaceSnapInfo Json Block | { "RecordID":, "ChannelID":, "EventType":, "PassingTime":, "ChannelName":, "CompareInfo": <ComapreInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| ChannelID | M | unsigned long | Channel ID. | 1 |
| EventType | M | unsigned long | Event type. Described by bit order. BIT=1 means the event is this type. BIT=0 means the event is not this type. BIT0: Face snapshot BIT1: Face match alarm BIT2: Face not match alarm | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| CompareInfo | M | Json Block | Comparison info. See CompareInfo json block. | - |

FileInfo

| FileInfo Json Block | { "Name":, "Size":, "Data":, "URL": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Name | 0 | string | File name. Range: [1, 16] Used to push images in real-time or historical records |  |
| Size | 0 | unsigned long | File size (unit: byte). Range: [0, 1M] Used to push images in real-time or historical records | 21783 |
| Data | 0 | string | Base64 data. Used to push images in real-time or historical records |  |
| URL | 0 | string | Image URL. Used to push image info when getting historical records | photo/20190729/1114/ 20190719181741268_2_56_0_0_unknown.jpg |

FlowReset

| FlowReset Json Block | { "FlowReset": { "ResetNowEnabled":, "AutoReset": { "Enabled":, "Time": } } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FlowReset | M | Jsonblock | Flow resetting configuration. | ---- |
| ResetNowEnabled | M | boolean | Enable or disable immediate counter reset. 0: Disable 1: Enable | 1 |
| AutoReset | 0 | Jsonblock | Required when timed reset is enabled. | ---- |
| Enabled | 0 | boolean | Enable or disable timer reset. Default: Disable 0: Disable 1: Enable | 1 |
| Time | 0 | string | Scheduled reset time. Default: 00:00:00. Reset cycle: 24 hours [00:00:00, 23:59:59]. Length range: [0, 12] | "12:34:56" |

GeolocationInfo

| GeolocationInfo Json Block | { "Longitude":, "Latitude": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Longitude | M | double | Longitude. | 50.1678 |
| Latitude | M | double | Latitude. | 50.1678 |

IdentificationInfo

| IdentificationInfo Json Block | { "Type": , "Number": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | ID type 0: ID card 1: IC card 2: Passport 3: Driver’s license 99: Others IC card is applicable to PTS only. | 0 |
| Number | M | String | ID number. Range: [1,127]. | "33022319890520222X" |

ImportInfo

| ImportInfo Json Block | { "MaxMemberNum": , "ImportedSum": , "Num":, "DatabaseImportList": [<DatabaseImportInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MaxMemberNum | M | unsigned long | Maximum number of face library members. | 2000 |
| ImportedSum | M | unsigned long | Total number of imported members. | 1000 |
| Num | M | unsigned long | Number of face libraries. | 2 |
| DatabaseImportList | C | array | Face libraries with imported data Optional when Num is 0. |  |
| DatabaseImportInfo | M | JsonBlock | Face library import info. | See DatabaseImportInfo |

LightActParamInfo

| LightActParamInfo Json Block | { "Interval":, "Luminance": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Interval | M | unsigned long | Light alarm interval (unit: second). Range: [1,60]. Default: 10s. | 10 |
| Luminance | M | unsigned long | Light brightness. Range: [1,1000]. Default: 50. | 50 |

HybridDetectionCapInfo

| HybridDetectionCapInfo Json Block | { "IsSupported": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsSupport | M | boolean | Whether mixed traffic detection is supported: 0: No 1: Yes | 0 |

MonitorAlarmInfo

| MonitorAlarmInfo Json Block | { "MonitorReason": , "MonitorAlarmType": , "MemberID": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MonitorReason | C | unsigned long | Monitoring cause 0: Robbed vehicle 1: Stolen vehicle 2: Suspect vehicle 3: Traffic violation 4: Emergency control 255: All types Valid only for vehicle alarm records and present only when the alarm is a vehicle match alarm. | 0 |
| MonitorAlarmType | C | unsigned long | Monitoring alarm type. Expressed using an enumeration value. Required in alarm records only. 0: Match alarm 1: Not match alarm | 0 |
| MemberID | C | unsignedlong | Vehicle member ID. Valid only for vehicle alarm records and present only when the alarm is a vehicle match alarm. | 0 |

MonitorDevResultInfo

| MonitorDevResultInfo Json Block | { "DevID": , "MonitorResult": <MonitorResultInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DevID | M | unsigned long | Monitoring device ID. | 3 |
| MonitorResult | M | Json Block | Info about device monitoring result. See MonitorResultInfo Json Block | - |

MonitorInfo

| MonitorInfo Json Block | { "ID":, "MointorBaseInfo": <MonitorRuleInfo> , “LinkageStrategyNum”, “LinkageStrategy”:[{ "Type":, "LinkageActions":<LinkageActionList> },…], "WeekPlan": <WeekPlanInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Face monitoring task ID. Required when using the GET method. | - |
| MointorBaseInfo | M | JsonBlock | Face monitoring task info. See MonitorRuleInfo Json Block. | - |
| LinkageStrategyNum | M | unsigned long | Number of alarm linkage strategies. | 1 |
| LinkageStrategy | C | array | Alarm linkage strategies. Optional when LinkageStrategyNum is 0. |  |
| Type | M | unsigned long | Alarm type 1: Match alarm 2: Not match alarm | 1 |
| LinkageActions | C | JsonBlock | Linkage actions of face monitoring task. See LinkageActionListJson Block. Applicable to NVR only. |  |
| WeekPlan | C | JsonBlock | Arming schedule of face monitoring task. See WeekPlanInfoJson Block. Applicable to NVR and IPC. |  |

MonitorList

| MonitorList Json Block | { "Num": , "MonitorList": [<MonitorInfo>, ...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of face monitoring tasks. | 3 |
| MonitorList | C | array | Face monitoring task list. This field is not returned when Num is 0. | - |
| MonitorInfo | M | JsonBlock | Info about face monitoring task. See MonitorInfoJson Block. | - |

MonitorRuleInfo

| MonitorInfoRule Json Block | { "Enabled": , "MonitorType":, "Name": , "Begin": , "End": "MonitorType": , "Reason": , "DatabaseID", "MemberNum" : , "MemberInfoList":[{ “MemberID”:, “MemberName”: },…] "DevNum" : , "MonitorDevIDList":[<DevID>,…] , } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | unsigned long | Whether face monitoring task is enabled. 0: Disable 1: Enable | 1 |
| MonitorType | C | unsigned long | Monitoring alarm type. Applicable to VMS only. 0: Match alarm 1: Not match alarm | 0 |
| Name | M | string | Name of face monitoring task. Range: [1,63]. | “Monitoring task” |
| Begin | C | unsigned long | Start time (unit: second, UTC format). Applicable to VMS only. | 1520412142 |
| End | C | unsigned long | End time (unit: second, UTC format). Applicable to VMS only. | 1520412142 |
| MonitorType | M | unsigned long | Monitoring alarm type. 0: Match alarm 1: Not match alarm | 0 |
| Reason | M | string | Cause of the face monitoring task. Range: [1,63]. | “Search target“ |
| DatabaseID | O | unsigned long | Face library ID. Required when getting details about a monitoring task. | 1 |
| MemberNum | C | unsigned long | Number of face members. Range: [0,32]. Required when getting details about a monitoring task. | 3 |
| MemberInfoList | M | array | Face member info list. Optional when MemberNum is 0. | - |
| MemberID | M | unsigned long | Face member ID. Required when getting details about a monitoring task. | 1 |
| MemberName | M | string | Member name. Range: [1,63]. | “Zhangsan” |
| DevNum | C | unsigned long | Number of monitoring devices used in the face monitoring task. Range: [1-4]. Required when getting details about a monitoring task. | 1 |
| MonitorDevIDList | M | array | List of monitoring devices used in the face monitoring task. Absent when DevNum is 0. | - |
| DevID | M | unsigned long | ID of monitoring device used in the face monitoring task. | 1 |

MonitorResultInfo

| MonitorResultInfo Json Block | { "Num":, "Result":[{ "ID":, "Name":, "ResultCode": }, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of returned batch operations. | 1 |
| Result | C | array | Result info list. Optional when Num is 0. | - |
| ID | M | unsigned long | Device ID | 1 |
| Name | M | string | Member name. Length range: [1,63]. | “” |
| ResultCode | M | unsigned long | Returned error code. See VMS error code. | 100 |

MonitorMemberInfo

| DatabaseInfo Json Block | { "MemberID": , "Name", "DevNum": , "DevMonitorInfoList":[<DevMonitorInfo>,…], "MonitorStatus": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MemberID | M | unsigned long | Member ID. | 3 |
| Name | M | String | Member name. | - |
| DevNum | C | unsigned long | Number of monitoring devices. Required when getting monitored face library members. | 0 |
| DevMonitorInfoList | M | unsigned long | Monitoring device info list. Optional when DevNum is 0. See DevMonitorInfo Json Block. | 3 |
| MonitorStatus | M | unsigned long | Monitoring status: 0: Monitoring failed 1: Monitoring succeeded | 0 |

MonitorVehicleInfo

| MonitorVehicleInfo Json Block | { " PlateNo": , " VehicleImage": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PlateNo | M | string | Plate number. Range:[1,15]. | “A00000“ |
| VehicleImage | 0 | Json Block | Vehicle image for comparison. See FileInfo Json Block. The size of the encrypted image should not exceed 4M. Absent when the monitoring type is “allowlist”. | - |

NonMotorVehicleRecord

| NonMotorVehicleRecord Json Block | { "RecordID":, "PassingTime":, "ChannelName":, , “NonMotorVehicleInfo”:<NonMotorVehicleInfo>, “NoMotorVehicleImage”:<FileInfo>, “PanoImage”:<FileInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | ID of structured non-monitor vehicle record. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | String | Camera name. Range: [1,63] | “XX road intersection” |
| NonMotorVehicleInfo | M | Json Block | Non-motor vehicle info. See NonMotorVehicleInfo Json Block。 | -- |
| NoMotorVehicleImage | M | Json Block | Snapshot of non-motor vehicle. See FileInfo Json Block. The size of the encrypted image should not exceed 1M. | -- |
| PanoImage | C | Json Block | Panoramic image. See FileInfo Json Block. Only image URL and size are present. Use /LAPI/V1.0/System/Picture to get image data. | -- |

ObjectCountInfo

| ObjectCountInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable counting in mixed traffic. 0: Disable 1: Enable | 0 |

ObjectSize

| ObjectSize Json Block | { "Width":, "Height": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Object width. Range: [0,9999]. | 5000 |
| Height | M | unsigned long | Object height. Range: [0,9999]. | 5000 |

OutputSwitchActParamInfo

| OutputSwitchActParamInfo Json Block | { "Num":, "IDs":[<ID>,<ID>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of linked output switches | M |
| IDs | C | array | List of linked output switches. | C |
| ID | C | unsigned long | Alarm output switch channel ID. For NVR, the value is video channel ID*100+alarm output ID (channel ID of NVR itself is 0). For VMS, the ID is configured on the VMS. | 1 |

PersonPassRecordInfo

| PersonPassRecordInfo Json Block | { "RecordID": , "PassingTime": , "ChannelName": , "FaceAttr": <FaceAttr>, “PersonAttr": <PersonAttr>, “FaceImage”:<FileInfo>, “PersonImage”:<FileInfo>, “PanoImage”:<FileInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | ID of structured person record. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| FaceAttr | 0 | Json Block | Face attribute info. See FaceAttr Json block. | - |
| PersonAttr | M | Json Block | Person attribute info. See PersonAttr Json Block |  |
| FaceImage | 0 | Json Block | Captured face image. See FileInfo Json Block. The size of the encrypted image should not exceed 1M. |  |
| PersonImage | M | Json Block | Captured person image. See FileInfo Json Block. The size of the encrypted image should not exceed 1M. |  |
| PanoImage | M | Json Block | Panoramic image. See FileInfo Json Block. Only image URL and size are present. Use /LAPI/V1.0/System/Picture to get image data. |  |

PersonSnapshotInfo

| PersonSnapshotInfo Json Block | { "Enabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable capturing person image (snapshot). 0: Disable 1: Enable | 1 |

PercentInfo

| PercentInfo Json Block | { "Percent": } |
| --- | --- |

| Param | Ranger | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Percent | M | unsigned long | Progress. Range: [0,100] | 10 |

PicDataInfo

| PicDataInfo Json Block | { "ID": , "Time":, “Size”:, “Name”:, “PicData”: } |
| --- | --- |

| Param | Ranger | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Channel ID. | 1 |
| Time | M | unsigned long | UTC time (unit: second) | 1451577600 |
| Size | M | unsigned long | Image size. Unit: Byte. | 61440 |
| Name | M | string | Image name. Length: [1,64] | “aaa” |
| PicData | M | String | Image data. Base64-encoded. | -- |

PlateAttr

| PlateAttr Json Block | { "PlateNo":, "Color":, "Type": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PlateNo | M | string | Plate number. Range:[1,15] | "A00000" |
| Color | M | unsigned long | Plate color 0: Black 1: White 2: Gray 3: Red 4: Blue 5: Yellow 6: Orange 7: Brown 8: Green 9: Purple 10: Cyan 11: Pink 12: Transparent 99: Others | 0 |
| Type | M | unsigned long | Plate type 0: Large vehicle plate 1: Small vehicle plate 2: Embassy vehicle plate 3: Consulate vehicle plate 4: Overseas vehicle plate 5: Foreign vehicle plate 6: Common motorcycle plate 7: Light motorcycle plate 8: Embassy motorcycle plate 9: Consulate motorcycle plate 10: Overseas motorcycle plate 11: Foreign motorcycle plate 12: Low speed vehicle plate 13: Tractor plate 14: Trailer plate 15: Coach vehicle plate 16: Coach motorcycle plate 17: Temporary entry vehicle plate 18: Temporary entry motorcycle plate 19: Temporary plate 20: Police vehicle plate 21: Police motorcycle plate 22: Agricultural vehicle plate 23: Border crossing vehicle plate (Hong Kong) 24: Border crossing vehicle plate (Macau) 25: Armed police plate 26: Military plate 99: Other | 1 |

Point

| Point Json Block | { “X”:, “Y”: } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| X | M | unsigned long | x-coordinate. Range: [0,9999] | 777 |
| Y | M | unsigned long | y-coordinate. Range: [0,9999] | 888 |

PresetActParam

| PresetActParam Json Block | { “Num“:, “ChannelPresetList”: [<ChannelPreset>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of linked PTZ presets | M |
| ChannelPresetList | C | array | List of linked PTZ presets | C |
| ChannelPreset | M | Json block | Channel and preset ID | See ChannelPreset Json Block |

PupilDistanceCapInfo

| PupilDistanceCapInfo Json Block | { "Resolution":< Resolution >, "PupilDistance":<PupilDistanceSize> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Resolution | M | Json Block | Resolution. See Resolution JsonBlock. | -- |
| PupilDistance | M | Json Block | Pupillary distance range. See PupilDistanceSize JsonBlock. | -- |

PupilDistanceSize

| PupilDistanceSize Json Block | { "Min": , "Max": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Min | M | unsigned long | Minimum pupillary distance, expressed using ‱. Range: [0, 9999]. | 416 |
| Max | M | unsigned long | Maximum pupillary distance, expressed using ‱. Range: [0, 9999]. | 2604 |

QueryMemberList

| QueryMemberList Json Block | { "Total": "Offset": "Num": "MemberList":[<MemberInfo>, ...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Total | M | unsigned long | Total number of members in the face library. | 200 |
| Offset | M | unsigned long | Library member ID from where the search starts. | 0 |
| Num | M | unsigned long | Total number of face library members returned. | 20 |
| MemberList | C | array | List of face library members returned. Optional when Num is 0. | - |
| MemberInfo | C | JsonBlock | List member info. Optional when Num is 0. | - |

RegionInfo

| RegionInfo Json Block | { "Nation": , "Province": , "City": , } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nation | M | string | Nationality. Range: [1,63] | “China” |
| Province | M | string | Province. Range: [1,63]. | “Zhejiang” |
| City | M | string | City. Range: [1,63]. | “Hangzhou” |

Resolution

| Resolution Json Block | { "Width": , "Height": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Width | M | unsigned long | Image width. | 1920 |
| Height | M | unsigned long | Image height. | 1080 |

RuleInfo

| RuleInfo Json Block | { "RuleType", "TrigerType", "PointNum": , "PointList": [ { "X": , "Y": }, … ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RuleType | M | unsigned long | Rule type: 0: Intrusion detection 1: Cross line detection 2: Leave area 3: Enter area | 0 |
| TrigerType | 0 | unsigned long | Trigger type: 0: None 1: Bi-directional 2: Clockwise 3: Counterclockwise 4: Enter 5: Leave | 0 |
| PointNum | M | unsigned long | Number of points (vertices) | 6 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |

SmartCompatibleRelationInfo

| SmartCompatibleRelationInfo Json Block | { "Num":, "IDs":[<ID>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of other smart functions that can be used simultaneously with a smart function. | 1 |
| IDs | M | Json Block | IDs of all the other smart functions that can be used simultaneously with the smart function. | - |
| ID | M | unsigned long | Smart function ID: 0: Face detection 1: People counting/people flow counting 2: Traffic monitoring/road monitoring 3: Auto tracking 4: Motion object detection 100: Cross line detection (CrossLineDetection) 101: Intrusion detection (IntrusionDetection) 102: Enter area (AccessZone) 103: Leave area (LeaveZone) 104: Loitering detection (HoverZone) 105: Quick moving (FastMove) (100-105 are defined as SMART1) 200: People gathering (Gather) 201: Parking detection (CarStop) 202: Object left behind (RemainArticle) 203: Object removed (RemainArticle) 204: Defocus detection (OutFocus) 205: Scene change (ScenceChange) 206: Heat map/Heatmap (200-206 are defined as SMART2) 300: Helmet detection (SafetyHelmetDetection) 301: Work clothes detection (WorkClothesDetection) | 1 |

SmartDetectionMode

| SmartDetectionMode Json Block | { "Mode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Service type: 0: Normal mode 1: Filter repetitive movement (objects with repetitive movement will be filtered during smart detection) | 0 |

SmartModeInfo

| SmartModeInfo Json Block | { "Mode": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Mode | M | unsigned long | Smart mode. 0: General 1: Traffic monitoring | 0 |

SmartMutexRelationInfo

- Smart function ID (grouped by mutual exclusion relationship)

| Smart Service Group ID | Smart Service ID | Function Description | Remarks |
| --- | --- | --- | --- |
| 1 | 0 | Face detection |  |
| 2 | 1 | People counting (people flow counting) |  |
| 3 | 2 | Traffic monitoring (road monitoring) |  |
| 4 | 4 | Motion object detection |  |
| 5 | 3 | Auto tracking | STRUCT1 (structured) |
|  | 6 | Mixed traffic detection |  |
| 6 | 5 | Box-dome camera linkage | STRUCT2 (structured) |
| 7 | 3 | Auto tracking | SMART1 |
|  | 100 | Cross line detection (CrossLineDetection) |  |
|  | 101 | Intrusion detection (IntrusionDetection) |  |
|  | 102 | Enter area (AccessZone) |  |
|  | 103 | Leave area (LeaveZone) |  |
|  | 106 | Box-dome camera linkage (BoxDomeLink) |  |
| 8 | 200 | People gathering (Gather) | SMART2 |
|  | 201 | Parking detection (CarStop) |  |
|  | 202 | Object left behind (RemainArticle) |  |
|  | 203 | Object removed (CareArticle) |  |
|  | 204 | Defocus detection (OutFocus) |  |
|  | 205 | Scene change (ScenceChange) |  |
|  | 206 | Heat map (Heatmap) |  |
|  | 207 | Loitering detection (HoverZone) |  |
|  | 208 | Quick moving (FastMove) |  |
| 9 | 300 | Helmet detection (SafetyHelmetDetection) | Production safety |
|  | 301 | Work clothes detection (WorkClothesDetection) |  |

NOTE!

- The smart service groups are mutually exclusive and conflict with each other. Exception: For non-deep learning perimeter protection devices, SMART1 does not conflict with SMART2.

- Smart services in the same group may co-exist. Exception: In group 7, Auto tracking does not co-exist with box-dome linkage.

- Auto tracking falls into both Group 5 (STRUCT1) and Group 7 (SMART1). Auto tracking can be enabled simultaneously with other functions in group 5 or group 7; it cannot be enabled simultaneously with other functions in both group 5 and group 7.

- Compatibility relationship between smart functions/smart functions that can be enabled simultaneously (reference)

| Priority level | Smart function | Remarks |
| --- | --- | --- |
| 0 | Face detection | After face detection is enabled, no other smart functions can be enabled. |
| 1 | People counting/people flow counting | After people counting is enabled, no other smart functions can be enabled. |
| 2 | Traffic monitoring/road monitoring | After traffic monitoring is enabled, no other smart functions can be enabled. |
| 3 | Auto tracking | After auto tracking is enabled, no other smart functions can be enabled. |
| 4 | Chain calculation phase I (obsolete, no longer supported) | After chain calculation (phase I) is enabled, no other smart functions can be enabled. |
| 5 | SMART1+SMART2 | SMART1 and SMART2 can be enabled simultaneously |

Remarks:

- 0 is the lowest priority, and 5 is the highest.

- In IPC, high-priority smart functions are configured by default. If conflicting or mutually exclusive smart functions are enabled simultaneously, then high-priority smart functions prevail.

- Mutually exclusive smart functions/smart functions that can't be enabled simultaneously (for reference)

- Face detection, people counting, traffic monitoring (road monitoring), and auto tracking, these four functions are mutually exclusive, and only one function is available at a time.

- Auto tracking conflicts with face detection, people counting, traffic monitoring, heat map, scene change detection, defocus detection, object left behind/removed, parking detection, and people gathering.

- Face detection conflicts with all the other functions.

- People counting conflicts with face detection, traffic monitoring (road monitoring), and auto tracking because people counting performance will be affected when any of the other three is enabled.
(Remarks: People counting algorithm can be used by the other three; the relationship is defined as “conflicting” because sharing the algorithm will affect people counting performance).

- Traffic monitoring conflicts with all the other functions.

- Heat map conflicts with auto tracking.

- Scene change detection, defocus detection, object left behind/removed, parking detection, people gathering, cross line detection, intrusion detection, enter/leave area, quick moving, loitering detection must not be enabled at the same time with the above conflicting functions.

| SmartMutexRelationInfo Json Block | { "Num":， "CompatibleRelations":[<SmartCompatibleRelationInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of arrays involving compatibility relationship. Currently 8 groups in total. | 8 |
| CompatibleRelations | M | Json Block | Compatibility relations between smart functions. See SmartCompatibleRelationInfo Json block. | - |
| SmartCompatibleRelationInfo | M | Json Block | Compatibility relationship between smart functions. | - |

SmartWorkingStatusInfo

| SmartWorkingStatusInfo Json Block | { "EnableNum": "EnableIDs": [<ID>,…], "DisableNum": "DisableIDs": [<ID>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| EnableNum | M | unsigned long | Number of enabled smart functions. Note: EnableNums+DisableNums=Total number of smart functions. | 6 |
| EnableIDs | M | array | IDs of enabled smart functions. | [100,101,102,103,104,105] |
| DisableNum | M | unsigned long | Number of disabled smart functions. Note: EnableNums+DisableNums=Total number of smart functions. | 11 |
| DisableIDs | M | array | IDs of disabled smart functions. | [0, 1, 2, 3, 4, 200, 201, 202, 203, 204, 205] |
| ID | M | unsigned long | Smart function ID: 0: Face detection 1: People counting/people flow counting 2: Traffic monitoring/road monitoring 3: Auto tracking 4: Motion object detection 5: Box-dome camera linkage 6: Mixed traffic detection 100: Cross line detection (CrossLineDetection) 101: Intrusion detection (IntrusionDetection) 102: Enter area (AccessZone) 103: Leave area (LeaveZone) 104: Loitering detection (HoverZone) 105: Quick moving (FastMove) (100-105 are defined as SMART1) 200: People gathering (Gather) 201: Parking detection (CarStop) 202: Object left behind (RemainArticle) 203: Object removed (CareArticle) 204: Defocus detection (OutFocus) 205: Scene change (ScenceChange) 206: Heat map/Heatmap (200-206 are defined as SMART2) 300: Helmet detection (SafetyHelmetDetection) 301: Work clothes detection (WorkClothesDetection) | 1 |

SnapshotImage

| SnapshotImage Json Block | { "BigImage":<FileInfo>, "SmallImage": <FileInfo>, “PersonImage”:<FileInfo>, "Area": { "left":, "top":, "right":, "bottom": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| BigImage | M | Json Block | Panoramic image. Absent when searching faces or pushing snapshots. See FileInfo Json Block. | -- |
| SmallImage | C | Json Block | Close-up image of face. See FileInfo Json Block. IPC can be configured not to push close-up images. | -- |
| PersonImage | C | Json Block | Close-up image of body. Present in structured queries only. See FileInfo Json Block. | -- |
| Area | M | Json Block | Coordinates of area. | -- |
| left | M | unsigned long | Coordinates of the left point. Range: [0,9999] | 0 |
| top | M | unsigned long | Coordinates of the top point. Range: [0,9999] | 0 |
| right | M | unsigned long | Coordinates of the right point. Range: [0,9999] | 100 |
| bottom | M | unsigned long | Coordinates of the bottom point. Range: [0,9999] | 100 |

StaticObjectDetectionInfo

| StaticObjectDetectionInfo Json Block | { "VehicleEnabled":, "NonVehicleEnabled":, "PersonEnabled":, "FaceEnabled": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| VehicleEnabled | M | boolean | Enable or disable motor vehicle detection. 0: Disable 1: Enable | 0 |
| NonVehicleEnabled | M | boolean | Enable or disable non-motor vehicle detection. 0: Disable 1: Enable | 0 |
| PersonEnabled | M | boolean | Enable or disable pedestrian detection. 0: Disable 1: Enable | 0 |
| FaceEnabled | M | boolean | Enable or disable face detection. 0: Disable 1: Enable | 0 |

VehicleAttr

| VehicleAttr Json Block | { "Type":, "Color":, “SpeedUnit”, "SpeedValue": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Vehicle type 0: 3-wheel vehicle 1: Large bus 999: Others | 0 |
| Color | M | unsigned long | Vehicle color 0: Black 1: White 2: Gray 3: Red 4: Blue 5: Yellow 6: Orange 7: Brown 8: Green 9: Purple 10: Cyan 11: Pink 12: Transparent 99: Others | 0 |
| SpeedUnit | C | unsigned int | Vehicle speed unit 0: kph 1: mph Absent when querying vehicle library resources. | 0 |
| SpeedValue | C | double | Vehicle speed Absent when querying vehicle library resources. | 70 |

VehicleInfo

| VehicleInfo Json Block | { "ReqSeq":, "MemberID”:, "PlateAttr":<PlateAttr>, "VehicleAttr":<VehicleAttr>, "VehicleImage":<FileInfo>, "IsMonitored":, "DatabaseNum":, "DatabaseIDList":[<DatabaseID>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ReqSeq | C | unsigned long | Sequence number of request, which will be returned with results. Required in batch adding only. Applicable to VMS only. |  |
| MemberID | C | unsigned long | Face library member ID. Required when using GET or PUT method. | 1 |
| PlateAttr | M | Json Block | Plate info. See PlateAttr Json Block. | - |
| VehicleAttr | M | Json Block | Vehicle info. See VehicleAttr Json Block. | - |
| VehicleImage | C | Json Block | Vehicle image. See FileInfo Json Block. The size of the encrypted image should not exceed 4M. | - |
| IsMonitored | M | boolean | Whether monitoring is enabled: 0: No 1: Yes | 0 |
| DatabaseNum | M | unsigned long | Number of vehicle libraries containing the vehicle. | 3 |
| DatabaseIDList | C | Array | IDs of vehicle libraries containing the vehicle. Absent when DatabaseNum is 0. | - |
| DatabaseID | M | unsigned long | Vehicle library ID. | 3 |

VehicleMonitorInfo

| VehicleMonitorInfo Json Block | { "ID":, "Rule": <VehicleMonitorRuleInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Monitoring task ID. Required when using the GET method. | 0 |
| Rule | M | Json Block | Monitoring task info. See VehicleMonitorRuleInfo Json Block | - |

VehicleMonitorRuleInfo

| VehicleMonitorInfoRule Json Block | { "Enabled": , "Name": , "Begin": , "End": , "MonitorAlarmType ":, "MonitorReason": , "Description": , "DatabaseID" , "MemberNum" : , "MemberInfoList":[{ “MemberID”:, “MemberName”: },…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | Boolean | Whether monitoring task is enabled. 0: Disable 1: Enable | 1 |
| Name | M | String | Name of vehicle monitoring task. Range: [1,63] | “Robbed vehicle” |
| Begin | M | unsigned long | Start time (unit: second, UTC format) | 1520412142 |
| End | M | unsigned long | End time (unit: second, UTC format) | 1520412142 |
| MonitorAlarmType | M | unsigned long | Monitoring alarm type. 0: Match alarm 1: Not match alarm | 0 |
| MonitorReason | C | unsigned long | Monitoring cause 0: Robbed vehicle 1: Stolen vehicle 2: Suspect vehicle 3: Traffic violation 4: Emergency control Absent when MonitorAlarmType is Not Match Alarm. | 0 |
| Description | M | String | Description of vehicle monitoring task. Range: [1,63] | “Searching robbed vehicle“ |
| DatabaseID | 0 | unsigned long | Vehicle library ID. Required when getting details about a monitoring task. | 1 |
| MemberNum | C | unsigned long | Number of vehicle members. Range: [0,32]. Required when getting details about a monitoring task. | 3 |
| MemberInfoList | M | Array | Vehicle member ID list. Not returned when MemberNum is 0. | - |
| MemberID | M | unsigned long | Vehicle member ID. | 1 |
| MemberName | M | string | Vehicle member name. Range: [1,15]. | “A66666” |

VehicleMonitorList

| VehicleMonitorList Json Block | { "Num": , "VehicleMonitorList": [<VehicleMonitorInfo>, ...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of monitoring tasks. | 3 |
| VehicleMonitorList | C | array | Monitoring task list. Absent when Num is 0. | - |
| VehicleMonitorInfo | M | Json Block | Monitoring task info. See VehicleMonitorInfo Json Block. | - |

VehicleMonitorRuleInfoIDList

| VehicleMonitorRuleInfoIDList Json Block | { "Num":, "VehicleMonitorRuleInfoIDList":[<VehicleMonitorRuleInfoID>, ...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of monitoring tasks. | 1 |
| VehicleMonitorRuleInfoIDList | C | array | Monitoring task ID list. Optional when Num is 0. | -- |
| VehicleMonitorRuleInfoID | M | Json Block | Monitoring task ID, Optional when Num is 0. | 1 |

VehiclePassRecordInfo

| VehiclePassRecordInfo Json Block | { "RecordID":, "PassingTime":, "ChannelName":, "MonitorAlarmInfo": <MonitorAlarmInfo>, "PlateAttr":<PlateAttr>, "VehicleAttr": <VehicleAttr>, "PlateImage": <FileInfo>, "VehicleImage":<FileInfo>, "PanoImage":<FileInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | LPR camera name. Range: [1,63]. | “XX road intersection” |
| MonitorAlarmInfo | C | Json Block | Plate monitoring alarm info. See MonitorAlarmInfoJson Block. Required when the record is a vehicle alarm record. |  |
| PlateAttr | M | Json Block | Plate attributes info. See PlateAttr Json Block. | - |
| VehicleAttr | M | Json Block | Vehicle attributes info. See VehicleAttr Json Block. | - |
| PlateImage | M | Json Block | Captured vehicle image. See FileInfo Json Block. The size of the encrypted image should not exceed 1M. | - |
| VehicleImage | C | Json Block | Captured vehicle image. Present in structured queries. See FileInfo Json Block. The size of the encrypted image should not exceed 1M. |  |
| PanoImage | C | Json Block | Panoramic image. Present in structured queries. See FileInfo Json Block. Only image URL and size are present. Use /LAPI/V1.0/System/Picture to get image data. |  |

VehicleSnapshotInfo

| VehicleSnapshotInfo Json Block | { "RecordID":, "ChannelID":, "EventType":, "PassingTime":, "ChannelName":, , "MonitorReason": , "MemberID": , "PlateAttr":<PlateAttr>, "VehicleAttr": <VehicleAttr>, "PlateImage": <FileInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Vehicle snapshot record ID. | 1 |
| ChannelID | M | unsigned long | Channel ID. | 1 |
| EventType | M | unsigned long | Event type. Described by bit order. BIT=1 means the event is this type. BIT=0 means the event is not this type. BIT0: Vehicle capture BIT1: Vehicle match alarm BIT2: Vehicle not match alarm | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | LPR camera name. Range: [1,63]. | “XX road intersection” |
| MonitorReason | C | unsigned long | Monitoring cause 0: Robbed vehicle 1: Stolen vehicle 2: Suspect vehicle 3: Traffic violation 4: Emergency control 255: All types Present when EventType is “vehicle match alarm”. | 0 |
| MemberID | C | unsigned long | Vehicle member ID. Present when EventType is “vehicle match alarm”. |  |
| PlateAttr | M | Json Block | Plate attributes info. See PlateAttr Json Block. | - |
| VehicleAttr | M | Json Block | Vehicle attributes info. See VehicleAttr Json Block. | - |
| PlateImage | M | Json Block | Captured vehicle image. See FileInfo Json Block. The size of the encrypted image should not exceed 100K. | - |

VolumeInfo

| VolumeInfo Json Block | { "IsAbnormal":, "Volume": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| IsAbnormal | M | boolean | Whether sound volume is normal. 0: Normal 1: Abnormal | 0 |
| Volume | M | unsigned long | Sound volume. Range: [0 400]. | 0 |

VolumeInfoList

| VolumeInfoList Josn Block | { "Nums":, "Volumes":[<VolumeInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nums | M | unsigned long | Frame rate. Range: [0 30] | 20 |
| Volumes | C | array | Volume info list. Optional when Nums is 0. | - |
| VolumeInfo | M | Json Block | Volume info. See VolumeInfo Json Block | - |

WeekPlanInfo

| WeekPlanInfo Json Block | { "Enabled": , "Num": , "Days": [<DayPlanInfo>, …] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | C | boolean | Enable or disable arming schedule. 0: Disable 1: Enable (default) Applicable to IPC only. | 1 |
| Num | M | unsigned long | Number of scheduled days NVR: Max 8 (seven days per week and holiday). IPC: Max 7 (seven days per week). | 8 |
| Days | M | array | Arming schedule list of every day in a week. |  |
| DayPlanInfo | M | Json Block | Arming schedule details of every day. See DayPlanInfo Json Block |  |

PassRecordInfo

| PassRecordInfo Json Block | { "RecordID": , "PassingTime": , "ChannelName":, "ChannelID":, "Similarity":, "AttributeInfo":<FaceAttr>, "SnapshotImage": <SnapshotImage> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| ChannelID | M | unsigned long | Channel ID | 1 |
| Similarity | M | unsigned long | Matching degree (similarity) | - |
| SnapshotImage | M | Json Block | Image info |  |
| AttributeInfo | C | Json Block | Face attribute info. Present if exists. See FaceAttr. | - |

SnapshotImage

| SnapshotImage Json Block | { "BigImage":<FileInfo>, "SmallImage": <FileInfo>, "Area": { "left":, "top":, "right":, "bottom": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| BigImage | M | Json Block | Original image. Absent when searching faces or pushing snapshots. See FileInfo Json Block. |  |
| SmallImage | C | Json Block | Close-up image. See FileInfo Json Block. IPC can be configured not to push close-up images. |  |
| Area | M | Json Block | Coordinates of the close-up image in the original image. |  |
| left | M | unsigned long | Coordinates of the left point. |  |
| top | M | unsigned long | Coordinates of the top point. |  |
| right | M | unsigned long | Coordinates of the right point. |  |
| bottom | M | unsigned long | Coordinates of the bottom point. |  |

TimeSectionInfo

| TimeSectionInfo Json Block | { "Begin": , "End": , "ArmingType": , } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Begin | M | string | Start time. Format: hh:mm:ss Length range: [0,31] NVR does not support ss (seconds). | 00:00:00 |
| End | M | string | End time. Format: hh:mm:ss Length range: [0,31] NVR does not support ss (seconds). | 24:00:00 |
| ArmingType | C | unsigned long | Arming type 0: Schedule 1: Motion detection 2: Alarm 3: Motion detection and alarm 4: Motion detection or alarm 5: No schedule 10: Event Arming schedules only supports “Schedule”. Recording/snapshot schedules support all. Optional for IPC, which only supports “Schedule”. |  |

ZonePolygonInfo

| ZonePolygonInfo Json Block | { "ID": , "Enabled": , "Sensitivity": , "PointNum": , "PointList": [ { "X": , "Y": }, … ], "Priority":, "Num":, "DetectTargetList":[<DetectTargetInfo>,<DetectTargetInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Index of detection area, starts from 1, and the maximum is the value of Nums in Areas. | 1 |
| Enabled | M | boolean | Enable or disable detection area 0: Disable 1: Enable | 1 |
| Sensitivity | M | unsigned long | Sensitivity. Range: [1,100] | 1 |
| PointNum | M | unsigned long | Number of vertices of the polygon detection area. | 4 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |
| Priority | C | unsigned long | Priority level High: 0 Medium: 1 Low: 2 | 0 |
| Num | C | unsigned long | Number of detection objects. | 3 |
| DetectTargetList | C | array | Detection object list. Required when object parameters are present. |  |
| DetectTargetInfo | M | Json Block | Detection object info. See DetectTargetInfo Json Block |  |

PeopleLibraries

PeopleLibraries

- /LAPI/V1.0/PeopleLibraries/BasicInfo

| URL | /LAPI/V1.0/PeopleLibraries/BasicInfo |
| --- | --- |
| Description | Get info about all the created person libraries on the device. |
| Method | GET |
| Input Data | None |
| Success Return Data | LibraryList |
| Note | Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/PeopleLibraries/BasicInfo |
| --- | --- |
| Description | Change person library info. |
| Method | PUT |
| Input Data | LibraryList |
| Success Return Data | None |
| Note | 1. Currently only one library can be changed at a time (Num is 1). 2. Only the library name can be changed. Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/PeopleLibraries/BasicInfo |
| --- | --- |
| Description | Create person library info. |
| Method | POST |
| Input Data | LibInfo |
| Success Return Data | { "ID"： } |
| Note | - Only one library can be created at a time Applicable to NVR, VMS, and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Library ID that is generated when a library is created. | 3 |

- /LAPI/V1.0/PeopleLibraries/<ID>

| URL | /LAPI/V1.0/PeopleLibraries/<ID> |
| --- | --- |
| Description | Delete a face library from a device. |
| Method | DELETE |
| Input Data | { “IsDeletePerson”: } |
| Success Return Data | None |
| Note | 1. Currently only one library can be deleted at a time. Applicable to NVR, VMS, and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Person library ID | 3 |
| IsDeletePerson | C | unsinged long | Whether to delete persons in the library. 0: Not delete 1: Delete Applicable to VMS. | 0 |

- /LAPI/V1.0/PeopleLibraries/Capacity

| URL | /LAPI/V1.0/PeopleLibraries/Capacity |
| --- | --- |
| Description | Get capacity info of all person libraries |
| Method | GET |
| Input Data | NONE |
| Success Return Data | PeopleLibrariesCapacityInfo |
| Notes | Applicable to NVR and IPC. |

People

- /LAPI/V1.0/PeopleLibraries/<ID>/People

| URL | /LAPI/V1.0/PeopleLibraries/<ID>/People |
| --- | --- |
| Description | Add persons to a library. |
| Method | POST |
| Input Data | PersonInfoList |
| Success Return Data | PersonResultList |
| Note | 1. If a photo of a person failed to be synced, then all the info of the person failed to be added. 2. The total size of the HTTP packet must not exceed 6M (Content-Length: 6291456). Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/PeopleLibraries/<ID>/People |
| --- | --- |
| Description | Edit a person. |
| Method | PUT |
| Input Data | PersonInfoList |
| Success Return Data | PersonResultList |
| Note | 1. Currently only one member can be modified at a time (Num is 1). Applicable to NVR, VMS, and IPC. |

- /LAPI/V1.0/PeopleLibraries/<ID>/People/<ID>

| URL | /LAPI/V1.0/PeopleLibraries/<ID>/People/<ID> |
| --- | --- |
| Description | Delete a person. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | Applicable to NVR, VMS, and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| <ID> | M | unsigned long | Person library ID Note: PeopleLibraries/<ID> | 3 |
| <ID> | M | unsigned long | Person ID Note: People/<ID> | 3 |

- /LAPI/V1.0/PeopleLibraries/<ID>/People/Info

| URL | /LAPI/V1.0/PeopleLibraries/<ID>/People/Info |
| --- | --- |
| Description | Get person info with search criteria. |
| Method | POST |
| Input Data | { "Num":, "QueryInfos": [<QueryInfo>, ...] , "Limit": , "Offset": } |
| Success Return Data | { "Total":, "Offset":, "PersonList":<PersonInfoList> } |
| Note | Search criteria shall be provided by respective products and can specify whether image data need to be returned. Applicable to NVR, VMS, and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Person library ID | 3 |
| Num | M | unsigned long | Number of search criteria. | 3 |
| QueryInfos | C | Array | Search criteria list. Optional when Num is 0. | - |
| QueryInfo | M | Json Block | Detailed info about search criteria. See QueryInfo Json Block | - |
| Limit | M | unsigned long | Number of members to search. Max 20. | 12 |
| Offset | M | unsigned long | Sequence number to start search from. The sequence number starts from 0. | 0 |
| Total | M | unsigned long | Total number of results that meet the search criteria. | 500 |
| Offset | M | unsigned long | Current sequence number. The sequence number starts from 0. | 0 |
| PersonList | C | Json Block | Info list of persons that meet search criteria See PersonInfoList Json Block. | - |

MonitorRule

- /LAPI/V1.0/Smart/Face/Recognition/Monitor?ID=<ID>&Limit=<Limit>&Offset=<Offset>

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor?ID=<ID>&Limit=<Limit >&Offset=<Offset> |
| --- | --- |
| Description | Get all face monitoring tasks. |
| Method | GET |
| Input Data | None |
| Success Return Data | MonitorList |
| Note | Applicable to NVR, VMS, and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Channel ID. Required when getting monitoring tasks by channel. Applicable to NVR only. | 1 |
| Limit | C | unsigned long | Number of tasks to search each time. Max 25. Optional when getting monitoring tasks by channel. Required when getting all monitoring tasks. Only NVR supports paginated search of monitoring tasks. | 12 |
| Offset | C | unsigned long | Sequence number to start search from. The sequence number starts from 0. Optional when getting monitoring tasks by channel. Required when getting all the monitoring tasks. Applicable to NVR only. | 0 |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor |
| --- | --- |
| Description | Add a face monitoring task. |
| Method | POST |
| Input Data | MonitorInfo |
| Success Return Data | MonitorResultInfo |
| Note | Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor |
| --- | --- |
| Description | Delete monitoring tasks in batches. |
| Method | DELETE |
| Input Data | { "Num": "MonitorIDList":[<ID>, <ID>,...] } |
| Success Return Data | BatchDeleteResultInfo |
| Note | Applicable to NVR and VMS. |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID> |
| --- | --- |
| Description | Get a face monitoring task. |
| Method | GET |
| Input Data | None |
| Success Return Data | MonitorInfo |
| Note | Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID> |
| --- | --- |
| Description | Set a face monitoring task. |
| Method | PUT |
| Input Data | MonitorInfo |
| Success Return Data | None |
| Note | Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID> |
| --- | --- |
| Description | Delete a face monitoring task. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | Applicable to NVR, VMS, and IPC. |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/RuleInfo

| URL | LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/RuleInfo |
| --- | --- |
| Description | Get rules of a monitoring task. |
| Method | GET |
| Input Data | None |
| Success Return Data | MonitorRuleInfo |
| Note | Applicable to NVR, VMS, and IPC. |

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/RuleInfo |
| --- | --- |
| Description | Set rules of a monitoring task. |
| Method | PUT |
| Input Data | MonitorRuleInfo |
| Success Return Data | None |
| Note | Applicable to NVR, VMS, and IPC. |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/LinkageActions

| URL | LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/LinkageActions |
| --- | --- |
| Description | Get linkage actions of a face monitoring task. |
| Method | GET |
| Input Data | None |
| Success Return Data | "LinkageStrategyNum", "LinkageStrategy":[ { Type, "LinkageActionList":<LinkageActionList> }, { Type, "LinkageActionList":<LinkageActionList> }] |
| Note | Applicable to NVR and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LinkageStrategyNum | M | unsigned long | Number of alarm linkage strategies | 1 |
| LinkageStrategy | C | array | Alarm linkage configurations. Optional when LinkageStrategyNum is 0. |  |
| Type | M | unsigned long | Alarm type 1: Match alarm 2: Not match alarm | 1 |
| LinkageActions | C | JsonBlock | Linkage actions of face monitoring task. See LinkageActionList Json Block. Applicable to NVR and IPC. |  |

| URL | LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/LinkageActions |
| --- | --- |
| Description | Set linkage actions of a face monitoring task. |
| Method | PUT |
| Input Data | { "LinkageStrategyNum", "LinkageStrategy":[ { Type, "LinkageActionList":<LinkageActionList> }, { Type, "LinkageActionList":<LinkageActionList> },] } |
| Success Return Data | None |
| Note | Applicable to NVR and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LinkageStrategyNum | M | unsigned long | Number of alarm linkage strategies | 1 |
| LinkageStrategy | C | array | Alarm linkage configurations. Optional when LinkageStrategyNum is 0. |  |
| Type | M | unsigned long | Alarm type 1: Match alarm 2: Not match alarm | 1 |
| LinkageActions | C | JsonBlock | Linkage actions of face monitoring task. See LinkageActionList Json Block. Applicable to NVR and IPC. |  |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Plan

| URL | LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Plan |
| --- | --- |
| Description | Get the arming schedule of a face monitoring task. |
| Method | Get |
| Input Data | None |
| Success Return Data | { "WeekPlan": <WeekPlanInfo>, } |
| Note | Applicable to NVR and IPC. |

| URL | LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Plan |
| --- | --- |
| Description | Set the arming schedule of a face monitoring task. |
| Method | PUT |
| Input Data | { "WeekPlan": <WeekPlanInfo>, } |
| Success Return Data | None |
| Note | Applicable to NVR and IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| WeekPlans | C | JsonBlock | Arming schedule of face monitoring task. See WeekPlanInfoJson Block. Applicable to NVR and IPC. |  |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Member?Limit=<Limit>&Offset=<Offset>

| URL | /LAPI/V1.0/Smart/FaceRecognition/Monitor/<ID>/Member?Limit=<Limit>&Offset=<Offset> |
| --- | --- |
| Description | Get monitored members of a face monitoring task. |
| Method | GET |
| Input Data | None |
| Success Return Data | { "Total": "Offset": "MemberNum":, "MonitorMemberInfoList":[<MonitorMemberInfo>,…] } |
| Note | Applicable to IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Limit | M | unsigned long | Number of members to search each time. Max 50. | 30 |
| Offset | M | unsigned long | Sequence number to start search from. The sequence number starts from 0. | 0 |
| Total | M | unsigned long | Total number of monitored face members. | 200 |
| Offset | M | unsigned long | Library member ID from where the search starts. | 0 |
| MemberNum | M | unsigned long | Total number of monitored members returned. | 50 |
| MonitorMemberInfoList | C | array | Monitored member info list. Optional when MemberNum is 0. | 3 |
| MonitorMemberInfo | M | Json Block | Info about the monitored member. See MonitorMemberInfo json block. | - |

- /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Member

| URL | /LAPI/V1.0/Smart/Face/Recognition/Monitor/<ID>/Member |
| --- | --- |
| Description | Re-configure monitored face members in batches. |
| Method | PUT |
| Input Data | { "MemberNum":, "MonitorMemberInfoList":[<MonitorMemberInfo>,…] } |
| Success Return Data | None |
| Note | Applicable to IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MemberNum | M | unsigned long | Total number of monitored members returned. | 50 |
| MonitorMemberInfoList | C | array | Monitored member info list. Optional when MemberNum is 0. | 3 |
| MonitorMemberInfo | M | Json Block | Info about the monitored member. See MonitorMemberInfo json block. | - |

PassRecord

/LAPI/V1.0/Smart/Face/PassRecord

| URL | /LAPI/V1.0/Smart/Face/PassRecord |
| --- | --- |
| Description | Get pass-through records. |
| Method | POST |
| Input Data | { "Num":, "QueryInfos": [<QueryInfo>, ...] , "Limit": , "Offset": , } |
| Success Return Data | { "Total":, "Offset": , "Num":, "FacePassRecordInfoList": [<FacePassRecordInfo>, ...] } |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of search criteria. | 1 |
| QueryInfos | C | array | Search criteria list. Optional when Num is 0. | - |
| QueryInfo | M | Json Block | Search criteria. See QueryInfo Json Block. | - |
| Limit | M | unsigned long | Number of records to search each time. Max 32. | 20 |
| Offset | M | String long | Sequence number to start search from. The sequence number starts from 0. | 1 |
| Total | M | unsigned long | Total number of pass-through records. | 20 |
| Offset | M | unsigned long | Record ID from where the search starts. | 1 |
| Num | M | unsigned long | Number of pass-through records returned. | 20 |
| FacePassRecordInfoList | C | array | List of pass-through records returned. Optional when Num is 0. | - |
| FacePassRecordInfo | M | Json Block | Detailed pass-through record info. See FacePassRecordInfo Json Block. | - |

Json Block

PersonInfoList

| PersonInfoList Json Block | { "Num": , "PersonInfoList":[<PersonInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of members in the person library For PTS, up to 6 members can be added, modified, or deleted at a time. | 1 |
| PersonInfoList | M | Array | Person info list | -- |
| PersonInfo | C | JsonBlock | Person info. Optional when PersonNum is 0. | -- |

PersonInfo

| PersonInfo Json Block | { "ReqSeq":, "PersonID":, "LastChange":, "PersonCode":, "PersonName":, "Gender":, "Birthday": "Nation": "Telephone":, "Region" :<RegionInfo>, "Address" :, "IdentificationNum":, "IdentificationList": [<IdentificationInfo>,...], "ImageNum":, "ImageList":[{ "FaceID":, "ModelStatus":, <FileInfo> },...], "PersonPIN":, "IsMonitored":, "BelongLibNum": , "BelongLibIDList" : [<LibID>,<LibID>,…] "Desc": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ReqSeq | C | unsigned long | Sequence number of request, which will be returned with results. Required in batch adding only. Applicable to VMS only. |  |
| PersonID | M | unsigned long | Person ID | 1 |
| LastChange | M | unsigned long | Last time person info was modified (Unix timestamp) | 1510925018 |
| PersonCode | 0 | string | Person ID. Can be student ID or employee ID. Range: [1, 15] Applicable to PTS. | "3060911012" |
| PersonName | M | string | Name of the person. Range: [1, 63]. | “Zhangsan” |
| Gender | M | unsigned long | Gender. 0: Unknown 1: Male 2: Female 9: Unspecified The Unspecified type is applicable to PTS only. | 1 |
| Birthday | 0 | string | Birth date (YYYYMMDD). Range: [1,31] | "20180112" |
| Telephone | 0 | string | Phone number Length range: [1, 64]. | "123456789" |
| Region | 0 | Json Block | Region info. See RegionInfo Json Block. | - |
| Address | 0 | string | Address Length range: [1, 64]. | "xxx" |
| IdentificationList | M | array | Member ID info | -- |
| IdentificationInfo | C | Json Block | Optional when IdentificationNum is 0. See IdentificationInfo Json Block. | -- |
| ImageNum | M | unsigned long | Number of face images. Range: [0, 6]. | -- |
| ImageList | M | array | Face image info list | -- |
| FaceID | C | unsigned long | Face image ID Optional when ImageNum is 0. | 23 |
| IsModeled | C | boolean | Whether modeling is complete (whether face image has been converted into semi-structured data). 0: No 1: Yes Required when getting library members. Applicable to NVR. |  |
| FileInfo | C | Json Block | Image info. See FileInfo Json Block. Optional when ImageNum is 0. At the same level as FaceID, and no need to add extra brackets. For example: {"FaceID", "Name":, "Size":, "Data":} Note: File size range for PTS: [0, 768K] | -- |
| ModelStatus | C | unsigned long | Modeling status (whether face image has been converted into semi-structured data). 0: No 1: Yes 2: Failed Required when getting library members. Applicable to NVR. | 1 |
| PersonPIN | 0 | String | Password of the person. Digits only. Range: [8, 32]. Applicable to PTS. | "12345678" |
| BelongLibNum | C | unsigned long | Number of libraries the person belongs to. Optional when adding a person. Applicable to VMS only. | 3 |
| BelongLibIDList | C | array | Person library ID list. Absent when BelongLibNum is 0. Applicable to VMS only. | - |
| LibID | M | unsigned long | Person library ID. Applicable to VMS only. | 1 |
| Desc | 0 | string | Description of the person. String length range: [1, 128]. | "zzz" |

FacePassRecordInfo

| FacePassRecordInfo Json Block | { "RecordID": , "Type":, "PassingTime": , "ChannelName":, "ChannelID":, "CompareInfo": <ComapreInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| Type | C | unsigned long | Pass-through record type. 0: Face snapshot 1: Face match alarm 2: Face not match alarm | 0 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| CompareInfo | M | Json Block | Comparison info. See ComapreInfo Json Block. | - |

CompareInfo

| CompareInfo Json Block | { "Similarity":, "PersonInfo": <PersonInfo> "FaceAttr":<FaceAttr>, "PersonAttr":<PersonAttr>, "SnapshotImage": <SnapshotImage> "FeatureNum": , "FeatureList":[{ "FeatureVersion":, "Feature":, "FeatureManufacturer": },...], "CapSrc": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Similarity | C | unsigned long | Matching degree. Required for match alarms Required when Type is “match alarm” or “not match alarm”. |  |
| PersonInfo | C | JsonBlock | Person info. See PersonInfo Json Block. Required for match alarms. Required when Type is “match alarm” or “not match alarm”. |  |
| FaceAttr | C | Json Block | Face attributes. See FaceAttr Json Block. Applicable to VMS when querying structured data. |  |
| PersonAttr | C | Json Block | Person attributes. See PersonAttr Json Block. Applicable to VMS when querying structured data. |  |
| SnapshotImage | M | JsonBlock | Snapshot image. See SnapshotImage Json Block. |  |
| FeatureNum | 0 | unsigned long | Number of semi-structured features. Optional when there is no semi-structured feature. Required for PTS. |  |
| FeatureList | 0 | array | List of semi-structured features. Optional when there is no semi-structured feature. Required for PTS. |  |
| FeatureVersion | 0 | String | Version number of algorithm extracting semi-structured face features, for example, "ISFRFR259.2.0”. Range: [0, 20]. Required for PTS. |  |
| Feature | 0 | String | Base64-encoded. Extracted face features. Used to assist the backend server to perform face comparison. Currently 512 bytes before encryption. Required for PTS. |  |
| FeatureManufacturer | 0 | String | Vendor of the algorithm that extracts semi-structured face features. Range: [0, 20]. | "Uniview" |
| CapSrc | C | unsigned long | Data collection source. 1. Face info collected by face recognition terminal. 2. Access card info collected by card reader. 3. ID card info collected by card reader. 4. Info collected by gate. For FaceInfo, choose 1. Required for PTS. |  |

CorrectImageParam

| CorrectImageParam Json Block | { "Enabled":, "FaceWidth":, "PupilDistance":, "MinImageSize":, "MaxImageSize":, "BlackAndWhiteThreshold": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Enabled | M | boolean | Enable or disable 0: Disable 1: Enable | 1 |
| FaceWidth | M | unsigned long | Width of generated face Range: [120,150] Unit: px | 150 |
| PupilDistance | M | unsigned long | Pupillary distance Range: [120,150] Unit: px | 50 |
| MinImageSize | M | unsigned long | Minimum image size Unit: KB | 10 |
| MaxImageSize | M | unsigned long | Maximum image size Unit: KB | 2048 |
| BlackAndWhiteThreshold | M | float | Black and white threshold Range: 0-1 | 0.95 |

MonitorStatusInfo

| MonitorStatusInfo Json Block | { "DevID": , "ChannelID":, "MonitorStatus": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DevID | M | unsigned long | Device ID. | 3 |
| ChannelID | M | unsigned long | Channel ID. | 10 |
| MonitorStatus | M | unsigned long | Monitoring status: 0: Success 1: Failed to create library 2: Failed to import member 3: Monitoring failed | 0 |

MonitorMemberInfo

| MonitorMemberInfo Json Block | { "MemberID": , "Name", "MonitorNum": "MonitorStatusList":[<MonitorStatusInfo>, <MonitorStatusInfo>,….], "MonitorResult": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MemberID | M | unsigned long | Member ID. | 3 |
| Name | M | string | Member name. | - |
| MonitorNum | M | unsigned long | Number of monitored members | 1 |
| MonitorStatusList | C | array | Monitoring status info list. Absent when MonitorNum is 0. | - |
| MonitorStatusInfo | M | Json Block | Monitoring status info. See MonitorStatusInfo Json Block | - |
| MonitorStatusList | C | array | Monitoring status info list. Absent when MonitorNum is 0. | - |
| MonitorStatusInfo | M | Json Block | Monitoring status info. See MonitorStatusInfo Json Block. | - |
| MonitorResult | M | unsigned long | Total monitoring result: 0: Monitoring succeeded 1: Monitoring failed Note: The general monitoring result is “failed” as long as one monitoring result is “failed”. | 0 |

MonitorResultInfo

| MonitorResultInfo Json Block | { "MemberID":, "Name":, "Num":, "ResultList":[<ResultInfo>,….] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MemberID | M | unsigned long | Member ID | 1 |
| Name | M | string | Member name. Length range: [1,63]. | "" |
| Num | M | unsigned long | Number of monitoring locations. |  |
| ResultList | C | array | Result info list. Optional when Num is 0. |  |
| ResultInfo | M | Json Block | Monitoring result info. See ResultInfo Json Block |  |

ResultInfo

| ResultInfo Json Block | { "DevID":, "ChlID":, "ResultCode" } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| DevID | M | unsigned long | Device ID | 1 |
| ChlID | C | unsigned long | Channel ID. Required when a channel is used for monitoring. | 2 |
| ResultCode | M | unsigned long | Cause of failed monitoring: 1: General execution failure. 1000: Algorithm initialization failed. 1001: Face detection failed. 1002: No face in image. 1003: Failed to decode JPEG image. 1004: Low image quality. 1005: Failed to zoom image. 1006: Smart function disabled. 1007: Image is too small. 1008: Image is too large. 1009: Image exceeds 1920*1080. 1010: Image does not exist. 1011: Number of face elements reached upper limit. 1012: Smart bar version does not match the device. 1013: Invalid ID number for imported library member. 1014: Invalid image format of imported library member. 1015: The number of monitoring tasks has reached the upper limit. 1016: Face library is being operated by another client 1017: Face library file is being updated. 1018: JSON deserialization failed. 1019: Base64 decode failed. 1020: The size of encoded face image does not match the received. 1021: Each monitoring task can only use monitoring devices that are connected via the same image access protocol. 1022: Face libraries on the device reached the upper limit 1023: Smart bar is not in position 1024: Device is busy 1025: VIID is not connected | 1000 |

LibraryList

| LibraryList Json Block | { "Num":, "LibList":[<LibInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of libraries in the device Range: [0, 16] | 3 |
| LibList | M | Array | Library list info | - |
| LibInfo | C | JsonBlock | This field is not returned when Num is 0. Detailed library info. See LibInfo Json Block. | -- |

LibInfo

| LibInfo Json Block | { "ID":, "Name":, "PersonNum":, "FaceNum":, "MemberNum":, "LastChange": , "BelongIndex": "IsMonitored":， "IsMonitored": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | C | unsigned long | Library ID. Optional when adding a library. Required when setting a library. | 3 |
| Name | M | string | Library name Range: [1,63] | "清沁花园居民" |
| PersonNum | O | unsigned long | Total number of persons (different PersonIDs) in the library. Returned when using the GET method. | 1 |
| FaceNum | C | unsigned long | Total number of face photos in the library. Returned when using the GET method. | 10 |
| MemberNum | C | unsigned long | Total number of person members (different FaceIDs) in the library. Note: If any person has more than one image in the library, this field is the total number of images in the library. A person without any image in the library is still counted as one member. Returned when using the GET method. | 10 |
| LastChange | M | unsigned long | Last time library info was modified (UNIX timestamp) Returned when using the GET method. | 1510811178 |
| BelongIndex | M | string | Unique index that identifies the device that the library belongs to. Identified by MD5-encrypted NVR serial number. Required when involving libraries of a channel. Required when using the GET method. |  |
| IsMonitored | C | Boolean | Whether monitoring is enabled. Required when using the GET method. Applicable to VMS only. 0: No 1: Yes | 0 |

PeopleLibrariesCapacityInfo

| PeopleLibrariesCapacityInfo Json Block | { "MaxPeopleNum":, "FreePeopleNum":, "MaxFaceNum":, "FreeFaceNum":, "MaxCardNum":, "FreeCardNum":, "MaxLibNum":, "FreeLibNum": "Num":, "CapacityInfoList":[<CapacityInfo>,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| MaxPeopleNum | M | unsigned long | Total library capacity. [Unit: person]. The sum of individual libraries must not be greater than the total library capacity. Only for display. Unmodifiable. | 100 |
| FreePeopleNum | M | unsigned long | Remaining capacity: [unit: Person]. Not applicable to the PUT method. | 80 |
| MaxFaceNum | M | unsigned long | Maximum library capacity: [Unit: Person]. Applicable to PTS | 100 |
| FreeFaceNum | 0 | unsigned long | Remaining face space. [Unit: person]. Applicable to PTS Not applicable to the PUT method. | 80 |
| MaxCardNum | M | unsigned long | Maximum card capacity: [Unit: card]. Applicable to PTS | 100 |
| FreeCardNum | 0 | unsigned long | Remaining card capacity. [Unit: card]. Applicable to PTS. Not applicable to the PUT method. | 80 |
| MaxLibNum | M | unsigned long | Maximum number of libraries allowed. | 5 |
| FreeLibNum | M | unsigned long | Remaining number of libraries allowed. Not applicable to the PUT method. | 5 |
| Num | C | unsigned long | Number of existing libraries. Range: [0,16] Applicable to IPC. | 5 |
| CapacityInfoList | C | array | Single library capacity info list Optional when Num is 0 Applicable to IPC. | -- |
| CapacityInfo | M | JsonBlck | Library capacity info. Applicable to IPC. | -- |

CapacityInfo

| CapacityInfo Json Block | { "LibID":, "Capacity": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LibID | M | unsigned long | Person library ID | 3 |
| Capacity | M | unsigned long | Library capacity info. [Unit: person] | 5 |

PersonResultList

| PersonResultList Json Block | { "Num": , "PersonList":[{ “ReqSeq”:, "PersonID", "FaceNum", "FaceList":[{ "FaceID", "ResultCode" }, ...] }, ...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ReqSeq | C | unsigned long | Sequence number of request, which will be returned with results. Applicable to VMS only. | 1 |
| Num | M | unsigned long | Number of persons. Range: [0,6] | 3 |
| PersonList | M | Array | Person list | -- |
| PersonID | M | unsigned long | Person ID | 2 |
| FaceNum | M | unsigned long | Number of faces Range: [0, 6]. | 2 |
| FaceList | M | Array | Face list | -- |
| FaceID | C | unsigned long | Face ID Optional when FaceNum is 0. | 22 |
| ResultCode | C | unsigned long | Status code for processing result. 0: Success. 1: General execution failure. 1000: Algorithm initialization failed. 1001: Face detection failed. 1002: No face in image. 1003: Failed to decode JPEG image. 1004: Low image quality. 1005: Failed to zoom image. 1006: Smart function disabled. 1007: Image is too small. 1008: Image is too large. 1009: Image exceeds 1920*1080. 1010: Image does not exist. 1011: Number of face elements reached upper limit. 1012: Smart bar version does not match the device. 1013: Invalid ID number for imported library member. 1014: Invalid image format of imported library member. Optional when FaceNum is 0. | 0 |

BatchDeleteResultInfo

| BatchDeleteResultInfo Json Block | { "Num": "Result":[{"ID":, "ResultCode":}, …….] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of batch operation results returned | 1 |
| Result | C | array | Result info list. Optional when Num is 0. | - |
| ID | M | unsigned long | Resource ID | 1 |
| ResultCode | M | unsigned long | Returned error code | 100 |

Event

Event

/LAPI/V1.0/System/Event/Subscription

| URL | /LAPI/V1.0/System/Event/Subscription |
| --- | --- |
| Description | Alarm subscription. |
| Method | POST |
| Input Data | { "AddressType": , "IPAddress": , "Port": , "Duration": , "SubscribePersonCondition":<SubscribePersonCondition>, |
| Success Return Data | { "ID":, "Reference":, "CurrentTime":, "TerminationTime": } |
| Note | Applicable to IPC, NVR, and VMS. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AddressType | M | unsigned long | IP address type: 0: IPv4 1: IPv6 2: Domain name 3: IPv4 and IPv6 Currently only IPv4 is supported. | 0 |
| IPAddress | M | string | Length range: [0,256]. |  |
| Port | M | unsigned long | Port. Range: [1, 65535]. |  |
| Duration | M | unsigned long | Subscription period. Unit: s. Range: [30, 3600]. |  |
| SubscribePersonCondition | C | Json Block | Conditions for person data subscription. See SubscribePersonCondition Json Block. |  |
| ID | M | unsigned long | Same as the last segment of the URL of the Reference parameter and is used to identify a subscription. Required when updating or deleting a subscription. | 0 |
| Reference | M | string | Descriptive information of the subscriber, in the form of a URL. | "192.168.0.13:80/Subscription/Subscribers/0" |
| CurrentTime | M | unsigned long | UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1477104900 |
| TerminationTime | M | unsigned long | UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1477104949 |

/LAPI/V1.0/System/Event/Subscription/<ID>

| URL | /LAPI/V1.0/System/Event/Subscription/<ID> |
| --- | --- |
| Description | Update subscription. |
| Method | PUT |
| Input Data | { "Duration": } |
| Success Return Data | { "Reference":, "CurrrntTime":, "TerminationTime": } |
| Note |  |
| Note | Applicable to IPC, NVR, and VMS. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Subscription ID, assigned by the device. If subscription is successful, the ID will be returned to the subscriber as a return value. Required when updating or cancelling alarm subscription. | 0 |
| Duration | M | unsigned long | The seconds from subscriptionstart(POST successed) to finish. The subscription server will be terminated when timeout. IPC will nerver push message to the subscriber unless subscribe again. If termination time is not reached, subscriber can renew(PUT) subscrtption. Subscription period. Unit: s. Range: [30, 3600]. | 300 |
| Reference | C | string | Descriptive information of the subscriber, in the form of a URL. | "192.168.0.13:80/Subscription/Subscribers/1" |
| CurrrntTime | M | unsigned long | Subscription start time UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1477104900 |
| TerminationTime | M | unsigned long | Subscription finish time UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1477104949 |

/LAPI/V1.0/System/Event/Subscription/<ID>

| URL | /LAPI/V1.0/System/Event/Subscription/<ID> |
| --- | --- |
| Description | Update subscription. |
| Description | Cancel subscription |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Subscription ID, assigned by the device. If subscription is successful, the ID will be returned to the subscriber as a return value. Required when updating or cancelling alarm subscription. | 0 |

/LAPI/V1.0/System/Event/Notification/Alarm

| URL | /LAPI/V1.0/System/Event/Notification/Alarm |
| --- | --- |
| Description | Push alarms. |
| Method | POST |
| Input Data | { "Reference":, "AlarmInfo":<AlarmInfo>, "RelatedObjects":<RelatedObjectInfo> } |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, VMS, and VM. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmInfo | M | Json Block | Alarm push message. See AlarmInfo Json Block. | - |
| RelatedObjects | C | Json Block | Alarm-related object info. Required when there are alarm-related object data to push. See RelatedObjectInfo | - |

/LAPI/V1.0/System/Event/Notification/Structure

| URL | /LAPI/V1.0/System/Event/Notification/Structure |
| --- | --- |
| Description | Push structured events. |
| Method | POST |
| Input Data | { "Reference":, "TimeStamp":, "Seq":, "SrcID":, "SrcName":, "RelatedID":, "StructureInfo":<StructureDataInfo> } |
| Success Return Data | None |
| Note |  |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| TimeStamp | M | unsigned long | Alarm time. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Alarm sequence number Applicable to VMS and IPC. | 327 |
| SrcID | M | unsigned long | Alarm source ID. | 1 |
| SrcName | M | String | Alarm source name. Length: [1,63] | Channel1 |
| RelatedID | C | string | ID that relates alarm with alarm data or data of objects from other channels. Unique in each camera. 15 characters. Required when there is related alarm or related object from other channels. | "5ED9FE4C0000001" |
| StructureDatatInfo | M | Json Block | Structured data info. See StructureDataInfo Json Block | - |

/LAPI/V1.0/System/Event/Notification/PersonInfo

| URL | /LAPI/V1.0/System/Event/Notification/PersonInfo |
| --- | --- |
| Description | Push person related events. |
| Method | POST |
| Input Data | { "Reference":, "PersonEventInfo":<PersonEventInfo> } |
| Success Return Data | None |
| Note | Applicable to IPC, NVR, and VMS. Use this interface to push person related events when the subscription type in /LAPI/V1.0/System/Event/Subscription is “face recognition”. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| PersonEventInfo | C | Json Block | Person event info. See PersonEventInfo Json Block | - |

/LAPI/V1.0/System/Event/Notification/PeopleCount/LineRuleData

| URL | /LAPI/V1.0/System/Event/Notification/PeopleCount/LineRuleData |
| --- | --- |
| Description | Push rules data of tripwire people counting. |
| Method | POST |
| Input Data | { "Reference": , "TimeStamp": , "Seq": , "ChannelID": , "LineNum":, "LineRuleDataList": [ <LineRuleDataInfo>,…] } |
| Success Return Data | None |
| Note | Applicable to IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | C | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| TimeStamp | M | unsigned long | Alarm notification time. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Sequence number of data to push. Applicable to IPC. | 327 |
| ChannelID | M | unsigned long | Channel ID, starts from 0. | 1 |
| LineNum | M | unsigned long | Number of tripwires, starts from 0. 0 means no tripwire. | 1 |
| LineRuleDataList | C | Json Block | People counting statistics collected based on a tripwire. Required when LineNum is not 0. See LineRuleDataInfo Json Block | - |

/LAPI/V1.0/System/Event/Notification/PeopleCount/AreaRuleData

| URL | /LAPI/V1.0/System/Event/Notification/PeopleCount/AreaRuleData |
| --- | --- |
| Description | Push rules data of area people counting. |
| Method | POST |
| Input Data | { "Reference": , "TimeStamp": , "Seq": , "ChannelID": , "AreaNum":, "AreaRuleDataList": [<AreaRuleDataInfo>,…] } |
| Success Return Data | None |
| Note | Applicable to IPC. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | C | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| TimeStamp | M | unsigned long | Alarm notification time. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Sequence number of data to push. Applicable to IPC. | 327 |
| ChannelID | M | unsigned long | Channel ID, starts from 0. | 1 |
| AreaNum | M | unsigned long | Number of areas, starts from 0. 0 means no area. | 1 |
| AreaRuleDataList | C | Json Block | People counting statistics collected based on a detection area. Required when AreaNum is not 0. See AreaRuleDataInfo Json Block |  |

/LAPI/V1.0/System/Event/Notification/FastMovingDetection

| URL | /LAPI/V1.0/System/Event/Notification/FastMovingDetection |
| --- | --- |
| Description | Push quick moving detection data. |
| Method | POST |
| Input Data | { "Reference":, "AlarmType":, "TimeStamp":, "Seq":, "SourceID":, "SourceName":, "AlarmPicture":<AlarmPicture> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type | "FastMoving" |
| TimeStamp | M | unsigned long | Time when the pushed data are generated. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Message ID. Applicable to VMS and IPC. | 327 |
| SourceID | C | unsigned long | Alarm source ID. Applicable to VMS and IPC. | 1 |
| SourceName | C | string | Alarm source name. Length: [1,63] | Channel1 |
| AlarmPicture | M | Json Block | Image data carried in the alarm. See AlarmPicture JsonBlock | - |

/LAPI/V1.0/System/Event/Notification/HatDetection

| URL | /LAPI/V1.0/System/Event/Notification/HatDetection |
| --- | --- |
| Description | Push helmet and chef hat detection data. |
| Method | POST |
| Input Data | { "Reference":, "AlarmType":, "TimeStamp":, "Seq":, "SourceID":, "SourceName":, "AlarmPicture":<AlarmPicture> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "SafetyHelmetAlarm" "ChefHatAlarm" |
| TimeStamp | M | unsigned long | Time when the pushed data are generated. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Message ID. Applicable to VMS and IPC. | 327 |
| SourceID | C | unsigned long | Alarm source ID. Applicable to VMS and IPC. | 1 |
| SourceName | C | string | Alarm source name. Length: [1,63] | Channel1 |
| AlarmPicture | M | Json Block | Image data carried in the alarm. See AlarmPicture JsonBlock | - |

/LAPI/V1.0/System/Event/Notification/WorkClothesDetection

| URL | /LAPI/V1.0/System/Event/Notification/WorkClothesDetection |
| --- | --- |
| Description | Push work clothes detection data. |
| Method | POST |
| Input Data | { "Reference":, "AlarmType":, "TimeStamp":, "Seq":, "SourceID":, "SourceName":, "AlarmPicture":<AlarmPicture> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "WorkClothesAlarm" |
| TimeStamp | M | unsigned long | Time when the pushed data are generated. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Message ID. Applicable to VMS and IPC. | 327 |
| SourceID | C | unsigned long | Alarm source ID. Applicable to VMS and IPC. | 1 |
| SourceName | C | string | Alarm source name. Length: [1,63] | Channel1 |
| AlarmPicture | M | Json Block | Image data carried in the alarm. See AlarmPicture Json Block | - |

/LAPI/V1.0/System/Event/Notification/TelephoningDetection

| URL | /LAPI/V1.0/System/Event/Notification/TelephoningDetection |
| --- | --- |
| Description | Push calling detection data. |
| Method | POST |
| Input Data | { "Reference":, "AlarmType":, "TimeStamp":, "Seq":, "SourceID":, "SourceName":, "AlarmPicture":<AlarmPicture> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "TelephoningAlarm" |
| TimeStamp | M | unsigned long | Alarm notification time. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Message ID. Applicable to VMS and IPC. | 327 |
| SourceID | C | unsigned long | Alarm source ID. Applicable to VMS and IPC. | 1 |
| SourceName | C | string | Alarm source name. Length: [1,63] | Channel1 |
| AlarmPicture | M | Json Block | Image data carried in the alarm. See AlarmPicture Json Block | - |

/LAPI/V1.0/System/Event/Notification/SmokingDetection

| URL | /LAPI/V1.0/System/Event/Notification/SmokingDetection |
| --- | --- |
| Description | Push smoking detection data. |
| Method | POST |
| Input Data | { "Reference":, "AlarmType":, "TimeStamp":, "Seq":, "SourceID":, "SourceName":, "AlarmPicture":<AlarmPicture> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Reference | M | string | URL used by the client to locate the alarm notification. <SubscribersID>: Used to differentiate different subscribing clients that a user is logged in to with the same IP and port. | "192.168.0.13:80/Subscription/Subscribers/1" |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "SmokingAlarm" |
| TimeStamp | M | unsigned long | Time when the pushed data are generated. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 1489040894 |
| Seq | C | unsigned long | Message ID. Applicable to VMS and IPC. | 327 |
| SourceID | C | unsigned long | Alarm source ID. Applicable to VMS and IPC. | 1 |
| SourceName | C | string | Alarm source name. Length: [1,63] | Channel1 |
| AlarmPicture | M | Json Block | Image data carried in the alarm. See AlarmPicture Json Block | - |

Json Block

AlarmInfo

| AlarmInfo Json Block | { "AlarmType":, "AlarmLevel":, "TimeStamp":, "AlarmSeq":, "AlarmSrcID":, "AlarmSrcName":, "RelatedID": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AlarmType | M | string | Alarm type. See Alarm&Operate Log Type. | "MotionAlarmOn" |
| AlarmLevel | C | unsigned long | Alarm level 0: Level 1 1: Level 2 2: Level 3 3: Level 4 4: Level 5 Level 1 is the most serious, and the severity decreases from level 1 to level 5. Applicable to VM, NVR, and VMS. | 0 |
| TimeStamp | M | unsigned long | Alarm time | 1489040894 |
| AlarmSeq | C | unsigned long | Alarm sequence number Applicable to VM, NVR, VMS, and IPC. | 327 |
| AlarmSrcID | C | unsigned long | Resource ID Applicable to VM, NVR, and VMS. | 860 |
| AlarmSrcName | C | string | Alarm source name String length range: [0, 128]. Applicable to VMS. | "206.9.252.7_V_6" |
| RelatedID | C | string | ID that relates an alarm with alarm data. Unique in each camera. 15 characters. Required when there are alarm-related data. | "5ED9FE4C0000001" |

AlarmPicture

| AlarmPicture Json Block | { "ImageNum", "ImageList":[<ImageInfo>,<ImageInfo>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ImageNum | M | unsigned long | Number of images. | 2 |
| ImageList | C | array | Image data list. See ImageInfo Json Block. Required when ImageNum is not 0. | - |

AreaRuleDataInfo

| AreaRuleDataInfo Json Block | { "AreaID": , "ObjectNum": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| AreaID | M | unsigned long | Detection area ID, starts from 0. | 3 |
| ObjectNum | M | unsigned long | Number of people present in the area | 20 |

FaceAttr

| Faceattr Json Block | { "Gender":, "AgeRange":, "GlassFlag":, "Temperature": } |
| --- | --- |

| Param | Requirement | Type | Description |
| --- | --- | --- | --- |
| Gender | C | unsigned long | Gender. Optional when detection is disabled. 0: Unknown 1: Male 2: Female |
| AgeRange | C | unsigned long | Age range. Optional when detection is disabled. 0: Unknown 1: Child 2: Juvenile 3: Youth 4: 4: Middle-aged 5: Senior |
| GlassFlag | C | unsigned long | Whether glasses are detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| MaskFlag | C | unsigned long | Whether mask is detected. Optional when detection is disabled. 0: Unknown 1: No 2: Yes |
| MaskColor | C | unsigned long | Mask color. Optional when detection is disabled. See definitions of person info elements. |
| Temperature | C | float | Body temperature. Optional when detection is disabled. Unit: Celsius degree Precision: 2 decimal places |

FaceInfo

| FaceInfo Json Block | { "FaceID":, "FaceDoforPersonID":, "FaceDoforNonMotorVehicleID":, "FaceDoforVehicleID":, "Position":, "AppearTime":, "DisAppearTime":, "Confidence":, "SmallPicAttachIndex":, "LargePicAttachIndex":, "FeaturVersion":, "Feature":, "AttributeInfo":<FaceAttr>, "RuleInfo":<RuleInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FaceID | M | unsigned long | Face ID, starts from 1. | 1 |
| FaceDoforPersonID | C | unsigned long | Corresponding person ID, which starts from 1. 0 is invalid. Optional when the face is not linked to any person. | 1 |
| FaceDoforNonMotorVehicleID | C | unsigned long | Corresponding non-motor vehicle ID, which starts from 1. 0 is invalid. Required when the face is linked to a non-motor vehicle. | 1 |
| FaceDoforVehicleID | C | unsigned long | Corresponding motor vehicle ID, which starts from 1. 0 is invalid. Required when the face is linked to a motor vehicle. | 1 |
| Position | M | string | Position of the face area. Coordinates: 0-10000. The top left and bottom right points of the rectangle: "138,315;282,684" Comma (,) and semicolon (;) are half-width characters. Range: [0,32] | "138,315;282,684" |
| AppearTime | C | string | Time when face appears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM. The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18. | "20190422083030789" |
| DisAppearTime | C | string | Time when face disappears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM. The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | "20190422083030789" |
| Confidence | C | unsigned long | Confidence of face detection. Optional when detection is disabled. Value range: [0-100] | 70 |
| SmallPicAttachIndex | C | unsigned long | Index of the close-up image of face in the image list, which starts from 1. 0 is invalid. Optional when no close-up image is pushed. | 0 |
| LargePicAttachIndex | C | unsigned long | Index of the original image of the face in the image list, which starts from 1. 0 is invalid. Optional when no original image is pushed. | 1 |
| FeatureVersion | C | string | Version number of algorithm extracting semi-structured features. Range: [0,32] | "ISFRFR259.2.0" |
| Feature | C | string | Base64-encoded. Extracted face features. The maximum length before encoding is 4096 bytes. |  |
| AttributeInfo | C | Json Block | Face attribute info. Optional when attribute detection is disabled. See FaceAttr Json Block. | - |
| RuleInfo | 0 | Json Block | Rule info. See RuleInfo Json Block | - |

FacePassRecordInfo

| FacePassRecordInfo Json Block | { "RecordID": , "PassingTime": , "ChannelName": "CompareInfo": <CompareInfo>, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| CompareInfo | M | Json Block | Comparison info. See CompareInfo json block. | - |

FileInfo

| FileInfo Json Block | { "Name":, "Size":, "Data":, "URL": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Name | 0 | String | File name. Range: [1, 16]. Used to push images in real-time or historical records |  |
| Size | 0 | unsigned long | File size (unit: byte). Range: [0, 1M] Used to push images in real-time or historical records | 21783 |
| Data | 0 | String | Base64-encoded file data. Used to push images in real-time or historical records |  |
| URL | 0 | String | Image URL. Used to push image info when getting historical records | photo/20190729/1114/ 20190719181741268_2_56_0_0_unknown.jpg |

ImageInfo

| ImageInfoJson Block | { "Index":, "Type":, "Format":, "Width":, "Height":, "CaptureTime":, "Size":, "Data": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Index | M | unsigned long | Image index, starts from 0. Index used to link original (large) image and close-up (small) images of objects in ObjectInfo when pushing structured data. | 0 |
| Type | M | unsigned long | Image type. See the ImageType of the extended members. |  |
| Format | M | unsigned long | Image format. 0: JPEG 1: BMP 2: PNG 3: GIF 4: TIFF | 1 |
| Width | M | unsigned long | Image width. | 1920 |
| Height | M | unsigned long | Image height. | 1080 |
| CaptureTime | M | unsigned long | Time when image was captured. UTC time, number of seconds since 0’o clock, Jan. 1, 1970. | 21090103121759999 |
| Size | C | unsigned long | Length of Base64-encoded image. Unit: Byte. Max. 3M. Required when the pushed is Base64-encoded data. | 128000 |
| Data | C | String | Base64-encoded image data. Required when the pushed is Base64-encoded data. | - |
| URL | C | String | Image URL. Use the URL to get the image. Length range: [0,256]. Required when the pushed is an image URL. | /LAPI/V1.0/System/Picture?Type=1&Index=ubs_5_0_0_0_0_0_5278_41_8719_1545200097&Size=80564 |

LineRuleDataInfo

| LineRuleDataInfo Json Block | { "LineID": , "BeginPassTime": , "EndPassTime": , "ObjectIn": , "ObjectOut": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LineID | M | unsigned long | Tripwire ID, starts from 0. | 3 |
| BeginPassTime | M | string | Detection start time: YYYYMMDDHHMMSS, in 24H format. Length range: [0,18]. | "20190919121212" |
| EndPassTime | M | string | Detection end time: YYYYMMDDHHMMSS, in 24H format. | "20190919121212" |
| ObjectIn | M | unsigned long | Number of people that have entered within the set time. | 10 |
| ObjectOut | M | unsigned long | Number of people that have left within the set time. | 15 |

NonMotorVehicleInfo

| NonMotorVehicleInfo Json Block | { "ID ":, "Position":, "SmallPicAttachIndex":, "LargePicAttachIndex":, "PlatePicAttachIndex":, "AppearTime": , "DisAppearTime": , "Confidence": , "Speed": , "DirectionType": , "AttributeInfo":<NonMotorVehicleAttr> "PlateAttributeInfo":<PlateAttr>, "PersonOnNoVehicleNum":, "PersonOnNoVehicleInfoList":[<PersonAttr> ,...], "RuleInfo":<RuleInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Non-motor vehicle ID, starts from 1. | 1 |
| Position | M | string | Position of the non-motor vehicle Coordinates: 0-10000. The top left and bottom right points of the rectangle: "138,315;282,684" Comma (,) and semicolon (;) are half-width characters. Range: [0,32]. | "138,315;282,684" |
| SmallPicAttachIndex | C | unsigned long | Index of the close-up image of non-motor vehicle in the image list, which starts from 1. 0 is invalid. Optional when no close-up image is pushed. | 0 |
| LargePicAttachIndex | C | unsigned long | Index of the original image of the non-motor vehicle in the image list, which starts from 1. 0 is invalid. Optional when no original image is pushed. | 1 |
| PlatePicAttachIndex | C | unsigned long | Index of the close-up image of plate in the image list, which starts from 1. Optional when no close-up image of plate is pushed. | 2 |
| AppearTime | C | string | Time when the detection object appears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | "20190422083030789" |
| DisAppearTime | C | string | Time when the detection object disappears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | "20190422083030789" |
| Confidence | C | unsigned long | Confidence of detection. Optional when detection is disabled. Value range: [0-100] | 70 |
| Speed | C | unsigned long | Speed. Optional when detection is disabled. Speed: km/h. | 70 |
| DirectionType | C | string | Driving direction. Optional when detection is disabled. | See Direction |
| FeatureVersion | C | string | Version number of algorithm extracting semi-structured features. Range: [0,32] | "ISFRFR259.2.0" |
| Feature | C | string | Base64-encoded. Extracted feature info of non-motor vehicles. The maximum length before encoding is 4096 bytes. |  |
| AttributeInfo | C | Json Block | Non-motor vehicle attribute info. Optional when detection is disabled. See NonMotorVehicleAttr | 0 |
| PlateAttributeInfo | C | Json Block | Plate attribute info. Optional when detection is disabled. See PlateAttr | - |
| PersonOnNoVehicleNum | M | unsigned long | Total number of persons in vehicle | 0 |
| PersonOnNoVehicleInfoList | C | Json Block | Vehicle occupant info list. Optional when PersonOnNoVehicleNum is 0. See PersonAttr | - |
| RuleInfo | O | Json Block | Rule info. See RuleInfo Json Block | - |

NonMotorVehicleAttr

| NonMotorVehicleAttrJson Block | { "SpeedType":, "ImageDirection", "NonVehicleType":, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SpeedType | C | unsigned long | Speed type of non-motor vehicles in structured scenes. Optional when detection is disabled. 0: Unknown 1: Static 2: Low speed 3: Medium speed 4: Fast | 0 |
| ImageDirection | C | unsigned long | Movement direction (relative to the picture) of non-motor vehicles in structured scenes. Optional when detection is disabled. 0: Unknown 1: Static 2: Up 3: Down 4: Left 5: Right 6: Up left 7: Down left 8: Up right 9: Down right | 0 |
| NonVehicleType | C | unsigned long | Type of non-motor vehicle. Optional when detection is disabled. 0: Unknown 1: Bicycle 2: 3-wheel 3: Motorcycle 4: Electric bicycle 5: 2-wheel (motorcycle, bicycle, electric bicycle) | 0 |

PersonEventInfo

| PersonEventInfo Json Block | { "ID":, "Timestamp":, "NotificationType":, "FaceInfoNum":, "FaceInfoList":[<FacePassRecordInfo> ,...], } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Notification record ID |  |
| Timestamp | M | unsigned long | Notification time (unit: second, UTC format). |  |
| NotificationType | M | unsigned long | Notification type 0: Real-time notification 1: Historical notification |  |
| FaceInfoNum | 0 | unsigned long | Face info number Range: [0, 1]. FaceInfo fields are optional when no face info is collected |  |
| FaceInfoList | 0 | array | Face info list FaceInfo fields are optional when no face info is collected |  |

PersonInfo

| PersonInfo Json Block | { "PersonID":, "Position":, "AppearTime":, "DisAppearTime": , "Confidence":, "SmallPicAttachIndex":, "LargePicAttachIndex":, "FeaturVersion":, "Feature":, "AttributeInfo":<PersonAttr>, "RuleInfo":<RuleInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PersonID | M | unsigned long | Person ID. Starts from 1. Optional when detection is disabled. | 1 |
| Position | M | string | Position of the person Coordinates: 0-10000. The top left and bottom right points of the rectangle: "138,315;282,684" Comma (,) and semicolon (;) are half-width characters. Range: [0,32]. | "138,315;282,684" |
| AppearTime | C | string | Time when the detection object appears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | "20190422083030789" |
| DisAppearTime | C | string | Time when the detection object disappears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | "20190422083030789" |
| Confidence | C | unsigned long | Confidence of face detection. Optional when detection is disabled. Value range: [0-100] | 70 |
| SmallPicAttachIndex | C | unsigned long | Index of the close-up image of person in the image list, which starts from 1. 0 is invalid. Optional when no close-up image is pushed. | 0 |
| LargePicAttachIndex | C | unsigned long | Index of the original image of the person in the image list, which starts from 1. 0 is invalid. Optional when no original image is pushed. | 1 |
| FeatureVersion | C | string | Version number of algorithm extracting semi-structured features. Range: [0,32] | "ISFRFR259.2.0" |
| Feature | C | string | Base64-encoded. Extracted body feature info. The maximum length before encoding is 4096 bytes. |  |
| AttributeInfo | C | Json Block | Human body attribute info. Optional when attribute detection is disabled. See PersonAttr | - |
| RuleInfo | O | Json Block | Rule info. See RuleInfo Json Block | - |

PersonAttr

| PersonAttrJson Block | { "Gender":, "AgeRange":, "SleevesLength":, "CoatColor":, "TrousersLength":, "TrousersColor":, "BodyToward":, "ShoesTubeLength":, "HairLength":, "BagFlag": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Gender | C | unsigned long | Gender. Optional when detection is disabled. 0: Female 1: Male 98: Unknown | 0 |
| AgeRange | C | unsigned long | Age range. Optional when detection is disabled. 0: Child 1: Juvenile 2: Youth 3: Middle-aged 4: Senior 98: Unknown | 0 |
| SleevesLength | C | unsigned long | Sleeve length. Optional when detection is disabled. 0: Unknown/not sure 1: Short sleeves 2: Long sleeves | 0 |
| CoatColor | C | unsigned long | Coat color. Optional when detection is disabled. See extended data type CommonColorType | "A |
| TrousersLength | C | unsigned long | Trousers length. Optional when detection is disabled. 0: Unknown 1: Short 2: Long | 0 |
| TrousersColor | C | string | Trousers color. Optional when detection is disabled. See extended data type CommonColorType | "A" |
| BodyToward | C | unsigned long | Body direction. Optional when detection is disabled. 0: Not sure/Unknown 1: Front 2: Side 3: Back | 0 |
| ShoesTubeLength | C | unsigned long | Shoes tube length. Optional when detection is disabled. 0: Unknown 1: Long tubes 2: Short tubes/ordinary shoes | 0 |
| HairLength | C | unsigned long | Hair length. Optional when detection is disabled. 0: Unknown/not sure 1: Long hair 2: Short hair | 0 |
| BagFlag | C | unsigned long | Whether bag is detected. Optional when detection is disabled. 0: No bag 1: Handbag 2: Backpack 98: Unknown | 0 |

PlateAttr

| PlateAttr Json Block | { "PlateNo":, "Color":, "Type":, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| PlateNo | C | string | Plate number. Optional when detection is disabled. | "A00000" |
| Color | C | unsigned long | Plate color. Optional when detection is disabled. See extended data type: plate color | 1 |
| Type | C | unsigned long | Plate type. Optional when detection is disabled. See extended data type: plate type | 0 |

RegionInfo

| RegionInfo Json Block | { "Nation": , "Province": , "City": , } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Nation | M | string | Nationality. Range: [1,63] | “China” |
| Province | M | string | Province. Range: [1,63]. | “Zhejiang” |
| City | M | string | City. Range: [1,63]. | “Hangzhou” |

RelatedObjectInfo

| RelatedObjectInfo Json Block | { "ObjectNum", "ObjectList":[<ObjectIDInfo>,<ObjectIDInfo>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ObjectNum | M | unsigned long | Number of objects. | 1 |
| ObjectList | C | array | Object list. Required when ObjectNum is not 0. See ObjectIDInfo | - |

RuleInfo

| RuleInfo Json Block | { "RuleType", "TrigerType", "PointNum": , "PointList": [ { "X": , "Y": }, … ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RuleType | M | unsigned long | Rule type: 0: Intrusion detection 1: Cross line detection 2: Leave area 3: Enter area | 0 |
| TrigerType | 0 | unsigned long | Type of rule trigger: 0: None 1: Bi-directional 2: Clockwise 3: Counterclockwise 4: Enter 5: Leave | 0 |
| PointNum | M | unsigned long | Number of points (vertices) | 6 |
| PointList | C | array | Coordinates of vertices of the detection area. Optional when PointNum is 0. |  |
| X | M | unsigned long | x-coordinate. Range: [0,10000] | 5000 |
| Y | M | unsigned long | y-coordinate. Range: [0,10000] | 5000 |

ObjectInfo

| ObjectInfo Json Block | { "FaceNum":, "FaceInfoList"[<FaceInfo> ,...], "PersonNum":, "PersonInfoList":[<PersonInfo> ,...], "NonMotorVehicleNum":, "NonMotorVehicleInfoList":,:[<NonMotorVehicleInfo> ,...] "VehicleNum":, "VehicleInfoList":[<VehicleInfo> ,...] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| FaceNum | M | unsigned long | Number of faces. | 1 |
| FaceInfoList | C | Json Block | Face info list. See FaceInfo Json Block; Optional if FaceNum is 0. | - |
| PersonNum | M | unsigned long | Number of persons. | 1 |
| PersonInfoList | C | Json Block | Person info list. See PersonInfo Json Block. Optional when PersonNum is 0. | - |
| NonMotorVehicleNum | C | unsigned long | Number of non-motor vehicles. Optional when the subscribed service type is face or body. | 1 |
| NonMotorVehicleInfoList | C | Json Block | Non-motor vehicle info list. Optional when NonMotorVehicleNum is 0 or when the subscribed service type is face or body. See NonMotorVehicleInfo Json Block | - |
| VehicleNum | C | unsigned long | Number of motor vehicles. Optional when the subscribed service type is face or body. | 1 |
| VehicleInfoList | C | Json Block | Motor vehicle info list. Optional when VehicleNum is 0 or when the subscribed service type is face or body. See VehicleInfo Json Block | - |

ObjectIDInfo

| ObjectIDInfo Json Block | { "ObjectType", "ObjectID" } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ObjectType | M | unsigned long | Object type: 0: Unknown/unrecognized object type 1: Face 2: Human body 3: Motor vehicle 4: Non-motor vehicle | 1 |
| ObjectID | M | unsigned long | Object ID | 2 |

SubscribePersonCondition

| SubscribePersonCondition Json Block | { "LibIDNum":, "LibIDList":[{"LibID":},{"LIbID":},…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LibIDNum | M | unsigned long | Number of IDs of subscribed libraries. LibIDNum and LibIDList are optional when subscription type is 0. When the subscription type is 1, it is required to specify the number and value of library IDs to subscribe from. When LibIDNum is 0xFFFF, it means subscription from all libraries. | 0 |
| LibIDList | M | Array | List of IDs of subscribed libraries. | -- |
| LibID | C | unsigned long | Optional when LibIDNum is 0. | 3 |

StructureDataInfo

| StructureInfo Json Block | { "ObjInfo":<ObjectInfo>, "ImageNum":, "ImageInfoList":,:[<ImageInfo> ,...], "FinishFaceNum": , "FinishFaceList" [<FaceID>,<FaceID>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ObjInfo | C | Json Block | Object info list. Optional when ObjNum is 0. See ObjectInfo | - |
| ImageNum | M | unsigned long | Number of images | 1 |
| ImageInfoList | C | Json Block | Image info list. Optional when ImageNum is 0. See ImageInfo | - |
| FinishFaceNum | 0 | unsigned long | Number of faces that have finished detection Range: [0,40] Absent when no face has finished detection. |  |
| FinishFaceList | 0 | array | List of faces that have finished detection. Absent when FinishFaceNum is 0. |  |

VehicleAttr

| VehicleAttr Json Block | { "Type":, "Color":, "SpeedType", "VehicleBrand":, "AimStatus": , "DriverSunVisorStatus": , "CodriverSunVisorStatus": , "DriverSeatBeltStatus": , "CodriverSeatBeltStatus": , "DriverMobileStatus": , "PendantStatus": , "ImageDirection":, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Vehicle type. See extended data type: vehicle type | 0 |
| Color | M | unsigned long | Vehicle body color. Optional when detection is disabled. See extended data type CommonColorType | 0 |
| SpeedType | C | unsigned long | Speed type of motor vehicles in structured scenes. Optional when detection is disabled. 0: Unknown 1: Static 2: Slow 3: Medium speed 4: Fast |  |
| VehicleBrand | C | string | Vehicle brand ID (custom). Optional when detection is disabled. See extended data type: vehicle brand |  |
| AimStatus | C | String | Status of inspection sticker. Optional when detection is disabled. 0: Not in use 1: In use 2: Unknown Range: [0-2] | "1" |
| DriverSunVisorStatus | C | String | Status of driver-side sun visor. Optional when detection is disabled. 0-Not in use 1-In use 2-Unknown Range: [0-2] | "0" |
| CodriverSunVisorStatus | C | String | Status of passenger-side sun visor. Optional when detection is disabled. 0-Not in use 1-In use 2-Unknown Range: [0-2] | "0" |
| DriverSeatBeltStatus | C | String | Status of driver-side seat belt. Optional when detection is disabled. 0: Not in use 1: In use 2: Unknown Range: [0-2] | "0" |
| CodriverSeatBeltStatus | C | String | Status of passenger-side seat belt. Optional when detection is disabled. 0: Not in use 1: In use 2: Unknown Range: [0-2] | "0" |
| DriverMobileStatus | C | String | Whether driver is calling while driving. Optional when detection is disabled. 0: No 1: Yes 2: Unknown Range: [0-2] | "0" |
| PendantStatus | C | String | Pendant. Optional when detection is disabled. 0: Not detected 1: Detected 2: Unknown Range: [0-2] | "0" |
| ImageDirection | C | unsigned long | Driving direction (relative to the image) of motor vehicle in structured scenes. Optional when detection is disabled. Same value as ImageDirection of non-motor vehicle |  |

VehicleInfo

| VehicleInfo Json Block | { "ID ":, "Position":, "SmallPicAttachIndex":, "LargePicAttachIndex":, "PlatePicAttachIndex":, "AppearTime": , "DisAppearTime": , "Confidence": , "FeaturVersion":, "Feature":, "VehicleAttributeInfo":<VehicleAttr> "PlateAttributeInfo":<PlateAttr>, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Vehicle ID, starts from 1. | 1 |
| Position | M | string | Position of the vehicle Coordinates: 0-10000. The top left and bottom right points of the rectangle: "138,315;282,684" Comma (,) and semicolon (;) are half-width characters. Range: [0,32]. | 138,315;282,684 |
| SmallPicAttachIndex | C | unsigned long | Index of the close-up image of vehicle in the image list, which starts from 1. 0 is invalid. Optional when no close-up image is pushed. | 0 |
| LargePicAttachIndex | C | unsigned long | Index of the original image of the vehicle in the image list, which starts from 1. 0 is invalid. Optional when no original image is pushed. | 1 |
| PlatePicAttachindex | C | unsigned long | Index of the close-up image of plate in the image list, which starts from 1. 0 is invalid. Optional when no close-up image is pushed. | 1 |
| AppearTime | C | string | Time when the detection object appears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | AppearTime |
| DisAppearTime | C | string | Time when the detection object disappears. Optional when detection is disabled. Format: YYYYMMDDHHMMSSMMM, The first MM represents month, the second MM represents minute, and the third MMM represents millisecond. Fixed length: 18 | DisAppearTime |
| Confidence | C | unsigned long | Confidence of detection Value range: [0-100] | Confidence |
| FeatureVersion | C | string | Version number of algorithm extracting semi-structured features. Range: [0,32]. | "ISFRFR259.2.0" |
| Feature | C | string | Base64-encoded. Extracted motor vehicle feature info. The maximum length before encoding is 4096 bytes. |  |
| VehicleAttributeInfo | C | Json Block | Vehicle attribute info. Optional when attribute detection is disabled. See VehicleAttr | - |
| PlateAttributeInfo | C | Json Block | Plate attribute info. Optional when face attribute detection is disabled. See PlateAttr | - |

SearchByImage

SearchByImage

/LAPI/V1.0/Smart/SearchByImage/Task

| URL | /LAPI/V1.0/Smart/SearchByImage/Task |
| --- | --- |
| Description | Create a search-by-image task |
| Method | POST |
| Input Data | None |
| Success Return Data | { "TaskID": } |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TaskID | M | unsigned long | Task ID | 854 |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Control

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Control |
| --- | --- |
| Description | Control a search-by-image task |
| Method | PUT |
| Input Data | Control |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Control | M | Json Block | Operation action |  |
| ID | M | unsigned long | Task ID |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Target

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/Target |
| --- | --- |
| Description | Add search parameters |
| Method | POST |
| Input Data | { "SearchTarget":<SearchTarget>, "CompareTarget":<CompareTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| SearchTarget | M | Json Block | Search target. See SearchTarget Json Block. |  |
| CompareTarget | M | Json Block | Comparison target. See CompareTarget Json Block |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget |
| --- | --- |
| Description | Add search target |
| Method | POST |
| Input Data | { "SearchTarget":<SearchTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| SearchTarget | M | Json Block | Search target. See SearchTarget Json Block. |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/CompareTarget

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/CompareTarget |
| --- | --- |
| Description | Add object search range. |
| Method | POST |
| Input Data | { "CompareTarget":<CompareTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| CompareTarget | M | Json Block | Comparison target. See CompareTarget Json Block |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget |
| --- | --- |
| Description | Add search target |
| Method | POST |
| Input Data | { "SearchTarget":<SearchTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| SearchTarget | M | Json Block | Search target. See SearchTarget Json Block. |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/CompareTarget

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/CompareTarget |
| --- | --- |
| Description | Update target search range. |
| Method | PUT |
| Input Data | { "CompareTarget":<CompareTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| CompareTarget | M | Json Block | Search target. See CompareTarget Json Block |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID>/SearchTarget |
| --- | --- |
| Description | Update search target |
| Method | POST |
| Input Data | { "SearchTarget":<SearchTarget> } |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID |  |
| SearchTarget | M | Json Block | Search target. See SearchTarget Json Block. |  |

/LAPI/V1.0/Smart/SearchByImage/Progress?TaskID=<ID>

| URL | /LAPI/V1.0/Smart/SearchByImage/Progress?TaskID=<ID> |
| --- | --- |
| Description | Get the search progress of a task. |
| Method | GET |
| Input Data | None |
| Success Return Data | SearchProgress |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID | 1 |
| SearchProgress | M | Json Block | Search progress |  |

/LAPI/V1.0/Smart/SearchByImage/Query?Task=<ID>&Limit=<Limit>&Offset=<Offset>

| URL | /LAPI/V1.0/Smart/SearchByImage/Query?Task=<ID>&Limit=<Limit>&Offset=<Offset> |
| --- | --- |
| Description | Get search results |
| Method | GET |
| Input Data | None |
| Success Return Data | SearchResult |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID | 1 |
| Limit | M | unsigned long | Number of tasks to search each time. Max 20. | 20 |
| Offset | M | unsigned long | Sequence number to start search from. The sequence number starts from 0. | 10 |
| SearchResult | M | Json Block | Search results |  |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID> |
| --- | --- |
| Description | Delete a task. |
| Method | DELETE |
| Input Data | None |
| Success Return Data | None |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ID | M | unsigned long | Task ID. |  |

/LAPI/V1.0/Smart/SearchByImage/Task

| URL | /LAPI/V1.0/Smart/SearchByImage/Task |
| --- | --- |
| Description | Get the search-by-image task list |
| Method | GET |
| Input Data | None |
| Success Return Data | { "Num":, "TaskList":[<SearchTaskBaseInfo>,…] } |
| Note | Applicable to NVR. |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Num | M | unsigned long | Number of tasks. | 1 |
| TaskList | C | array | Task list. Invalid when Num is 0. | - |
| SearchTaskBaseInfo | M | Json Block | Basic info about search-by-image task. See SearchTaskBaseInfo Json Block | - |

/LAPI/V1.0/Smart/SearchByImage/Task/<ID>

| URL | /LAPI/V1.0/Smart/SearchByImage/Task/<ID> |
| --- | --- |
| Description | Get task info |
| Method | GET |
| Input Data | None |
| Success Return Data | { "TaskInfo": <TaskInfo> } |
| Note | Applicable to NVR. |

Json Block

SearchPassRecordResult

| SearchPassRecordResult Json Block | { "Total":, "Offset": , "Num":, "FacePassRecordList": [ { "TargetFileName":, "PassRecordInfo":<PassRecordInfo> },… ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Total | M | unsigned long | Total number of search results. | 5000 |
| Offset | M | unsigned long | Offset from the start of the data | 100 |
| Num | M | unsigned long | Number of returned records | 10 |
| FacePassRecordList | C | array | Pass-through records. Absent when there are no data. |  |
| TargetFileName | M | string | Filename of object image. Specified in SearchTarget. Length range: [0,770] |  |
| PassRecordInfo | M | Json Block | Pass-through record info |  |

PassRecordInfo

| PassRecordInfo Json Block | { "RecordID": , "PassingTime": , "ChannelName":, "ChannelID":, "Similarity":, "AttributeInfo":<FaceAttr> "SnapshotImage": <SnapshotImage> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| RecordID | M | unsigned long | Pass-through record ID. | 1 |
| PassingTime | M | unsigned long | Pass-through time (unit: second, UTC format) | 1520412142 |
| ChannelName | M | string | Name of the channel capturing the snapshot. Range: [1, 63]. | “XX road intersection” |
| ChannelID | M | unsigned long | Channel ID | 1 |
| Similarity | M | unsigned long | Matching degree (similarity) | - |
| SnapshotImage | M | Json Block | Image info See SnapshotImage |  |
| AttributeInfo | C | Json Block | Face attribute info. Present when there is face attribute info. See FaceAttr. | - |

SnapshotImage

| SnapshotImage Json Block | { "BigImage":<FileInfo>, "SmallImage": <FileInfo>, "Area": { "left":, "top":, "right":, "bottom": } } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| BigImage | M | Json Block | Original image. Absent when searching faces or pushing snapshots. See FileInfo Json Block. |  |
| SmallImage | C | Json Block | Close-up image. See FileInfo Json Block. IPC can be configured not to push close-up images. |  |
| Area | M | Json Block | Coordinates of the close-up image in the original image. |  |
| left | M | unsigned long | Coordinates of the left point. |  |
| top | M | unsigned long | Coordinates of the top point. |  |
| right | M | unsigned long | Coordinates of the right point. |  |
| bottom | M | unsigned long | Coordinates of the bottom point. |  |

SearchPeopleLibrarieResult

| SearchPeopleLibrarieResult Json Block | { "Total":, "Offset": , "Num":, "PersionList": [ { "TargetFileName": "TargetInfo":<PersonInfo> },… ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Total | M | unsigned long | Total number of search results. | 5000 |
| Offset | M | unsigned long | Offset from the start of the data | 100 |
| Num | M | unsigned long | Returned records. | 10 |
| PersionList | C | array | Person list. Absent when there are no data. |  |
| TargetFileName | M | string | Filename of object image. Length range: [0,770]. |  |
| TargetInfo | M | Json Block | Person info. See PersonInfo |  |

Control

| Control Json Block | { "Control":, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Control | M | unsigned long | Action type 0: Start 1: Pause 2: Stop (currently not supported) | 0 |

SearchTarget

| SearchTarget Json Block | { "TargetInfo":<TargetInfo> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TargetInfo | M | Json Block | Object info |  |

TargetInfo

| TargetInfo Json Block | { "SimilarityThreshold":, "TargetNum":, "TargetInfoList":[<FileInfo>,…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SimilarityThreshold | M | unsigned long | Matching threshold (result below the threshold will not be returned). Max: 100. Min: 0. |  |
| TargetNum | M | unsigned long | Number of objects. Currently only one object is supported. |  |
| TargetInfoList | M | array | Object list info |  |
| FileInfo | M | json block | Object image info. JPEG. Max resolution: 1080*1960. Max. file size: 300KB. |  |

CompareTarget

| CompareTarget Json Block | { "Type":, "CompareLib":<CompareLib>, "CompareRecord":<CompareRecord>, } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Type | M | unsigned long | Object type: 1: Face 2: Motor vehicle (currently not supported) 3: Human body (currently not supported) |  |
| CompareLib | C | Json Block | If the object type is “Face”, the object is compared with a face library. If the object type is “Motor vehicle”, the object is compared with a vehicle library. Required. |  |
| CompareRecord | C | Json Block | If the object type is “Face”, the object is compared with a person pass-through record. If the object type is “Motor vehicle”, the object is compared with a vehicle pass-through record. Required. |  |

CompareLib

| CompareLib Json Block | { "LibType":, "LibNum":, "LibList":[<ID>,<ID>… ] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| LibType | M | unsigned long | Type of compared library. 0: Face library 1: Vehicle library | 0 |
| LibNum | M | unsigned long | Number of compared libraries. |  |
| LibList | M | array | Library list |  |

CompareRecord

| CompareRecordJson Block | { "BeginTime":, "EndTime":, "Num":, "ChannelList":[<ID>,<ID>…] } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| BeginTime | M | unsigned long | Start time (UTC format) |  |
| EndTime | M | unsigned long | End time (UTC format) |  |
| Num | 0 | unsigned long | Number of channels. If not specified, all channels will be searched by default. |  |
| ChannelList | C | array | Channel list. Required when Num is not 0. |  |
| ID | M | unsigned long | Channel ID |  |

SearchProgress

| SearchProgress Json Block | { "Percent":, "BeginTime":, "EndTime": , "MatchNum": "TotalNum":, "CompleteNum":, "RunStatus": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| Percent | M | unsigned long | Search progress. Range: [0,100].100 means the search task has completed. |  |
| BeginTime | M | unsigned long | Start time of the searched data (UTC format) |  |
| EndTime | M | unsigned long | End time of the searched data (UTC format) |  |
| MatchNum | M | unsigned long | Number of matched records. NVR can buffer 1000 matching results for each task. |  |
| TotalNum | 0 | unsigned long | Total number of records to compare. Numerical. Applicable to VMS only. |  |
| CompleteNum | 0 | unsigned long | Number of compared records. Numerical. Applicable to VMS only. |  |
| RunStatus | M | unsigned long | Task status: 0: Normal 1: Paused 2: Ended 3: Failed (common error) 4: Operation failed (modeling failed) |  |

SearchResult

Comparison object type and result structure

| Object Type | Comparison Object | SearchResult Structure |
| --- | --- | --- |
| Face | Pass-through record | SearchPassRecordResult |
| Face | Face library | SearchPeopleLibrarieResult |

SearchTaskBaseInfo

| SearchTaskBaseInfo Json Block | { "TaskID":, "Speed":, "Priority":, "SimilarityThreshold":, "BeginTime":, "EndTime":, "Status": } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| TaskID | C | unsigned long | Task ID. Optional when creating a task. | 1 |
| Speed | 0 | unsigned long | Search speed level (0-5). Currently not in use. | 0 |
| Priority | 0 | unsigned long | Execution priority level (0-3). High priority tasks are executed first. Currently not in use. | 0 |
| SimilarityThreshold | M | unsigned long | Matching threshold (results below the threshold will not be returned). Max: 100. Min: 0. Default: 80. | 80 |
| BeginTime | M | unsigned long | Start time (UTC format) |  |
| EndTime | M | unsigned long | End time (UTC format) |  |
| Status | M | unsigned long | 0: Not started 1: Searching 2: Paused 4: Stopped |  |

TaskInfo

| PassRecordSearchTaskInfo Json Block | { "TaskBaseInfo":<SearchTaskBaseInfo>, "SearchTarget":<SearchTarget>, "CompareTarget":<CompareTarget> } |
| --- | --- |

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| SearchTaskBaseInfo | M | Json Block | Basic info about search-by-image task. See SearchTaskBaseInfo Json Block | - |
| SearchTarget | M | Json Block | Search target. See SearchTarget Json Block. |  |
| CompareTarget | M | Json Block | Comparison target. See CompareTarget Json Block |  |

Extended Data Types

Common Color (CommonColorType)

Vehicle body colors are defined as common colors, and other things use this definition, such as hat color, shoe color, pedestrian clothes color.

| Code | Description | Remarks |
| --- | --- | --- |
| 0 | Black |  |
| 1 | White |  |
| 2 | Gray |  |
| 3 | Red |  |
| 4 | Blue |  |
| 5 | Yellow |  |
| 6 | Orange |  |
| 7 | Brown |  |
| 8 | Green |  |
| 9 | Purple |  |
| 10 | Cyan |  |
| 11 | Pink |  |
| 12 | Transparent |  |
| 13 | Silver |  |
| 14 | Dark |  |
| 15 | Light |  |
| 16 | No color |  |
| 17 | Yellow & green |  |
| 18 | Gradient green |  |
| 99 | Others |  |
| 100 | Unknown |  |

Structured Attributes (AttributeType)

| Code | Description | Remarks |
| --- | --- | --- |
| 0 | Gender |  |
| 1 | Age |  |
| 2 | / |  |
| 3 | Emotion |  |
| 4 | / |  |
| 5 | Glasses |  |
| 6 | Smile |  |
| 7 | Mask |  |
| 8 | Mustache |  |
| 9 | Facial attractiveness |  |
| 10 | Upper half body color |  |
| 11 | Lower half body color |  |
| 12 | Coat style |  |
| 13 | Coat textures |  |
| 14 | Trousers style |  |
| 15 | Hat |  |
| 16 | Bag |  |
| 17 | Trolley case |  |
| 18 | Posture |  |
| 19 | Umbrella |  |
| 20 | Hairstyle |  |
| 21 | Shoes |  |
| 22 | Temperature |  |
| Reserved | / |  |
| 100 | Movement speed |  |
| 101 | Movement direction |  |
| Reserved | / |  |
| 200 | Non-motor vehicle type |  |
| 201 | Non-motor vehicle plate |  |
| Reserved | / |  |
| 300 | Plate number |  |
| 301 | Plate type |  |
| 302 | Plate color |  |
| 303 | Vehicle type detection |  |
| 304 | Vehicle color |  |
| 305 | Vehicle brand detection |  |
| 306 | Sun visor detection |  |
| 307 | Seat belt detection |  |
| 308 | Calling detection |  |
| 309 | Vehicle model detection |  |
| 310 | Yellow label vehicle detection |  |
| 311 | Close-up face image of driver |  |
| 312 | Close-up face image of front-row passenger |  |
| 313 | Front-row passenger seat belt detection |  |
| 314 | Pendant detection |  |
| 315 | Inspection sticker detection |  |
| 316 | Dangerous goods detection |  |